3GPP TSG-RAN WG4 Meeting #112	R4-2412802

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on LP-WUS RRM requirements

Agenda item:	8.22.4.2

Document for:	Discussion

Introduction

In the last meeting, the following agreements were made:

Issue 2-1-1: Core requirements to be specified for LP-WUR measurement Agreement: - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS Issue 2-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode Agreement: - The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study. - Aspects not considered in RAN1 (if any) can be further discussed in RAN4. Issue 2-2-3: Accuracy Agreement: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON.

Discussion

MR RRM Relaxation/Offloading

So far, RAN2 has reached the following agreements on relaxation / offloading.

RAN2#126 Agreements: Serving cell measurements related: For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring Cell measurements related: RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Our understanding of relaxation in the context of LP-WUS is the following:

Measurement periodicity relaxation: MR serving cell or neighbouring cell measurement periodicity is relaxed allowing UE longer time to collect necessary number of samples according to the current requirements. Relaxation factor may be applied on top of existing measurement requirements.

Relaxation of the number of carriers to measure: At least some of the MR neighbouring cell measurement are not required to be performed while UE is in LP-WUS mode.

Measurement offloading: MR measurements of a serving cell are offloaded to LR.

In the context of LP-WUS, the UE should only be allowed to perform relaxation and / or measurement offloading when the UE is operating in LP-WUS mode, in other words, the LP-WUS entry condition has been met. It is not currently clear from RAN2 design if the entry conditions for MR neighbouring cell, MR serving cell relaxation and MR serving cell offloading are the same, or can those be configured differently. Regardless of the signalling, the relaxations and serving cell offloading may only be applied when the corresponding entry conditions have been fulfilled, and UE has been configured with LP-WUS.

The thresholds when relaxation as well as offloading are expected to be configurable by the network

MR Measurement relaxations requirements may only be applied when the corresponding entry conditions have been fulfilled, and the UE has been configured with LP-WUS/WUR configuration.

MR Serving cell measurement offloading requirements may only be applied when the serving cell entry conditions have been fulfilled, and the UE has been configured with LP-WUS/WUR configuration.

The UE may enter LP-WUS mode, i.e., at least some of the entry conditions for measurement relaxation or serving cell offloading are fulfilled. This may occur either directly when being released to the idle-mode, or alternatively during the idle-mode. It is not yet clear whether the UE is required to read idle-mode configuration information, for instance LP-WUS specific SIB information prior to applying relaxation and / or offloading. These details are up to RAN2, but this may have impact on whether the relaxations can be activated once the UE enters idle-mode, for instance based on MR connected mode measurements.

Relaxation / offloading may be applied based on connected mode MR measurements

RRM relaxation scenarios

Agreed Scenario:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| (Agreed) #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

In the previous RAN4 meeting, the following was agreed on RAN4 RRM measurements. At least the scenario where MR serving cell measurements and neighboring cell measurements are off is supported, and LR serving cell measurements are operational. This scenario is seen to bring the most power saving benefits as both serving cell and In this case the UE may be already listening LP-WUS, or UE may be listening paging occasions.

According to previous LP-WUS study, the average power saving is the highest when the MR offloads RRM measurements to LR, and the MR does not perform neighbouring cell measurements. The expected power saving gain depends on whether the LR supports PSS/SSS based measurements, and LP-SS design.

In scenario 1, the LR is only ON and MR is OFF, hence the measurement results from LR will need to decide whether the MR shall be waken up or not.

In scenario 1, the LR is only ON and MR is OFF, hence the measurement results from LR will need to decide whether the MR shall be waken up or not.

In fully offloading case (#1), the UE is performing LR measurements, and UE may be either listening to LP-WUS or legacy paging.

#2 Relaxed case b:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

In the scenario two the MR serving cell measurements are performed in parallel with the LR measurements. The LR measurements are either PSS/SSS measurements or LP-SS measurements. We see that this scenario may occur, for instance, if instead of turning MR measurements off, the MR measurements are relaxed with a high scaling factor. In this case, the MR may be even turned off before the next measurement occasion, and the case is similar to scenario 3.

In case MR measurement periodicity scaling is long, the MR may be turned off, in which case the case #2 may be similar to case #3.

Discuss if case #2 scenario can be covered by case #3.

#3 Relaxed case b:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b <> | On with relaxation measurement | On with relaxation measurement | ON |

In the case #3 both MR serving and neighboring cell measurements are both relaxed while the LR is performing serving cell measurements. The case is similar to scenario #2 except the neighbouring cell relaxations are included. We think that this case is important to support as there are procedures such as cell reselection that relies on neighbouring cell measurements, and LR is not capable of performing these.

Define requirements for Scenario #3

#4 Relaxed case c:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Our understanding is that the relaxed case c refers to the case where neighboring cell measurements are performed as the UE is configured with carriers that are important to be measured (e.g. high priority carriers).  While we agree that this is important topic, from the requirements point of view, high priority carrier discussion can be covered as a separate discussion. It is also expected that RAN2 will be also discuss this topic in detail.

Discuss high priority carrier topic separately, and no need to define specific scenario where MR serving cell measurements are offloaded to LR but the neighbouring cell measurements are still being performed.

Neighboring cell measurement relaxation

In R16, the neighboring cell RRM measurement relaxation for intra-frequency or inter-frequency/inter-RAT frequency is allowed for UEs not at cell edge and/or with low mobility. In general, the neighbour cell measurements only need to be performed by the UE if the serving cell is not strong enough, i.e., when the intra-frequency cell re-selection criterion (Srxlev > SIntraSearchP and Squal > SIntraSearchQ) or the inter-frequency cell re-selection criterion (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) is not fulfilled.

|  | Rel-16 low mobility criteria | Rel-16 Not-at-cell-edge criteria | Rel-16 Both low mobility and Not-at-cell-edge criteria | Rel-17 stationary criteria (Redcap) | Rel-17 stationary and not-at-cell edge criteria (Redcap) |
| --- | --- | --- | --- | --- | --- |
| Measurement relaxation factor for neighbour cell measurement (intra/inter frequency measurement and inter-RAT measurement when applicable) | 3 times | 3 times | 1 hour | 6 times | 4 hours |

Table 2: Existing RRM measurement criteria and corresponding relaxation

LR is not going to do neighboring cell measurements.

RAN4 to discuss which, if any, of the legacy MR neighboring cell RRM measurement relaxation applicable to MR while configured with LP-WUS.

Mobility

Ensuring robust mobility in idle and connected mode is an essential feature of NR just like it has been an essential feature in legacy systems. UE measurement activity in idle mode has always been defined with very relaxed UE requirements in order to enable UE implementation freedom to optimize any possible UE power savings. However, requirements are still defined and defined in a way that these can be controlled such that robust mobility can be ensured. In legacy the UE only has the option of using the MR.

In legacy mobility measurements, the UE only has the option of using the MR

Introducing LP-WUR (LR), LP-WUS and LR measurements changes this basic scheme. UE will now, if configured, be allowed to offload some of the serving cell measurements currently performed by the MR to be performed by the LR. Depending on the offloading scheme, the UE may completely stop MR measurements on both serving and neighbor cells (including neighbor cells on inter-frequency carriers).

Hence, UE may partially or fully offload the MR measurements to the LR. Fully or partial offloading of MR measurements to LR leads to MR measurement relaxation and hence delayed neighbor cell detection and measurements.

Fully or partial offloading of MR measurements to LR leads to MR measurement relaxation and hence delayed neighbor cell detection and measurements.

It is, as mentioned, essential to maintain robust NR mobility also when UE is allowed using LR for MR measurement offloading (fully or partial). This means, that a UE using LR need to have time enough to detect and measure potential (MR) neighbor cells, in a timely manner, to ensure that the existing MR cell reselection requirements can be maintained.

MR measurements need to be performed such that existing MR idle mode mobility is not impacted by use of LR on UE side.

Existing idle mode mobility test cases applies for a UE configured with LP-WUR usage

In last meeting RAN4 agreed to support the fully offloading case. In this scenario the UE is not required to perform serving or neighbor cell measurements when UE is allowed to offload MR measurements to LR. Hence, UE will only be required to perform LR measurements (on serving cell). UE will not be required to perform neighbor cell measurements (including detection of new cells).

To enable use of the fully offloading scenario it needs to be ensured that the UE will initiate MR measurements in a timely manner accounting both MR cell detection and measurement delay. Currently we only see such trigger being based on LR measurements when fully offloading is applied. If the network shall be able to configure any offloading thresholds and exiting thresholds, RAN4 need to define LR measurement accuracy requirements at least for the fully offloading scenario.

RAN4 need to define LR measurement accuracy requirements at least for the fully offloading scenario.

LP-WUS monitoring and paging monitoring

Currently, RAN1 has made the following agreements:

RAN1#116: Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered.

Based on RAN1 agreement, UE may start LP-WUS monitoring once the entry condition for MR serving cell measurements is fulfilled. However, the agreement leaves it open whether the case is supported where UE does not support LP-WUS signal but monitors legacy paging.

In the case where the UE offloads MR serving cell evaluations to LR, but UE would still be doing PO monitoring with the MR, the power saving gain would be limited as the UE would need to monitor PO every DRX cycle, and MR would need to receive at least one SSB(s) before paging monitoring, hence the serving cell could be monitored then with MR.

If MR does paging monitoring and LP-WUS is not monitored, power saving gains would be limited because UE would still need to wake up every PO and read at least one SSB.

Based on the current RAN1 design both LP-WUS and paging montoring are always performed together. The LP-WUS on the LR side wakes up UE MR to monitor paging. As long as the UE is monitoring legacy paging, there is no difference from the network point of view if the LP-WUS or paging woke up the UE. From the network point of view, the LP-WUS signal is a signal that mainly benefits the UE power saving and makes network scheduling more complex.

LP-WUS introduces implementation complexity to the network side and currently only brings UE power saving benefits to the UE side.

RAN2 agreement: Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Based on RAN2 agreement on entry, when configured, the UE may start to monitor LP-WUS and stop monitoring of Legacy PO currently only if MR measured RSRP / RSRQ is above threshold.

RAN4 requirements are applicable for UEs supporting LP-WUS signal, and configured with LP-WUS configuration by higher layers.

Once the UE starts LP-WUS monitoring, the UE may stop the legacy PO monitoring, while the exit condition is not met. Once the UE receives LP-WUS signal, the UE shall check if the UE is being paged, and begin to exit the LP-WUS mode. Regarding the requirements, RAN4 should consider whether the UE starts MR measurements directly after LP-WUS is triggered, or after the UE resolves that the paging is intended for the UE.

UE may receive LP-WUS signal but is not paged. Discuss whether the UE starts measurements after the LP-WUS signal, or after LP-WUS signal + when UE has discovered it has been being paged.

In the case of where UE does not support LP-WUS signal, the UE may support measurement relaxation and offloading, as long as the UE is capable of performing paging monitoring.

Entry / Exit condition discussion for LP-WUS monitoring (Manosha)?

It is still unclear in RAN1 and RAN2 how to evaluate the entry/exit conditions for a UE to start entering to monitor LP-WUS and stop monitoring LP-WUS. As discussed in the Section 2.3, in Meeting#126 RAN2 has made the baseline agreements about monitoring LP-WUS. The following elements seem to be playing a role while deciding the entry/exit conditions:

- Measurements are based on serving cell MR measurements, serving cell LR, serving cell MR & LR together

- Signal to be measured for entry/exit criteria evaluation: PSS/SSS, LP-SS, SSB

- WUR architecture to be considered: envelope detector based WUR, sequence-based WUR (IQ-based WUR)

- Measurement metric: SS-RSRP, SS-RSRQ, SS_SINR or LP-RSRP, LP-RSRQ

For e.g., to evaluate the entry criteria, a UE may use serving cell measurements done by the LR. The LR may be an envelope detector type of a receiver, and measure the LP-RSRP of the LP-SS. Similarly, serving cell measurements could be done by an LR which has a sequence-based type WUR that could measure the SS-RSRP of the PSS/SSS.

It is still open whether the measurements are done by LR, MR, or LR & MR together, and whether PSS/SSS is used or LP-SS. what are the measurement metrics to be evaluated, and what is the considered WUR architecture.

RAN4 to discuss whether to consider LR based measurements, MR based measurements or LR & MR combined measurements to be used for evaluating LP-WUS entry and exit criteria.

Conclusion

The paper the following Observations and Proposals were made:

Observation 1: Measurement periodicity relaxation: MR serving cell or neighbouring cell measurement periodicity is relaxed allowing UE longer time to collect necessary number of samples according to the current requirements. Relaxation factor may be applied on top of existing measurement requirements.

Observation 2: Relaxation of the number of carriers to measure: At least some of the MR neighbouring cell measurement are not required to be performed while UE is in LP-WUS mode.

Observation 3: Measurement offloading: MR measurements of a serving cell are offloaded to LR.

Observation 4: The thresholds when relaxation as well as offloading are expected to be configurable by the network

Proposal 1: MR Measurement relaxations requirements may only be applied when the corresponding entry conditions have been fulfilled, and the UE has been configured with LP-WUS/WUR configuration.

Proposal 2: MR Serving cell measurement offloading requirements may only be applied when the serving cell entry conditions have been fulfilled, and the UE has been configured with LP-WUS/WUR configuration.

Proposal 3: Relaxation / offloading may be applied based on connected mode MR measurements

Proposal 4: In fully offloading case (#1), the UE is performing LR measurements, and UE may be either listening to LP-WUS or legacy paging.

Observation 6: In case MR measurement periodicity scaling is long, the MR may be turned off, in which case the case #2 may be similar to case #3.

Proposal 5: Discuss if case #2 scenario can be covered by case #3.

Proposal 6: Define requirements for Scenario #3

Proposal 7: Discuss high priority carrier topic separately, and no need to define specific scenario where MR serving cell measurements are offloaded to LR but the neighbouring cell measurements are still being performed.

Observation 7: LR is not going to do neighboring cell measurements.

Proposal 8: RAN4 to discuss which, if any, of the legacy MR neighboring cell RRM measurement relaxation applicable to MR while configured with LP-WUS.

Observation 8: In legacy mobility measurements, the UE only has the option of using the MR

Observation 9: Fully or partial offloading of MR measurements to LR leads to MR measurement relaxation and hence delayed neighbor cell detection and measurements.

Observation 10: MR measurements need to be performed such that existing MR idle mode mobility is not impacted by use of LR on UE side.

Proposal 9: Existing idle mode mobility test cases applies for a UE configured with LP-WUR usage

Proposal 10: RAN4 need to define LR measurement accuracy requirements at least for the fully offloading scenario.

Observation 11: If MR does paging monitoring and LP-WUS is not monitored, power saving gains would be limited because UE would still need to wake up every PO and read at least one SSB.

Observation 12: LP-WUS introduces implementation complexity to the network side and currently only brings UE power saving benefits to the UE side.

Observation 13: Based on RAN2 agreement on entry, when configured, the UE may start to monitor LP-WUS and stop monitoring of Legacy PO currently only if MR measured RSRP / RSRQ is above threshold.

Proposal 11: RAN4 requirements are applicable for UEs supporting LP-WUS signal, and configured with LP-WUS configuration by higher layers.

Proposal 12: UE may receive LP-WUS signal but is not paged. Discuss whether the UE starts measurements after the LP-WUS signal, or after LP-WUS signal + when UE has discovered it has been being paged.

Observation 14: It is still open whether the measurements are done by LR, MR, or LR & MR together, and whether PSS/SSS is used or LP-SS. what are the measurement metrics to be evaluated, and what is the considered WUR architecture.

Proposal 13: RAN4 to discuss whether to consider LR based measurements, MR based measurements or LR & MR combined measurements to be used for evaluating LP-WUS entry and exit criteria.