3GPP TSG-RAN WG4 Meeting # 112	R4-2412847

Maastricht, The Netherlands, August 19-23, 2024

Source:	Ericsson

Title:	UE RF requirements for type 4a/4b capable FWA UE for EN-DC/NR-CA

Agenda item:	8.5.2.1

Document for:	Approval

# 1	Introduction

In RAN4#111 the following WF was approved [1]:

… < Issue 2-1-3: Minimum DL frequency separation > Way Forward: - Continue further discussion on the following two options and conclude it in the next meeting. - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC. - Option 2. Max (5/2* BWanother, [50] MHz). …

In this tdoc we analyze the open issue.

# 2	Discussion

## 2.1	Issue 2-1-3:  Minimum DL frequency separation

Option 2 is legacy, for CA, for instance,  in TS 38.101-1 we have:

… ### 7.10A.2 Minimum requirement For the test parameters in table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1). Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Center of BWanother Relative to edge of BWwanted. 1; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz. Another wanted carrier; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS NOTE 4 + 1; BWwanted > BWanother. Another wanted carrier; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS NOTE 4 + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another wanted carrier; Power of wanted carrier + 25. NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void.. For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2. …

The text above corresponds to option 2 and the requirement is max (5/2* BWanother, 50MHz).

However, in [2] it was shown that for the particular bands of this WI we have 80MHz +BWanother/2 (between B2 and B3). This expression corresponds to option 1 and is derived below:

For example, the spectrum allocation has suggested a minimum the frequency separation between CCs. As shown in figure 2.1-1 as primarily considered in this WI, the minimum frequency separation for non-contiguous CA, which is defined as the center of BWanother relative to edge of BWwanted, could be at least 80MHz +BWanother/2 (between B2 and B3). Therefore, such limitation should be specified associated the corresponding requirements. For time being, if operators come up with new spectrum allocation which suggests a smaller frequency separation between CC. The corresponding spec can be updated accordingly. Otherwise, the two CCs theoretically can be apart as small as single subcarrier, e.g. 15kHz or 30kHz. Obviously, such unrealistic scenario should not be considered.

Figure 2-1 3.5GHz spectrum allocation in Japan

Observation: As shown in figure 2.1-1 as primarily considered in this WI, the minimum frequency separation for non-contiguous CA, which is defined as the center of BWanother relative to edge of BWwanted, should be at least 80MHz +BWanother/2 (between B2 and B3). Otherwise, the two CCs theoretically can be apart as small as a single subcarrier, e.g. 15kHz or 30kHz. Obviously, such unrealistic scenario should not be considered and can unnecessarily complicate the overall implementation.

The issue of “80 MHz” has been discussed at least since Toulouse meeting RAN4 #108 and in RAN4 #110bis we had:

… < Issue 2-2-4: DL frequency separation > Way Forward: - Check and conclude the following requirement in the next meeting. - No limitation on DL maximum frequency separation is needed for type 4 UE. - For DL minimum frequency separation for type 2 and type 4 UE, 80MHz is used. …

Ericsson has not objected to option 1, “Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC”, since this is the particular case in this WI.

Observation 1:  “Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC”, is fine, since this is the particular case in this WI.

# 3 	Summary

Observation 1:  “Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC”, is fine, since this is the particular case in this WI.

# References

[1] 	R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI.

[2]	R4-2311306, On frequency separation for intra-band non-collocated CA, Apple