3GPP TSG-RAN WG4 Meeting # 112	R4-2412848

Maastricht, The Netherlands, August 19-23, 2024

Source:	Ericsson

Title:	UE behavior and network signaling for type 4a/4b capable FWA UE for EN-DC/NR-CA

Agenda item:	8.5.2.2

Document for:	Approval

# 1	Introduction

In RAN4#111 the following WF was approved [1]:

… < Issue 2-2-1: New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) > Way Forward: - Continue further discussion on the following two options and conclude it in the next meeting. - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication. - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication. … < Issue 2-2-3: New BS Signalling to switch between Type 4a/4b and Type 1(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. < Issue 2-2-4: New BS Signalling to switch between Type 4a/4b and Type 2 > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. …

In this tdoc we analyze the open issues.

# 2	Discussion

## 2.1	Issue 2-2-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA)

There are two options:

- Option 1. Add new UE capabilities for Type 4a and Type 4b support indication.

- Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

The UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE [2]):

Table 1: UE Types [2]

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

The type 4a and 4b UE RF architectures are also illustrated in [3]:

Figure 1: Architecture of type 4a(left) and type 4b(right) (Fig. 2.1 in [3])

From table 1 and figure 1, we can conclude that type 4a ⊂ 4b , i.e. all type 4a properties, #CC, antennas/LNA, mixer,  BB, #RX, NRCA/EBDC-feature support and power imbalance, are a subset of type 4b. Moreover the difference between the type 4a and 4b are small.

Observation 1: 	All type 4a properties, #CC, antennas/LNA, mixer,  BB, #RX, NRCA/EBDC-feature support and power imbalance, are a subset of type 4b. Moreover the difference between the type 4a and 4b are small.

Proposal 1: 	We support option 2: add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

## 2.2	Issue 2-2-3:  New BS Signaling to switch between Type 4a/4b and Type 1(collocated)

The WF [1] states that we first, conclude discussions of whether to add new UE capability(s) for Type 4 and then, continue further discussion on BS signaling in the next meeting. That is fine, but we nevertheless state our opinion here. As discussed in previous section 2.2 above we prefer a single type 4, but besides that, we would like to observe that we now have two type 1, as previously pointed out by Apple:

Type 12 ←Type 2

and

Type 14 ←Type 4

We have Type 12 ≠ Type 14, since Type 12 has 4 RX and Type 14 has 8 RX.

It is fair to discuss a Type 14 ←Type 4 transition, since we already have Type 12 ←Type 2 in release 8, but we then need to discuss what we mean with Type 1.

Observation 2: 	Type 1, as defined in rel-18 is not the same UE architecture as Type 1 in rel-19. Both are co-located, but the maximum number of supported layers and therefore number of RX differ from 4 to 8.

Observation 3: 	We are ok, in principle to define Type 1 ⟵Type 4, if we can agree what Type 1 means.

We see no reason to have a special signal indicating colocation and Type 1 behavior for release 18 Type 12 ←Type 2 and relrease-19 Type 14 ←Type 4. In principle this could be the same signal. The rest is up to UE capabilities.

Proposal 2: Make rel-18 Type 1 and Type 2 transition and rel-19 Type 1 and Type 2 transition the same signal.

## 2.3	Issue 2-2-4:  New BS Signaling to switch between Type 4a/4b and Type 2

It has already been analysed in [3], that the signalling will become quite complicated:

Table 1 UE behavior for Rel-19 non-collocated intra-band NR CA (Table 2.1 in [3])

| Index | UE indicates type 4a/4b? | UE indicates type 2? | Rel-19 NW signaling provided? | UE type |
| --- | --- | --- | --- | --- |
| 1 | × | × | × | Type 1 |
| 2 | × | ○ | × | Type 2 |
| 3 | ○ | × | × | Type 4a/4b |
| 4 | ○ | × | ○ | Type 1 |
| 5 | ○ | ○ | × | Type 4a/4b? |
| 6 | ○ | ○ | ○ | Type 1? |

We will also get a parallel discussion to the one in section 2.3 with two different type 2, one type 2 from rel-18 and one as a result of Type 2 ⟵Type 4 transition. It is therefore better to just rely on configuring the rank of the type 4 connection.

Proposal 3: Do not define new signalling to switch between Type 4 and Type 2, instead rely on Type 4 MIMO rank reconfiguration.

# 3 	Summary

Observation 1: 	All type 4a properties, #CC, antennas/LNA, mixer,  BB, #RX, NRCA/EBDC-feature support and power imbalance, are a subset of type 4b. Moreover the difference between the type 4a and 4b are small.

Proposal 1: 	We support option 2: add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

Observation 2: 	Type 1, as defined in rel-18 is not the same UE architecture as Type 1 in rel-19. Both are co-located, but the maximum number of supported layers and therefore number of RX differ from 4 to 8.

Observation 3: 	We are ok, in principle to define Type 1 ⟵Type 4, if we can agree what Type 1 means.

Proposal 2: Make rel-18 Type 1 and Type 2 transition and rel-19 Type 1 and Type 2 transition the same signal.

Proposal 3: Do not define new signalling to switch between Type 4 and Type 2, instead rely on Type 4 MIMO rank reconfiguration.

# References

[1] 	R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI.

[2]	R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI.

[3]	R4-2408361, Discussion on UE Capability/UE behavior and network signaling for Type 4a Type 4b EN-DC/NR-CA, ZTE Corporation, Sanechips.