3GPP TSG-RAN WG4 Meeting # 112	R4-2412849

Maastricht, The Netherlands, August 19-23, 2024

Source:	Ericsson

Title:	On RRM impact related to support of intra-band non-collocated EN-DC/NR-CA deployment Phase2 WI

Agenda item:	8.5.3

Document for:	Approval

# 1	Introduction

WID [1] states:

… The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signalling is provided. R18 handling of UE behavior can be used as a reference. …

In this contribution we analyze the RRM impact related to support of intra-band non-collocated EN-DC/NR-CA deployment Phase2 WI: : new receiver type(s).

# 2	Discussion

## 2.1 	MTTD and MRTD requirements

In 3GPP Release-18 Support of intra-band non-collocated EN-DC/NR-CA deployment we have the following requirements regarding MRTD and MTTD:

… For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in Table 7.6.3-1 provided that UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18. For E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed if UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.6.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16. Table 7.6.2.1-1: Maximum receive timing difference requirement for inter-band synchronous EN-DC Sub-carrier spacing of E-UTRA cell in MCG (kHz); DL Sub-carrier spacing of cell in SCG (kHz) (Note1); Maximum receive timing difference (µs). 15; 15; 33. 15; 30. 15; 60. 15; 120. Note 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}.. ========= … For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference as shown in 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below if nonCollocatedTypeNR-CA-r18 is provided. … Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2-1; 25. Between FR1 and FR2-2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA.. ========= … For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting [requirementTypeIndication-r18] is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in Table 7.5.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18. For E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed if UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1 if UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.5.3-1 if the UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16. … Table 7.5.2.1-1 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 35.21. 15; 30; 35.21. 15; 60; 35.21. 15; 120; 35.21. ========= … For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in Table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided. Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of TAGs; Maximum uplink transmission timing difference (µs). FR1; 34.6. FR2-1; 8.5 Note1. Between FR1 and FR2-1; 26.1. Between FR1 and FR2-2; 26.1. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA.. …

Observation 1: In 3GPP Release-18 Support of intra-band non-collocated EN-DC/NR-CA deployment we have the following requirements regarding MRTD and MTTD:

| Feature | MRTD (µs) | MTTD (µs) |
| --- | --- | --- |
| EN-DC | 33 | 35.21 |
| CA | 33 | 34.6 |

The deployment considerations for the operators will not change from rel-18 to release-19. This rel-19 WI is mainly about new receiver type(s). In release-19 WI we still have the same two CC for ENDC and NRCA as in release-18. The increase of MIMO rank from 2 to 4 does not change deployment of sites and MRTD and MTTD requirement.

Observation 2: The deployment considerations for the operators will not change from rel-18 to release-19.

Proposal 1: 	Reuse MRTD and MTTD for intra-band non-collocated EN-DC/NR-CA rel-18, also for release-19, i.e., ED-DC: MRTD = 33 µs, MTTD = 35.21 µs and CA: MRTD = 33 µs, MTTD = 34.6 µs

## 2.2	Scheduling restriction/availability, SCell operation related delay and interruption

The release-18 WI worked out Scheduling restriction/availability and SCell operation related delay and interruption for non colocated intra band ENDC and NRCA. We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19.

Observation 3: 	We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19.

Proposal 2:	The development for release-19 non-colocated intra band ENDC and NRCA requirements can use release-18 Scheduling restriction/availability, SCell operation related delay and interruptions as starting points for analysis.

# 3 	Summary

Observation 1: In 3GPP Release-18 Support of intra-band non-collocated EN-DC/NR-CA deployment we have the following requirements regarding MRTD and MTTD:

| Feature | MRTD (µs) | MTTD (µs) |
| --- | --- | --- |
| EN-DC | 33 | 35.21 |
| CA | 33 | 34.6 |

Observation 2: The deployment considerations for the operators will not change from rel-18 to release-19.

Proposal 1: Reuse MRTD and MTTD for intra-band non-collocated EN-DC/NR-CA rel-18, also for release-19, i.e., ED-DC: MRTD = 33 µs, MTTD = 35.21 µs and CA: MRTD = 33 µs, MTTD = 34.6 µs

Observation 3: 	We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19.

Proposal 2:	The development for release-19 non-colocated intra band ENDC and NRCA requirements can use release-18 Scheduling restriction/availability, SCell operation related delay and interruptions as starting points for analysis.

# References

[1] 		RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple