3GPP TSG-RAN4 Meeting #112	R4-2412850

# Maastricht, Netherlands, 19th Aug 2024 – 23rd Aug 2024

Source:	KDDI, Samsung, Murata

Title:	TP for TR 38719-02-01 CA_n18A-n41A

Agenda item:			7.3.3

Document for:	Approval

1. Introduction

This contribution is a text proposal (TP) for TR 38.719-02-01 to add CA_n18A-n41A.

The MSD test points and MSD value of PC2 case for DL CA_n18A-n41A with UL CA_n18A-n41A have been defined in [1] as in Table 7.2A.5-1a : 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA.

However, those of PC3 case were missing in [1]. In this TP, the MSD value of PC3 case is evaluated using the following two steps.

- The test points are the same as those of the PC2 case.

- The MSD value was determined by first borrowing from that of PC3 DC_26_n41 in Table 7.3B.2.3.5.1-1 of TS 38.101-3[2], and then compensated by taking consideration of the REFSES difference of band 26 and n18 using Table 7.3.2-1a in TS 38.101-1[1].

2. Text Proposal

---Start of changes---

5.x		CA_n18-n41

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n18-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n18-n41 | n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n18-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A | CA_n18A-n41A | n3 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |

Unaffected sessions are omitted for the sake of simplicity.

### 5.x.1.5	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements due to IMD introduced by inter-band UL CA_n18A-n41A shows that:

- the 3rd order harmonic mixing may fall into Rx frequencies of n18.

- IMD3 of inter-band UL CA, CA_n18A-n41A, may fall into Rx frequencies of n18.

- No crossband isolation interference.

The MSD exception is defined as below.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18A-n41A | n18 | 820 | 5 | 25 | 865 | 18.1 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |

---End of changes---

Reference

[1] 3GPP TS 38.101-1 V18.6.0 (2024-06): "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[2] 3GPP TS 38.101-3 V18.6.0 (2024-06): "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".