3GPP TSG-RAN WG4 Meeting #112                                    R4-2412851

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:            Discussion on L3 measurement delay by optimizing CSSF

Agenda item:			8.15.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC. Finally, the performance requirement issue for L3 measurement enhancement CSSF was confirmed. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

<Agreement> Issue 2-2-6: measurement performance requirement when apply L3 measurement delay reduction by optimizing CSSF - Agreement: - RAN4 is not to change existing measurement performance requirement when consider optimization of CSSF in L3 measurement delay reduction.

2 Discussion

2.1 Clarification on the bullets in WID for this CSSF optimization

From our understanding, the topic is in CA/DC scenarios, which is very different from the first topic which is on single carrier, so we think it should be independent from the topic 1: FR2-1 SSB based L3 measurement delay reduction for connected mode by optimizing Rx beam sweeping factor. In the future, it is likely to introduce a (new) capability/signalling or new UE behaviour corresponding to CSSF based FR2-1 L3 measurement delay reduction feature, but it is should only applicable to legacy UE, i.e., single Rx chain available at receiver, and the RRM measurement requirements shall be derived based on the assumption that UE could sweep one beam direction at any single time instance, i.e., K=1.

Proposal 1: Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement to L3 measurement are independent features

Proposal 2: The RRM measurement requirements of CSSF optimization shall be derived based on the assumption that UE could sweep one beam direction at any single time instance

2.2 UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

From RRM point of view, CSSFoutside_gap,i is accounted in the following requirements (just for FR2/FR2-1, and just list DRX cycle≤ 320ms):

Table 1. Spec. impact brought by CSSFoutside_gap,i

| Intra frequency measurements without measurement gaps | Intra-frequency cell identification | Time period for PSS/SSS detection, (Frequency range FR2) | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| --- | --- | --- | --- |
|  |  | Time period for PSS/SSS detection, deactivated SCell (FR2) | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
|  |  | Time period for PSS/SSS detection, deactivated PSCell (FR2) | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
|  | Measurement period | Measurement period for intra-frequency measurements without gaps (FR2) | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
|  |  | Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR2) | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
|  |  | Measurement period for intra-frequency measurements without gaps (deactivated SCG applicable for PSCell) (FR2) | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| Inter frequency measurements without measurement gaps | Inter frequency Cell identification | Time period for PSS/SSS detection, (FR2) | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
|  |  | Time period for time index detection (FR2) | Max(200ms, Ceil(1.5 MSSB_index_inter x Kp) Max(SMTC period, DRX cycle)) CSSFinter |
|  | Measurement period | Measurement period for inter-frequency measurements without gaps (FR2) | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |

Based on the above requirement:

Proposal 3: The following RRM requirements shall be studied on whether SSB based L3 measurement delay can be reduced by optimizing CSSF outside gap in CA/DC scenarios

- SSB based Intra-frequency measurement without MG. Including: TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG. Including: TPSS/SSS_sync_inter , TSSB_time_index_inter and TSSB_measurement_period_inter

Since TSSB_time_index is not required for intra-frequency measurements since useServingCellTimingForSync is always enabled for FR2.

2.3 Applicability requirement of L3 measurement delay reduction by optimizing CSSF

To define the CSSFoutside_gap,i, in the current Spec., the following BC modes are considered: EN-DC/SA/NR-DC and NE-DC. More specifically, EN-DC mode contains: EN-DC with FR1 only CA, EN-DC with FR2 only intra/inter band CA, EN-DC with FR1 +FR2 CA (FR1 PSCell/ FR2 PSCell); SA mode contains: FR1 only CA, FR2 only intra/inter band CA, FR1 +FR2 CA (FR1 Pcell/ FR2 PCell); NR-DC mode contains: FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell), FR1 + FR1 NR-DC (FR1 pCell and FR1 PScell); NE-DC mode contains: NE-DC with FR1 only CA, NE-DC with FR2 only intra/inter band CA, NE-DC with FR1 +FR2 CA (FR1 PCell). We think the considered BC should align to the RF BC configuration specified in 38.101-1/2/3.

Proposal 4: The considered BC should align to the RF BC configuration specified in 38.101-1/2/3

However, although legacy CSSFoutside_gap,i are specified for all such scenarios except new BC with two FR2 operating band (e.g., CA_n77-n79-n257-n259), the objective of the WID is to reduce FR2-1 L3 measurement delay, we suggest to deprioritize all the FR1 only scenarios, that is

Proposal 5: RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i:

- EN-DC: EN-DC with FR2 only intra band CA; EN-DC with FR2 only inter band CA; EN-DC with FR1 +FR2 CA (FR1 PSCell); EN-DC with FR1 +FR2 CA (FR2 PSCell)

- SA: FR2 only intra band CA; FR2 only inter band CA; FR1 +FR2 CA (FR1 PCell); FR1 +FR2 CA (FR2 PCell)

- NR-DC: FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

- NE-DC: NE-DC with FR2 only intra band CA; NE-DC with FR2 only inter band CA; NE-DC with FR1 +FR2 CA (FR1 PCell)

2.4 Candidate Solutions

1. Optimize searcher occupancy/sharing ratio among PCC/PSCC/SCC.

From our understanding, it might be a possible way, however, based on the previous discussions, the existing CSSF scaling factors are defined based on the necessity and the priority of the serving carriers. For example, in Rel-15, based on 2 searchers assumption, for CA mode:

- One searcher is dedicated for PCC, the other searcher is shared by all the SCCs.

  - Half of the other searcher (50%) is dedicated to SCC where neighbour cell measurement is required, which is prioritized than other SCC(s)

  - The remaining half of the searcher is shared among FR1 SCCs and FR2 SCCs where neighbour cell measurement is not required.

From this, if RAN4 to optimize the CSSF by changing the searcher sharing ratio among all serving carriers, the condition and the scenario should be confirmed. Besides, evaluation on whether the change will bring extra complexity at UE side is also needed.

Proposal 6: If RAN4 agree to take: Optimize the CSSF by changing the searcher sharing ratio among all serving carriers as the solution to specify L3 measurement delay reduction

- RAN4 should to discuss the corresponding applicable condition and the scenario

2. Optimize NSCC_SSB for FR2 intra-band CA

For FR2 intra-band CA, from measurement perspective, UE only needs to measure the serving SCell in the same band as PCell according to the existing principle specified in 9.2.3.2

- 9.2.3.2 Requirements for FR2 [omit] where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or ..[omit] The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band

In this sense, theoretically, there is no need to define the scaling factor for SCC(s) as. e.g., 2×Number of configured SCell(s), shown below

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. |  |  |  |  |  |  |  |

Proposal 7: RAN4 to discuss the feasibility of reducing NSCC_SSB to optimize the CSSF for FR2 intra-band CA

4 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 measurement delay by optimizing CSSF. The following observations and proposals are obtained:

Proposal 1: Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement to L3 measurement are independent features

Proposal 2: The RRM measurement requirements of CSSF optimization shall be derived based on the assumption that UE could sweep one beam direction at any single time instance

Proposal 3: The following RRM requirements shall be studied on whether SSB based L3 measurement delay can be reduced by optimizing CSSF outside gap in CA/DC scenarios

- SSB based Intra-frequency measurement without MG. Including: TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG. Including: TPSS/SSS_sync_inter , TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 4: The considered BC should align to the RF BC configuration specified in 38.101-1/2/3

Proposal 5: RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i:

- EN-DC: EN-DC with FR2 only intra band CA; EN-DC with FR2 only inter band CA; EN-DC with FR1 +FR2 CA (FR1 PSCell); EN-DC with FR1 +FR2 CA (FR2 PSCell)

- SA: FR2 only intra band CA; FR2 only inter band CA; FR1 +FR2 CA (FR1 PCell); FR1 +FR2 CA (FR2 PCell)

- NR-DC: FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

- NE-DC: NE-DC with FR2 only intra band CA; NE-DC with FR2 only inter band CA; NE-DC with FR1 +FR2 CA (FR1 PCell)

Proposal 6: If RAN4 agree to take: Optimize the CSSF by changing the searcher sharing ratio among all serving carriers as the solution to specify L3 measurement delay reduction

- RAN4 should to discuss the corresponding applicable condition and the scenario

Proposal 7: RAN4 to discuss the feasibility of reducing NSCC_SSB to optimize the CSSF for FR2 intra-band CA

5 References

[1] R4-2410260, “Way Forward for [111][228] NR_RRM_Ph5”, Apple

[2] R4-2406392, “Way Forward of [110bis][230]NR_RRM_Ph5” , Apple