3GPP TSG-RAN WG4 Meeting #112                             R4-2412852

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:            Discussion on L3 measurement enhancement by optimizing Rx BSF

Agenda item:			8.15.2.1

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on FR2-1 SSB based L3 measurement delay reduction for connected mode. Finally, the applicability issue for L3 measurement enhancement was confirmed. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

<Agreement> Applicability requirement: Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. FFS: UE is in RRC CONNECTED mode (newly added issue in this meeting based on ZTE’s proposal) Agreement: Only support multi-Rx L3 measurement for CONNECTED Issue 2-1-5: measurement performance requirement when apply L3 measurement delay reduction by optimizing Rx BSF - Agreement: - RAN4 is not to change existing measurement performance requirement when consider optimization of Rx BSF in L3 measurement delay reduction.

2 Discussion

2.1 Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF

FFS: UE power class and UE’s mobility status

From our understanding, although L3 measurement delay reduction is more meaningful in high speed scenario, HST scenario is very different from static/low/medium mobility scenario, we cant ensure all the requirements to be enhanced in handheld UE/PC3 are also applicable to PC6 before extensively discussion. Actually, from multi-Rx simultaneous L1 measurement perspective, very little conclusions achieved in PC3 can be reused in PC6. From this, we are worried about that if HST is introduced, more workload will be brought.

Furthermore, for different power classes, the spherical coverage provided by Rx beam/antenna pannel could be widely different. It likely not be able to make any conclusions directly that whether the outcomes of the WI discussion catering to PC3 would be compliant with other power classes, let alone, a number of aspects are still suspending for PC3. From this, we agree with Moderator, supporting Option 4:

Proposal 1: RAN4 to consider UE supporting FR2-1 power class 3 as first priority

- Suggest to preclude HST at current stage

- Suggest to discuss the applicability of the outcome of the WI discussion to other power classes after concluding on PC3

2.2 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature

Obviously, if to reduce the L3 measurement delay by optimizing Rx BSF, multi-Rx simultaneous reception of UE should be in active mode. But the controversial point is that how to define the active mode in L3 measurement case. Some companies suggest to take “UE is configured with group-based beam reporting (GBBR) report” as the considered condition, however, from our understanding, there is no need to consider the same applicable conditions specified in Rel-18 multi-Rx (i.e., GBBR) in L3 multi-Rx. The reasons are:

- GBBR is a L1 behaviour, which was initially discussed in Rel-15 RAN1. And GBBR-r17 was discussed thoroughly for beam measurement/reporting enhancement to facilitate inter-TRP beam paring. In Rel-18, RAN4 introduced it to L1 measurement enhancement with multi-Rx, which focus on serving cell mTRP scenario. However, Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment. One example is that intra-frequency HO decision (A3), the UE is triggered to report the L3 cell quality measurement of the target cell and L3 beam measurement to its serving cell when the A3 trigger condition expires at the certain time instance. While, for such procedure, we believe UE don’t need to aware the association between TRP and beams.

- In NR, UE needs to identify neighbour cells and perform RRM measurements (intra-f/inter-f measuremets) for L3 mobility purpose, especially, if the target cell is unknown, UE to perform target cell detection based on PSS/SSS first. At this time, CMR setting/set/resource and CSI-report for BM are not considered, take “UE is configured with group-based beam reporting (GBBR) report” as the satisfied condition is a little inappropriate for the traditional L3 measurement procedure

- One case but not typical: For measurments on serving cell, there could be a possibility that the UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with GBBR for L3. That is, based on the high-level measurement model of L1 and L3, for the measurements in RRC_CONNECTED mode in intra-NR, UE first perform beam-specific measurements at L1, and then, L3 does filtering on the L1 measurement results. Since in Rel-18, “groupBasedBeamReporting-r17 is configured” is considered as the pre-condition for fast beam sweeping for L1 measurement, accordingly, it can comply with the L3 multi-Rx condition. However, the mTRP L3 multi-Rx measurement is not what we want to study in this WI.

Observation 1: In RAN1, GBBR-r17 was introduced and discussed for beam measurement/reporting enhancement to facilitate inter-TRP beam paring

Observation 2: In the traditional L3 neighbour cell measurement, L1-RSRP based beam management is not involved

Proposal 2: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

Proposal 3: Do not reuse the same applicable conditions specified in Rel-18 L1 multi-Rx (i.e., GBBR is configured ) in Rel-19 L3 multi-Rx measurement WI

FFS: Power consumption issue

From our understanding: 1) The cell/beam identification delay and the measurement period might be variant for devices in the field with different N-factor, and such devices will likely not have the same system performance under similar conditions in mobility scenario, which may bring different impact on UE ability to track cells and NW decision; 2) The minimum RRM requirements defined in the current Spec., are based on the assumption that UE could scan/sweep one beam direction at one time instance; 3) The new multi-Rx L3 measurement is a new feature and most likely be optional, the NW needs to be functional with UEs that do/do not support the optional feature, and without capability signalling, the NW will be unaware of which UEs can support lower N (N1,N2,N3), and cant configure/handle them differently. From this, we prefer to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional.

Proposal 4: Prefer to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional

FFS: Simultaneous operation between L3 and L1 measurements (Scheduling restriction)

In L1 measurement enhancement, the scheduling restriction/measurement restriction under multi-Rx simultaneous reception were discussed. Actually, some possible factors would cause scheduling restriction, e.g., Collision with UL transmission and DL measurement on TDD carrier; Rx beam sweeping in FR2; Mix-numerology between data and SSB simultaneous reception.

In the current Spec., the scheduling restriction was defined for the following two cases in Table 1 at least:

Table 1. UE behavior cases. The channel or RS in group 1 overlaps with a RS in group 2 on the same OFDM symbol

| Case | Channels/RS in group 1 | RS in Group 2 |
| --- | --- | --- |
| Case 1 | PDCCH/PDSCH reception on serving cell | SSB reception for L3 measurements of neighbor cell |
| Case 2 | CSI-RS reception for RLM/BFD/L1-RSRP on serving cell (CSI-RS with repetition=OFF) | SSB reception for L3 measurements of neighbor cell |

From our understanding, for L3 measurement with multi-Rx simultaneous reception, new UE behavior on reception channel/RS in the same time granularity (e.g., same OFDM symbol) might occur under certain conditions for the above two cases.

Proposal 5: RAN4 to discuss the feasibility of defining scheduling availability requirement of UE performing SSB based L3 measurement under multi-Rx simultaneous reception in FR2-1.

2.3 Scenarios to use L3 measurement delay reduction by optimizing Rx BSF

2.3.1 Scenarios 1~4

For the following scenarios:

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

Existing: Mpss/sss_sync_w/o_gaps=24 samples, Mmeas_period_w/o_gaps=24 samples

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

Existing: Mpss/sss_sync_w/o_gaps=24 samples, Mmeas_period_w/o_gaps=24 samples

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

Existing: Mpss/sss_sync_w/o_gaps=24 samples, MSSB_index_inter=24 samples, Mmeas_period_w/o_gaps=24 samples

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Existing: Mpss/sss_sync_w/o_gaps=24 samples, MSSB_index_inter=24 samples, Mmeas_period_w/o_gaps=24 samples

Theoretically, one of the benefits on faster beam sweeping is that short delay in cell detection and measurement will shorten the difference between valid RSRP measurement report and actual condition when the report is provided, which can improve the system performance. However, there is a tradeoff between mobility performance and number of samples to be measured

- The delay requirements are based on total number of required samples for measurement M. To be specific, the M is proportional to the number of samples and RX beam sweeping factor N (i.e., 24= 3 × 8 ).

- In the previous release, there were two candidate values for the N, that is N=4 and N=8. The N=8 was finally compromised by taking mobility performance verification based on SLS and UE antenna implementation (spherical coverage) into consideration. Actually, from our understanding, from system performance point of view only, the N=4 for both PSS/SSS detection and RSRP measurement delay requirements can be accepted by companies.

- If the UE is able to simultaneously sweep K beam directions at one time instance, the multi-Rx capable and available UE need Nbeam/K measurement samples for scanning once over all the Rx beam directions. The BSF due to Rx beam sweeping operations equals to Nbeam/K. For K, here we can assume, e.g., K=2, for 2 Rx chains

- If N/measured sample is reduced, the performance degradation issue may unavoidably arise. While, since no SLS results were obtained based on the new sweeping behaviour, we cant observe how much performance loss the UE will experience with different number of Rx beam sweeping, and cant conclude how to reduce the N, i.e, the total number of required samples for measurement accordingly under the new scenario. But we prefer to use N=4, the corresponding total samples is 20= 5 × 4 as baseline, at least one of the candidate values to define the delay requirements of inter-/intra-frequency measurement for Rel-19 L3 multi-Rx measurement, since as we mentioned before, the performance based on N=4 can be accepted by the companies in the previous release.

Observation 3: The delay requirements are based on total number of required samples for measurement, which is proportional to the number of samples and RX beam sweeping factor N

Observation 4: From system performance point of view only, the N=4, M=20 for both PSS/SSS detection and RSRP measurement delay requirements can be accepted by companies in the previous release

Observation 5: If N/measured sample is reduced, the mobility performance will be degraded

Proposal 6: Suggest to take the following scenarios as the highest priority:

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 7: To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

- Whether to/How to define the other values can be based on SLS results (if necessary)

2.3.2  Scenario 5

For the following scenarios: Handover delay to unknown target cell

We have the following observations and understandings:

1. Regarding the delay requirements:

- If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8* Trs ms

- If the target cell is an unknown inter-frequency cell and Tsearch = 8*3* Trs ms if the target cell Es/Iot≥-2 dB

2. Handover to an unknown cell may occur in some corner cases, e.g., when UE has not sent a valid measurement report before receiving handover command or if the handover configuration from network cannot be guaranteed

3. For the target cell is FR2 and unknown, UE need to sweep/train all the Rx beams to find out reliable DL timing from PSS/SSS. The total detection delay is [Sample Number] × [N] × [Periodicity]. For intra-frequency, Sample Number=1, for inter-frequency, Sample Number=3.

4. For HO (non blind) scenario, the UE will have been measuring the target cell and will at least be configured with a valid SMTC periodicity/SSB periodicity including period, offset etc. While for blind HO, there was an LS sent to RAN2 regarding missing SSB-related assistance information [3]. And in case that the optional information is not provided (UE does not have prior information on Trs), the expected behavior is that the UE assumes 5ms SSB periodicity to perform blind search for SSB specified in TS 38.133.

From this, if the N is reduced, we have concerns on whether the handover performance of mobility management may be effected too much, for example, whether the reliable and accurate timing information from PSS/SSS at the UE sided can be found, especially, when the UE has no prior knowledge.

Observation 6: The handover performance of mobility management may be effected if Rx BSF is reduced for L3 measurement

Proposal 8: It is pessimistic to reduce handover delay to unknown target cell by optimizing Rx BSF

2.3.3  Scenario 6~10

For the following scenarios: PSCell addition/RRC Re-establishment/RRC Connection Release with Redirection/SCell activation/SCG activation/CGI identification

We suggest to put the above scenarios with low priority, since the corresponding requirements are related to DC/idle mode (cell selection procedure first)/CA.

Proposal 9: Put the following scenarios: PSCell addition, RRC Re-establishment/RRC Connection Release with Redirection, SCell activation, SCG activation, CGI identification as low priority.

2.3.4  Scenario 11

The CSI-RS is configured associatedSSB

If associatedSSB is configured for CSI-RS, CSI-RS based cell identification contain: 1) Cell search via SSB; 2) PBCH decoding; 3) CSI-RS measurement.

From our understanding, the scenario and the scope are not contradictory, our intention is not to define the requirement of the step 3): L3 CSI-RS measurement by using multi-Rx but to focus on the potential impact brought by SSB based L3 multi-Rx measurement enhancement on Step 1) and 2) for CSI-RS based cell identification when associatedSSB is configured. Since obviously, the step 1) and 2) are related to SSB based L3 measurement enhancement. Combining with the Spec., an example of text proposal for Section 9.10.2.5 may be

If a UE [supporting Rel-19 L3 multi-Rx] is configured with the [(new) L3 multi-Rx related configurations] and higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the CSI-RS based measurement shall include PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap. - PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in Clause 9.2.5.1.

Since no conclusions for SSB based L3 measurement delay reduction for the time being. We propose:

Observation 7: The intra-/inter-f measurement delay requirements of CSI-RS based cell identification when associatedSSB is configured contain SSB based cell search and PBCH decoding

Proposal 9: The impact on the L3 measurement requirement of CSI-RS based cell identification when associatedSSB is configured could be revisited when SSB based L3 measurement delay reduction is concluded.

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 multi-Rx measurement. The following observations and proposals are obtained:

Observation 1: In RAN1, GBBR-r17 was introduced and discussed for beam measurement/reporting enhancement to facilitate inter-TRP beam paring

Observation 2: In the traditional L3 neighbour cell measurement, L1-RSRP based beam management is not involved

Observation 3: The delay requirements are based on total number of required samples for measurement, which is proportional to the number of samples and RX beam sweeping factor N

Observation 4: From system performance point of view only, the N=4, M=20 for both PSS/SSS detection and RSRP measurement delay requirements can be accepted by companies in the previous release

Observation 5: If N/measured sample is reduced, the mobility performance will be degraded

Observation 6: The handover performance of mobility management may be effected if Rx BSF is reduced for L3 measurement

Observation 7: The intra-/inter-f measurement delay requirements of CSI-RS based cell identification when associatedSSB is configured contain SSB based cell search and PBCH decoding

Proposal 1: RAN4 to consider UE supporting FR2-1 power class 3 as first priority

- Suggest to preclude HST at current stage

- Suggest to discuss the applicability of the outcome of the WI discussion to other power classes after concluding on PC3

Proposal 2: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

Proposal 3: Do not reuse the same applicable conditions specified in Rel-18 L1 multi-Rx (i.e., GBBR is configured ) in Rel-19 L3 multi-Rx measurement WI

Proposal 4: Prefer to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional

Proposal 5: RAN4 to discuss the feasibility of defining scheduling availability requirement of UE performing SSB based L3 measurement under multi-Rx simultaneous reception in FR2-1.

Proposal 6: Suggest to take the following scenarios as the highest priority:

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 7: To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

- Whether to/How to define the other values can be based on SLS results (if necessary)

Proposal 8: It is pessimistic to reduce handover delay to unknown target cell by optimizing Rx BSF

Proposal 9: The impact on the L3 measurement requirement of CSI-RS based cell identification when associatedSSB is configured could be revisited when SSB based L3 measurement delay reduction is concluded.

4 References

[1] R4-2410260, “Way Forward for [111][228] NR_RRM_Ph5”, Apple

[2] R4-2405439, “General discussion on the L3 measurement enhancement”, Samsung

[3] R4-1809547, “LS on missing SSB information”