3GPP TSG-RAN WG4 Meeting #112                                                                 R4-2412853

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:                        General discussion on fast SCell activation for UE supporting eEMR

Agenda item:			8.15.3

Document for:	Discussion

1. Introduction

The following objective for fast SCell activation for UE supporting Rel-18 EMR can be found in RP-240830.

Objective - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this paper, we would like to share some preliminary understandings on fast Scell activation for UE supporting Rel-18 EMR in Rel-19.

2 Background on Rel-16 EMR and Rel-18 eEMR

The initial intention of introducing the EMR is to enable SN addition based on IDLE mode measurement results. In order to speed up the SN addition procedure, the early measurement should be used by MN to decide which SN can be configured to UE. While, the main problem of Rel-16 EMR is that the validity of the reported measurement results cant be always guaranteed. To resolve such issue, the UE is allowed to perform additional/enhanced measurement during and/or after RRC connection setup/resume to validate the available measurement results for faster FR2 SCG/SCell setup, if necessary.

In Rel-18 SCG/SCell setup delay improvement discussion, companies agreed to focus on inter-band target cell in FR2 and consider FR1+FR2 as main scenario. Lots of issues were discussed in Rel-18, e.g., the validity duration X & definition of valid measurement results, new UE feature group, NW assistant information/configurations/starting point/ending point for Rel-18 measurement enhancement, relationship between Rel-16 EMR and Rel-18 eEMR to SCell/SCG setup delay and accuracy requirements. However, skipping all the details discussed in the previous release, from our understanding, the basic thing is that the UE supporting Rel-18 EMR, that is the UE

- supports eEMR features including measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18

- to be configured enhanced measurement parameters for fast CA/DC setup, validity duration X

shall be capable to guarantee the available IDLE/INACTIVE mode measurement results are valid and report them to the NW ASAP, which allows for accurate and faster FR2 SCG/SCell setup/addition.

Actually, based on the eEMR discussions in Rel-18, for enhanced measurement, which starts from RRC setup/resume procedure, majority proposed the following discussion point: measurement report after MO configuration, and defined two scenarios, however, for the time being, no conclusions yet.

Observation 1: At current stage, it is expected UE to send the validated measurement report before MO configuration for IDLE/INACTIVE

3 Discussion

Based on the Rel-18 eEMR background, the subsequent story is that since the UE has valid measurement results, it can reliably leverage the informations, e.g., target frequency and/or Cell ID, and/or target SSB information. Then, we can see the possibility of improvement on SCell activation delay if consider to utilize the valid measurement results/prior informations on eEMR under some scenarios/conditions.

Proposal 1: RAN4 to discuss the possibility of reducing the SCell activation delay with the valid measurement results/prior informations on eEMR under certain scenarios/conditions

3.1 Scope and RRM requirement impact

Regarding SCell activation delay, the following requirements were specified in Section 8.3: SCell Activation Delay Requirement for Deactivated Scell; Direct SCell Activation at SCell addition; Direct SCell Activation at Handover; Direct SCell Activation at RRCResume; SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells; Direct SCell Activation of Multiple Downlink SCells at SCell addition; Direct SCell Activation of Multiple Downlink SCells at Handover; Direct SCell Activation of Multiple Downlink SCells at RRC Resume; SCell Activation Delay Requirement for Deactivated PUCCH SCell; SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells; SCell Deactivation Delay Requirement for Activated PUCCH SCell; SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells.

From our understanding, theoretically, the scope of fast SCell activation for UE supporting Rel-18 EMR shall contain normal SCell activation and direct SCell activation. Thereinto, the procedure of direct SCell activation combining with eEMR for IDLE/INACTIVE mode please find below, where C is a fixed value defined in 38.331 shown in the following Table.

Fig 1. Direct SCell activation combining with eEMR for IDLE/INACTIVE mode, NW provides measurement configurations for Rel-18 FR2 CA/DC setup at RRCRelease

<TS 38.331>

However, considering that the scope maybe is too large if we think of everthing, we suggest to focus on impact on the following RRM requirements:

- 8.3.2 SCell Activation Delay Requirement for Deactivated Scell

- 8.3.4 Direct SCell Activation at SCell addition

Proposal 2: The scope of fast SCell activation for UE supporting Rel-18 EMR shall contain normal SCell activation and direct SCell activation, the impact on the following RRM requirements can be studied as the highest priority:

- 8.3.4 Direct SCell Activation at SCell addition

- 8.3.2 SCell Activation Delay Requirement for Deactivated SCell

For FR2 intra-band scenario, the Tactivation_time defined in RAN4 summarized as below:

| Conditions |  | Tactivation_time |
| --- | --- | --- |
| If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band |  | TFirstSSB+ 5ms |
| If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band | If the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell | 3 ms |
| If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band | If the FR2 power class 6 UE supporting scellWithoutSSB and [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode] is configured with highSpeedMeasFlagFR2-r17 | 3 ms |

Observation 2: For FR2 intra-band scenario, if the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, there is no extra room to reduce the SCell activation delay even the UE supports Rel-18 EMR

Besides, since Rel-18 SCG/SCell setup WI takes FR1+FR2 as main scenario into consideration, we have the following proposal

Proposal 3: Suggest to focus on the following scenario for fast SCell activation for UE supporting Rel-18 EMR: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1

3.2 Use cases

3.2.1 Fast direct SCell activation at SCell addition

From our understanding, this is one of the possible use cases for fast SCell activation for UE supporting Rel-18 EMR. To be specific, for direct SCell activation, the configured SCell(s) can be configured in activated state by RRC reconfiguration message upon addition without any separate MAC-CE for SCell activation, allowing the SCell to be used more quickly. From this, for the target SCell unknown case, for the UE supporting Rel-18 EMR, the available IDLE/INACTIVE mode measurement results shall be reported to NW ASAP/AEAP (e.g., in 3s for PC3) after they are done with the validation, and it is expected NW to add/activate the SCell faster by using such measurements via RRC. In this sense, there is no need to perform cell detection (remain detectable) and L1-RSRP measurement (UE has valid result in hand) beforehand if the to-be-activated SCell is the first cell on that FR2 band and is unknown. Then theoretically, unknown case becomes known case, the SCell activation delay reduction accordingly.

Proposal 4: For direct SCell activation at SCell addition, if the to-be-activated SCell is the first cell on that FR2 band and is unknown, it is feasible to fast SCell activation for UE supporting Rel-18 EMR

- RAN4 to discuss how to define the conditions

3.2.2 Fast normal SCell activation

For the normal SCell activation latency procedure, there is a time gap between RRC configuration (SCell configuration) and the SCell activation command. And the known condition for SCell (FR2 case) is defined as below

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Based on our Fig 1, if UE can transmit the valid measurement results along with the L3 measurement report, i.e., after MO configuration, there is a possibility that the SCell can be activated AFAP based on the valid measurement results report. However, in this case, the known condition/fast SCell activation condition & mechanism shall be extensively discussed.

Proposal 5: If the UE supporting Rel-18 EMR can report the validated measurement results after MO configuration, RAN4 to discuss the feasibility of fast normal SCell activation

3 Conclusion

In this contribution, we provided our initial viewpoints to requirement impact analyses on fast Scell activation for UE supporting Rel-18 eEMR. The following observations and proposals are obtained:

Observation 1: At current stage, it is expected UE to send the validated measurement report before MO configuration  for IDLE/INACTIVE

Observation 2: For FR2 intra-band scenario, if the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, there is no extra room to reduce the SCell activation delay even the UE supports Rel-18 EMR

Proposal 1: RAN4 to discuss the possibility of reducing the SCell activation delay with the valid measurement results/prior informations on eEMR under certain scenarios/conditions

Proposal 2: The scope of fast SCell activation for UE supporting Rel-18 EMR can contain normal SCell activation and direct SCell activation, the impact on the following RRM requirements can be studied as the highest priority:

- 8.3.2 SCell Activation Delay Requirement for Deactivated SCell

- 8.3.4 Direct SCell Activation at SCell addition

Proposal 3: Suggest to focus on the following scenario for fast SCell activation for UE supporting Rel-18 EMR: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1

Proposal 4: For direct SCell activation at SCell addition, if the to-be-activated SCell is the first cell on that FR2 band and is unknown, it is feasible to fast SCell activation for UE supporting Rel-18 EMR

- RAN4 to discuss how to define the conditions

Proposal 5: If the UE supporting Rel-18 EMR can report the validated measurement results after MO configuration, RAN4 to discuss the feasibility of fast normal SCell activation

4 References