3GPP TSG-RAN WG4 Meeting #112                                        R4-2412854

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung, KDDI

Title:              General discussion on intra-band non-collocated EN-DC/NR-CA type 4

Agenda item:			8.5.3

Document for:	Discussion

1 Introduction

In RANP, the objectives for intra-band non-collocated EN-DC/NR-CA were agreed in RP-240838. In this meeting, we will give some initial discussions for the WI from RRM perspective.

[RP-240838] Support of intra-band non-collocated EN-DC/NR-CA deployment; Phase2: new receiver type(s) The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

2 Backgrounds

2.1 Scenarios to be supported for EN-DC/NR-CA in previous Releases

2.1.1 Inter-band EN-DC

- In Rel-15, inter-band EN-DC/NE-DC with overlapping DL bands was discussed, no matter non-contiguous or contiguous EN-DC with 2CC scenarios, only co-located deployment is allowed.

- In Rel-16/Rel-17/Rel-18, inter-band EN-DC/NE-DC with overlapping DL bands in non-collocated deployment was discussed.

2.1.2 Intra-band NR-CA

- In Rel-15, intra-band NR-CA with overlapping DL bands was discussed, no matter non-contiguous or contiguous NR-CA with 2CC scenarios, only co-located deployment is allowed.

- In Rel-18, intra-band non-contiguous synchronous NR-CA in non-collocated deployment was discussed.

2.2 MRTD requirements defined in previous Releases

2.2.1 Inter-band EN-DC

|  | MRTD/MTTD |
| --- | --- |
| Rel-15 | MRTD＜3us |
| Rel-16/Rel-17 | Section 7.6.2 in 38.133 |
| Rel-18[3] | For inter-band synchronous EN-DC with overlapping DL bands Type 2 UE in non-collocated deployment: MRTD=33us; MTTD=35.21us |

2.2.2 Intra-band NR-CA

|  | MRTD/MTTD |
| --- | --- |
| Rel-15 | MRTD＜3us |
| Rel-18 | - For intra-band non-contiguous synchronous NR-CA Type 2 UE in non-collocated deployment: MRTD=33us, MTTD=34.6us - For intra-band asynchronous NR-CA Type 2 UE in non-collocated deployment: Not introduced |

Observation 1: In Rel-18,

- For FR1 intra-band non-contiguous synchronous NR-CA Type 2 UE in non-collocated deployment:

MRTD=33us and MTTD=34.6us

- For FR1 intra-band asynchronous NR-CA Type 2 UE in non-collocated deployment: No MRTD/MTTD requirement was introduced

- For FR1 inter-band synchronous EN-DC Type 2 UE in non-collocated deployment: MRTD=33us and MTTD=35.21us

  - UE behaviours discussed in previous Releases

2.3.1 Inter-band non-collocated EN-DC with overlapping DL bands

- In Rel-16, RAN2 has introduced the following UE capability: interBandMRDC-WithOverlapDL-Bands-r16.

  - The type 2 UE was described as a UE indicating interBandMRDC-WithOverlapDL-Bands-r16 for inter-band non-collocated EN-DC with overlapping DL bands

    - For UE supporting the capability, it needs to satisfy type 2 requirements.

<38.306> interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE).; BC; No; N/A; FR1 only.

- In Rel-18, the new capability requirementTypeIndication-r18 was introduced for the inter-band EN-DC type 2 UE to indicate whether it supports reconfigure to type 1 (4 Rx/CC) with the new BS signalling nonCollocatedTypeMRDC-r18

  - The new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2

    - For the UE supporting both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18, it needs to satisfy both type 1 requirement and type 2 requirement, of which type 2 requirement is the default behaviour

      - Type 1 requirement applies if nonCollocatedTypeMRDC-r18 is indicated

<38.331> nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.. <38.306> requirementTypeIndication-r18 Indicates whether the UE supports network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-r18 for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands as specified in TS 38.331 [9]. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.; UE; No; No; FR1 only. interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements.; BC; No; N/A; FR1 only. < R2-2313889, LS to RAN4 on Intra-band non-collocated NR-CA. EN-DC> - For UEs supporting new capability, adopt default type2 for nonCollocatedTypeMRDC-r18. - For UEs supporting new capability, adopt default type2 for nonCollocatedTypeNR-CA-r18. - RAN2 agree to remove Editor’s note for MTTD RAN4 spec reference for Type1. - The new RRC signalling would not be applied to the FDD-FDD inter-band EN-DC with overlapping or partially overlapping bands.

Observation 2: In Rel-18,

- The new capability requirementTypeIndication-r18 was introduced for the UE supporting TDD-TDD inter-band EN-DC with overlapping DL bands to indicate whether it supports reconfigure to type 1 with the new BS signalling nonCollocatedTypeMRDC-r18

    - For the UE supporting both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18, it needs to satisfy both type 1 requirement and type 2 requirement, of which type 2 requirement is the default behaviour

    - Type 1 requirement applies if nonCollocatedTypeMRDC-r18 is indicated

    - Intra-band non-collocated NR-CA with overlapping DL bands

- In Rel-18, the following new UE capability: intraBandNR-CA-non-collocated-r18 was introduced.

  - The type 2 UE is described as a UE indicating intraBandNR-CA-non-collocated-r18 for intra-band non-collocated NR-CA with overlapping DL bands

    - For the UE supporting intraBandNR-CA-non-collocated-r18, it needs to satisfy both type 1 requirement and type 2 requirement, of which type 2 requirement is the default behaviour

      - Type 1 requirement apply if nonCollocatedTypeNR-CA is indicated

<38.306> intraBandNR-CA-non-collocated-r18 Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9].; BC; No; N/A; FR1 only. <38.331> nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18..

Observation 3: In Rel-18,

- For the UE supporting intraBandNR-CA-non-collocated-r18, it needs to satisfy both type 1 requirement and type 2 requirement, of which type 2 requirement is the default behaviour

    - Type 1 requirement apply if nonCollocatedTypeNR-CA is indicated

2.4 Rel-19 RAN4 Progress on UE RF aspects

In the previous meetings, RF achieved the following agreements

< R4-2406627 WF on NonCol_intraB_ENDC_NR_CA > UE Capability for Type 4 EN-DC/NR-CA Agreement: - UE having Type 4 capability support the following capabilities (Set 1) when operating in non-collocated scenarios. - Maximum 4 MIMO layers on each CC - Maximum power imbalance of 25dB - UE having Type 4 capability support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios. - Maximum 8 MIMO layers on each CC - It may be optionally supported - Maximum PSD imbalance of 6dB < R4-2410686 WF on Rel-19 non-collocated UE RF requirements > UE RF requirements for Type 4 EN-DC/NR-CA Agreement: - Use 25dB power imbalance for 4Rx REFSENS + 1dB for Type 4 Agreement: - Define the Type 4 receiver imbalance requirements in the same table as for Type 2. - FFS on the details of modification Agreement: - No limitation for maximum DL frequency separation UE Capability(s)/UE behavior and network signalling for Type 4 EN-DC/NR-CA Agreement: - If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities. - If the issue is identified for these bullets, RAN4 will revisit them.

Observation 4: In RF, it was agreed that Type 4 reuses the Type 2 RF requirements

- Discussion on RRM requirements impact

Regarding the UE RF architecture, in the Rel-18, the following RF architecture on type 2 was agreed as baseline

< R4-2214458 WF on NonCol_intraB_ENDC_NR_CA> Agreement: - Total four RF antenna is assumed. - Reuse UE RF architecture of inter-band non-contiguous DC_42_n77/78 EN-DC Type-2 (i.e. 2 layer/2 Rx Chain per CC, total 4 Rx Chain) as the baseline.

Observation 5: The UE RF architecture of intra-band non-collocated non-contiguous NR-CA type 2 is 2 layer/2 Rx Chain per CC, total 4 Rx Chain

For non-collocated non-contiguous intra-band NR-CA and inter-band EN-DC 4 layer MIMO case, it was agreed to discuss the requirements based on the following Type 4a and type 4b UE RF architectures [4]. The discussions were initiated in the previous release and will be formally moved forward in Rel-19.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | NRCA/ ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | ENDC | 25dB full range | Requires 6 antennas and LNA => FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | NRCA ENDC | 25dB full range | Requires 8 antennas and LNA => FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |

From the table, we can find that for type 4:

- Focus on only FWA

- The two CCs use separate antennas and LNAs, and each of the CCs are received via separate Rx chains. The difference between the type 4a and 4b is that type 4a supports 2 DL MIMO on one CC and 4 DL MIMO on the other CC, while type 4b supports 4 DL MIMO on both CCs.

- The maximum power imbalance can be 25dB

By comparing the RF architectures between type 2 and type 4, from our understanding, there are no substantial differences except the supported MIMO layers per CC.

Observation 6: By comparing the RF architectures between type 2 and type 4, the main difference is the supported MIMO layers per CC

3.1 MRTD/MTTD

In accordance to the observations above mentioned, from RRM perspective, since type 4 UE has the similar architecture with type 2 UE except the number of Rx chain/layer per CC, from our understanding, Type 4 UE can reuse the same MRTD/MTTD requirements as Type 2 UE.

Proposal 1: For non-collocated FR1 intra-band non-contiguous synchronous NR-CA Type 4 UE, the same MRTD/MTTD requirements as Type 2 UE can be reused

Since it was agreed in RRM that not to introduce MRTD/MTTD requirements for non-collocated intra-band asynchronous NR-CA Type 2 UE in Rel-18, and the following notes were agreed in RF 38.101-1:

<Section 4.2> All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PCell and SCells for NR SA.

We propose:

Proposal 2: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

  - Other RRM requirements

In Rel-18, in addition to MRTD/MTTD requirements, the following requirements were also discussed: Scheduling restriction requirements; Interruption requirements (at SCell addition/release/activation/deactivation, during measurements on deactivated SCC); SCell activation delay requirements; BFD/CBD requirements.

From our understanding, the essential reasons to define such requirements for non-collocated intra-band CA are that: 1) The aggregated CCs will be received using separate Rx chain; 2) SCell addition/release/activation/deactivation may impact on not only the active serving cells are in the same band but also the active serving cells are in the overlapping DL bands (i.e., inter-band CA shall be considered)

The same logic, since the type 4 UE has the similar architecture with type 2 UE except the number of Rx chain/layer per CC, and still focus on overlapping or partially overlapping bands, we believe

Proposal 3: The impact on the following RRM requirements for type 4 UE can be the same as for type 2 UE

- Scheduling restriction

- Interruption

- SCell activation delay

- SSB/CSI-RS based BFD

- SSB/CSI-RS based CBD

However, considering the following controversial points:

- How to/whether to introduce new NW signalling (s) /capabilities for type 4 EN-DC/NR-CA

- How to deal with the backward compatible issues brought by the introduction of new NW signalling (s) /capabilities and the relationship between type 2 UE capabilities and Rel-19 type 4 UE capabilities

are still under discussion in RF, from our understanding, how to introduce/specify the impacted RRM requirements for Rel-19 type 4 UE cannot be concluded at current stage.

Proposal 4: How to specify the impacted RRM requirements for Rel-19 type 4 UE need to wait for RF progress on the design of new UE capabilities/NW signallings

4 Conclusion

In this contribution, we provided our initial viewpoints to RRM requirement impact analyses on c intra-band non-collocated EN-DC/NR-CA type 4. The following observations and proposals are obtained:

Observation 1: In Rel-18,

- For FR1 intra-band non-contiguous synchronous NR-CA Type 2 UE in non-collocated deployment:

MRTD=33us and MTTD=34.6us

- For FR1 intra-band asynchronous NR-CA Type 2 UE in non-collocated deployment: No MRTD/MTTD requirement was introduced

- For FR1 inter-band synchronous EN-DC Type 2 UE in non-collocated deployment: MRTD=33us and MTTD=35.21us

Observation 2: In Rel-18,

- The new capability requirementTypeIndication-r18 was introduced for the UE supporting TDD-TDD inter-band EN-DC with overlapping DL bands to indicate whether it supports reconfigure to type 1 with the new BS signalling nonCollocatedTypeMRDC-r18

    - For the UE supporting both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18, it needs to satisfy both type 1 requirement and type 2 requirement, of which type 2 requirement is the default behaviour

    - Type 1 requirement applies if nonCollocatedTypeMRDC-r18 is indicated

Observation 3: In Rel-18,

- For the UE supporting intraBandNR-CA-non-collocated-r18, it needs to satisfy both type 1 requirement and type 2 requirement, of which type 2 requirement is the default behaviour

    - Type 1 requirement apply if nonCollocatedTypeNR-CA is indicated

Observation 4: In RF, it was agreed that Type 4 reuses the Type 2 RF requirements

Observation 5: The UE RF architecture of intra-band non-collocated non-contiguous NR-CA type 2 is 2 layer/2 Rx Chain per CC, total 4 Rx Chain

Observation 6: By comparing the RF architectures between type 2 and type 4, the main difference is the supported MIMO layers per CC

Proposal 1: For non-collocated FR1 intra-band non-contiguous synchronous NR-CA Type 4 UE, the same MRTD/MTTD requirements as Type 2 UE can be reused

Proposal 2: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

Proposal 3: The impact on the following RRM requirements for type 4 UE can be the same as for type 2 UE

- Scheduling restriction

- Interruption

- SCell activation delay

- SSB/CSI-RS based BFD

- SSB/CSI-RS based CBD

Proposal 4: How to specify the impacted RRM requirements for Rel-19 type 4 UE need to wait for RF progress on the design of new UE capabilities/NW signallings

5 References

[1] RP-240838, “New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)”, Apple

[2] R4-2217734, “WF on NonCol_intraB_ENDC_NR_CA for New Type UE”, KDDI

[3] R4-2220398, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, HiSilicon, Samsung

[4] R4-2303696, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI