3GPP TSG-RAN WG4 Meeting #112                                    R4-2412855

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:            General discussion on NES RRM impact

Agenda item:			8.21.2

Document for:	Discussion

1. Introduction

In RANP, the objectives for NES were agreed in [1] and updated in [2]. In this meeting, we will give some general discussions for the first and third objective captured as below.

[RP-241650] The first objective of NES WI regarding on-demand SSB - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. The third objective of NES WI regarding adaptation of common signal/channel transmissions - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

2 Backgrounds

2.1 Brief introduction of NES

Start with the energy consumption model, shown as below (equ. (1) and Fig. 1)

Etotal=Eactive+Esleep+Etransition, (1)

Fig.1 The illustration of energy consumption model

the intention of the NES is to reduce the overall energy consumption Etotal. Since the sleep state can allow NW to apply resources muting on transmission and/or reception or to turn off some of the high energy demanding HW components for a period to obtain the huge energy reduction consequently, how to provide/extend NW more sleep opportunities is appealing.  In time domain, the common signals/channel: SSB, SIB1, other SI, paging, and PRACH are transmitted/received by NW periodically to ensure that the cell detection/measurement, UE paging message reception and random access can be performed successfully even if there is no data transmission. However, such common signals occupy a certain amount of symbols, particually, as the SSB indexes increase in high frequency range, the active time incurred by SSB symbols increase, NW has to wake up from sleep frequently and will not get the chance to be in (deep) sleep state for energy saving. From this, on-demand SSB SCell operation and adapting the transmission of common signals/channel (i.e., SSB, paging and PRACH here) to provide possibility for NW sleeping deeper and longer in between/after transmission occassions, especially for low/medium load scenarios, is rather important.

2.2 Discussion progress in RAN1 regarding on-demand SSB Scell operation

In RAN1#116bis and RAN1#117 meetings, some agreements have been achieved as below:

For the scenarios where on-demand SSB applied, Scenario#2 and Scenario#2A with both Case#1 and Case#2 have been agreed, while other Scenarios are still FFS.

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Based on RAN1 discussion and the agreements above, different scenarios means the different time in which on-demand SSB transmitted. In order to understand each scenario clearly, a summary figure could be described as Fig.2.

Fig.2 Descriptions of scenarios mentioned in RAN1 discussion

Furthermore, Case #1 means only on-demand SSB is transmitted, but without the legacy alway-on SSB. Case #2 means both on-demand SSB and legacy always-on SSB are transmitted. Fig.3 summarizes the main difference between Case #1 and Case #2.

Fig.3 Descriptions of cases mentioned in RAN1 discussion

Observation 1: For on-demand SSB deployment, Scenario#2 and Scenario#2A with both Case#1 and Case#2 have been agreed in RAN1 discussion, while other Scenarios are still FFS.

About the signaling to indicate on-demand SSB transmission, agreements are as following and could be summarized:

- Further discussion on separate signaling or single signaling are expected;

- RRC based and MAC-CE based signaling are supported, FFS on the DCI based signaling.

Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable

Observation 2: About the signaling to indicate on-demand SSB transmission, further discussion on separate signaling or single signaling are expected, and RRC based and MAC-CE based signaling are supported, FFS on the DCI based signaling.

For the on-demand SSB indicated via MAC CE, the on-demand SSB burst(s) is expected to be transmitted from time instance A, however the details need further discussion in following meetings.

Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Observation 3: For the on-demand SSB indicated via MAC CE, the on-demand SSB burst(s) is expected to be transmitted from time instance A, however the details need further discussion in following RAN1 meetings.

For the measurement of on-demand SSB, the latest status are L1 measurement should be supported at least as below. No conclusion on whether L3 measurement based on on-demand SSB is supported.

Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Observation 4: For the measurement of on-demand SSB, L1 measurement based on on-demand SSB should be supported at least, and no conclusion on L3 measurement based on on-demand SSB.

2.3 Discussion progress in RAN1 regarding adaptation of common signal/channel transmissions

In #106 and #106-bis, RAN1 had some initial discussions on adaptation of SSB, paging and PRACH in time domain, some summaries can be found as follows.

2.3.1 Adaptation of SSB in time domain

- For adaptation transmission mechanisms, discussions on (1) SSB burst level adaptation (i.e., SSB burst non-uniform adaptation transmission, adaptation/new SSB burst periodicity value), (2) SSB beam level adaptation (i.e., SSB adaptation transmission within a burst), and (3) combination (i.e., adaptation based on two SSB configurations where up to two configurations can be active and cell DTX) are ongoing.

- For the applicable/supported scenarios, the following agreements were achieved with FFS. So far, any RRC state (idle/inactive mode and/or connected mode) and any carrier operation (PCell and/or SCell(s)) are not precluded.

Agreement in #116 For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) Agreement in #116-bis Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

- For adaptation transmission indication, the following agreement was achieved. Specifically, the indication via dynamic signalling in idle mode is considered as a promising way and extensively discussed.

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

Observation 5: Based on RAN1 agreements, SSB adaptation triggered by NW is taken as a preferred solution

2.3.2 Adaptation of paging in time domain

The discussions on techniques for adaptation of POs/PFs with new paging configuration, e.g., gather/confined together the POs in each DRX cycle within a short duration, and concentrated/compact/confine distributed PFs are initiated but without any strong agreement achieved.

2.3.3 Adaptation of PRACH in time domain

- For adaptation transmission mechanisms, adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs were confirmed with the critical Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs, while how to realize the adaptation of PRACH in time-domain is still under discussion

- For RRC states applicability, the following agreement was achieved

Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Observation 6: For the applicability of PRACH adaptation, UEs in idle/inactive mode and connected mode UEs are both supported

- For SSB-RO association, it was agreed that SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs with the Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

- For adaptation transmission indication, configuration of additional PRACH resources is provided by semi-static signalling

3 Discussion

3.1 Applicable scenarios analysis for on-demand SSB

Considering Scenario #2 and Scenario #2A have been agreed in RAN1 discussion, while Scenario #3A and Scenario #3B are still FFS, it would be better if RAN4 discussion could prioritize Scenario #2 and Scenario #2A with case #1 and case #2. For more details, Scenario #2 with case #1 and case #2 is described in Fig.4, and Scenario #2A with case #1 and case #2 is described in Fig.5.

Proposal 1: For on-demand SSB, prioritize the discussion in RAN4 based on Scenario #2 and Scenario #2A with case #1 and case #2.

Fig.4 On-demand SSB based Scenario #2 with case #1 and case #2

Fig.5 On-demand SSB based Scenario #2A with case #1 and case #2

3.2 Applicable scenarios analysis for SSB adaptation transmission

3.2.1 RRC states (UEs in inactive/idle mode analysis)

Regarding which of the scenarios to support for SSB adaptation transmission in time-domain, companies achieve alignment on the scenario of UE in connected mode, while whether to support UEs in idle/inactive mode was discussed in RAN1 without conclusions yet. Majority think it is reasonable and feasible to support UE in idle/inactive mode, while the minority have concern on the backward-compatible issue, they gauge the adaptation of CD-SSB based on sparse periodicity for NES capable UE would large impact legacy UE initial access/initial cell selection performance, e.g., the worst case is that the legacy UE with a typical 20 ms CD-SSB can not access and connect to the NW/cell, shown in the following figure (legacy case 3 and legacy case 4). However, from our understanding, to support UE in idle/inactive mode is possible. The main reasons are as follows with R19 possible/feasible cases 1-3 and legacy feasible cases 1-2 shown in the Fig.6.

- The NES features can be implemented in carriers that legacy UEs are barred based on NW choice

- When Rel-19 NES cell within the coverage of normal cell 2, although the legacy UE can’t access the NES cell, it still has opportunity to access a normal cell correctly, no huge negactive impact to legacy UE

- Actually, 20 ms for cell search/initial cell selection is just a default configuration assumption, it is not specified in the current Spec. officially, that is, it is not forbidden in the current Spec. for NW to operate a SSB with longer periodicity (larger than 20 ms) on PCell at least under low load traffic. In other words, the legacy UE can still detect SSBs with long periodicity on NES cell to camp on

Fig.6 Feasibility analyis for UE in idle mode scenario

Observation 7: For the adaptation of SSB in time-domain, it is possible to support the scenario of UEs in idle/inactive mode.

For connected mode, based on RAN1 online discussions and conclusions, seems that companies agree to support UEs in connected mode, so we think

Proposal 2: Since RAN1 agree to support UEs in connected mode, for adaptation mechanisms of SSB in time-domain, the RRM requirements for the scenarios of Rel-19 NES-capable UEs in connected mode can be discussed at current stage

3.2.2 PCell and/or Scell (s)

First, based on the agreements above mentioned, so far, RAN1 cannot reach consensus on which the following scenarios shall be considered: Rel-19 NES-capable UE’s PCell and/or Rel-19 NES-capable UE’s SCell. At the same time, from our understanding, SSB on-demand transmission is one kind of SSB adaptation transmission mechanisms from the rationale perspective, which mainly focus on SCell operation and is most likely triggered by NW with an on-demand SSB request. The NW will trigger the on-demand denser SSB transmission (shorter SSB periodicity) for a certain duration for SCell activation and measurements. From this, 1) It is too early to conclude that whether to define requirements for a PCell and/or SCell(s), i.e., multi-carrier scenario, from RAN4 side; 2) Considering the similarity in between SSB on-demand and SSB adaptation transmission to some extent, even though to define the corresponding SCell measurment/activation requirements for SSB adaptation, how to define the requirements, how to consider the requirements compatibility and whether to define conditions for the two technologies are vague at current stage.

Proposal 3: RAN4 to wait for RAN1 progress on which scenario (s): PCell and/or SCell(s) are supported for adaptation mechanisms of SSB in time-domain

Proposal 4: If Rel-19 NES-capable UE’s SCell (s) are supported for adaptation of SSB in time-domain, suggest to wait and combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

3.2.3 Deployment assumption to define the requirement

Before going deeper into the analysis of RRM requirements impact, we do feel compelled to provide some discussions on the deployment assumption, and which applicable case(s) can be determined to define RRM requirements.

- Deployment scenario 1: Intra-cell

There is a Note in RAN1 scenario discussion

Support adaptation mechanisms of SSB in time-domain for following scenarios: - UE in idle/inactive mode - UE in connected mode - PCell and SCell(s) - Note: applicable for the following cases - cell with both legacy and Rel-19 UE - cell with only Rel-19 UEs

Based on this, from our understanding, the intra-cell can be splitted into two applicable cases, only single TRP is, shown as Fig.7

:

- Serving cell with only Rel-19 NES-capable UE                        (b)  Serving cell with both legacy and Rel-19 NES-capable UE

Fig.7 Applicable cases for intra-cell deployment assumption

- Applicable case 1-1: NES only cell case. Only Rel-19 NES-capable UE is served in the cell area. The legacy UEs can be barred based on NW choice or are forbidden to camp on the NES cell via other ways. In such case, the new NES-specific requirements (if any) can be defined following the same basic logic considered for legacy UE.

- Applicable case 1-2: NES cell case. Both legacy and Rel-19 NES-capable UEs are served in the cell area. Adaptation based on configuration of additional SSB/Paging/PRACH resources for Rel-19 NES-capable UE in addition to the resources for legacy UE. That is the Rel-19 NES-capable UE can “see” both adaptation resource configurations and legacy resource configurations (i.e., the resources for legacy UE), while the legacy UE can only “see” legacy resource configurations. In such applicable case, since up to two configurations can be active, the new occasion/resources overlapping issues may arise if the additional adaptation resources overlap with legacy resources. In this case, miscellaneous factors and new measurement rules will be introduced case by case.

Observation 8: Based on RAN1 agreements, there are two applicable cases for intra-cell deployment. 1) Serving cell with only Rel-19 NES-capable U E; 2) Serving cell with both legacy and Rel-19 NES-capable UE

Observation 9: For the case of serving cell with both legacy and Rel-19 NES-capable UE assumption, occasion/resources collision issues may arise if the additional adaptation resources overlap with the legacy resources

- Deployment scenario 2: Inter-cell

From our understanding, there could be an applicable case for inter-cell deployment is that Rel-19 NES (only) cell within the coverage of sync normal cell 1, shown in Fig.8. In such case,

- For Rel-19 NES-capable UE.

  - If the cell 2 is a NES only cell, the Rel-19 NES-capable UE can derive adaptation resources configurations of cell 2, meanwhile, they can also derive legacy resources configurations of normal cell 1.

  - If the cell 2 is a NES cell, the Rel-19 NES-capable UE can derive both additional adaptation resources and legacy resources configurations of cell 2, meanwhile, they can also derive legacy resources configurations of normal cell 1. How to handle more complicated resources overlapping issue shall be discussed.

Fig.8 Applicable cases for inter-cell deployment assumption

In summary, considering different deployments and applicable cases may bring different/new impacts on RRM requirements, we prefer that

Proposal 5: For adaptation of common signal/channel transmissions, RAN4 to discuss:

- Which deployment assumption should be considered to define the new requirements (if any)

  - Intra-cell and/or inter-cell deployment

- For the deployment assumption, which applicable case should be considered to define the new requirements (if any)

  - Serving cell with only Rel-19 NES-capable UE and/or serving cell with both legacy and Rel-19 NES-capable UE

3.3 Potential RRM requirements impact analysis

First of all, from our understanding, no matter whether to/how to define requirements for Rel-19 NES-capable UEs configuring on-demand SSB and/or common signals/channels adaptation resources under which applicable scenarios/deployments. Theoretically, it should be no impact to the legacy UE.

3.3.1 Idle mode

In idle mode, the following main behaviours can be found: 1) Cell selection; 2) Cell re-selection; 3) Paging reception; 4) Initial access from RRC_IDLE. For initial access, the PRACH resources and the threshold for SSB selection for PRACH resource association is configurable by network.

However, Based on RAN1 progress:

- How to adapt the SSB transmission is left open. The adaptation designs can be a new SSB transmission pattern (e.g., compact/compress/squeeze SSB in time domain) or new/changeable SSB transmission periodicity (e.g., long/long-short/longer periodicity). Different backup adaptation transmission solutions may present different RRM requirements impact and different level of impact. For example, if new SSB transmission periodicities are configured on top of the existing ones {5 ms,…,160 ms} or new SSB patterns are configured, in this sense, new measurement timing window may be defined instead of reusing the SMTC easily, the existing measurement rules including measurement interval, granularity, and measurement result filtering, etc may be modified and the detection/measurement/evaluation periods may be re-defined accordingly. While, if the adapted periodicity is the subset of the existing ones, from our understanding, level of impact will be relatively small. Besides, if two SSB configurations are active (one for NES, one for legacy), resources overlapping in between legacy resources and additional adaptation resources may not be ignored.

Observation 10: For adaptation of SSB in time domain, different adaptation mechanisms may present different RRM requirement impacts and different level of impact

- Based on the discussions above mentioned, whether the scenarios of UEs in idle/inactive can be supported is still FFS. From this, before discussing RRM requirements impact in idle mode,

Proposal 6: For adaptation of SSB in time domain, RAN4 to wait for RAN1 progress on whether the scenarios of UEs in idle/inactive can be supported

- RA procedure can be performed by the UE in both of RRC_IDLE/INACTIVE state and RRC_CONNECTED state. RAN1 are discussing the RO periodicity/pattern design for whatever idle mode or connected mode. The contention-based PRACH resources can be used for initial access, SR failure process, etc. However, no RA procedure requirements in idle mode were defined in the current Spec. TS 38.133.

Observation 11: From RAN4 side, no RA procedure requirements in idle mode were defined in the current Spec. TS 38.133

3.3.2 Connected mode

- On-demand SSB

As the WID mentioned, on-demand SSB is targeting on SCell operation in connected mode configured with CA. Hence the measurements and procedures in connected mode may be impacted.

For the measurements, the agreement in RAN1 is L1 measurements based on on-demand SSB should be supported at least, no conclusion for L3 measurements. Therefore, whether L3 measurements based on on-demand SSB is needed or not from RAN4 perspective need to be clarified.

Proposal 7: For on-demand SSB, RAN4 need to clarify if L3 measurements based on on-demand SSB is needed or not from RAN4 perspective, and discuss the impacts on L1 measurements.

For the procedures, related requirements for SCell activation and link recovery should be updated according on-demand SSB.

Proposal 8: For on-demand SSB, related requirements for SCell activation and link recovery should be updated accordingly.

- Adaptation of SSB in time domain

In connect mode, a lot of RRM requirements in relation to SSB based measurement, from our understanding, at least the following (DL related only, TRP specific is not considered):

Observation 12: For adaptation of SSB in time domain in NES, the following requirements may be impacted:

- Section 6: RRC_CONNECTED state mobility. 6.1.1 Handover; 6.2 RRC Connection Mobility Control

- Section 8: Signalling characteristics. 8.1.2 Requirements for SSB based radio link monitoring; 8.3 SCell Activation and Deactivation Delay; 8.5.2 Requirements for SSB based beam failure detection; 8.5.5 Requirements for SSB based candidate beam detection; 8.9 NR-DC: PSCell Addition and Release Delay; 8.10 Active TCI state switching delay; 8.11 PSCell Change;

- Section 9: Measurement Procedure. 9.2 NR intra-frequency measurements (including cell identification, measurement);9.3 NR inter-frequency measurements (including cell identification, measurement); 9.5 L1-RSRP measurements for Reporting; 9.8 L1-SINR measurements for Reporting; 9.13 L1-RSRP measurements for a cell with different PCI from serving cell [inter-cell, mTRP]

From this, SSB measurement related RRM requirement impact need to be considered in possible priority order to avoid huge workload.

Proposal 9: In connected mode, the impact on the following RRM requirements can be studied on adaptation of SSB in time domain with high priority under the applicable scenario

- L1 measurements including L1-RSRP/BFD/CBD/L1-SINR

- L3 measurement including intra-frequency and inter-frequency

- SCell activation and deactivation delay

Besides, the same issues discussed in Idle mode regarding: different adaptation mechanisms may present different RRM requirements impact and different level of impact may also arise in connected mode. Then we prefer that

Proposal 10: On adaptation of SSB in time domain for NES, how to define RRM requirements (if any) should depend on RAN1 progress on SSB adaptation mechanism discussions.

- Adaptation of PRACH in time domain

Based on 38.300, a lot events can trigger RA procedure. Thereinto, two aspects might be focused on: timeline and SSB-RO association. From our understanding, at least the following:

Observation 13: For adaptation of PRACH in time domain in NES, the following requirements shown in Table 1 may be impacted

Table 1 Potential Spec. impact brought by adaptation of PRACH in time domain

| Requirements | Impacted Parameter | Comments |
| --- | --- | --- |
| Random Access | - Correct behaviour when transmitting Random Access Preamble for CBRA - Correct behaviour when transmitting Random Access Preamble for non-contention based random access | - Wait for RAN1 progress on - Collision/Overlap between additional PRACH resources and PRACH resources for legacy UE. - Adaptation at PRACH association period - SSB-RO mapping for the additional PRACH resources |
| Handover Delay | TIU/TIU_PCell/TPCell_DU/TIU_PSCell/TPSCell_DU |  |
| NR PSCell Addition/Release/Change | TPSCell_DU |  |
| RRC connection release with redirection to NR | TRACH |  |
| UE Re-establishment delay requirement | TPRACH |  |

Proposal 11: On adaptation of PRACH in time domain for NES, how to define RRM requirements connected mode (if any) should depend on RAN1 progress on:

- Collision/Overlap between additional PRACH resources for NES-capable UEs and PRACH resources for legacy UEs

- Adaptation at PRACH association period

- SSB-RO mapping for the additional PRACH resources

4 Conclusion

In this contribution, we provided our initial viewpoints to requirement impact analyses on on-demand SSB and common signals/channels adaptation. The following observations and proposals are obtained:

Observation 1: For on-demand SSB deployment, Scenario#2 and Scenario#2A with both Case#1 and Case#2 have been agreed in RAN1 discussion, while other Scenarios are still FFS.

Observation 2: About the signaling to indicate on-demand SSB transmission, further discussion on separate signaling or single signaling are expected, and RRC based and MAC-CE based signaling are supported, FFS on the DCI based signaling.

Observation 3: For the on-demand SSB indicated via MAC CE, the on-demand SSB burst(s) is expected to be transmitted from time instance A, however the details need further discussion in following RAN1 meetings.

Observation 4: For the measurement of on-demand SSB, L1 measurement based on on-demand SSB should be supported at least, and no conclusion on L3 measurement based on on-demand SSB.

Observation 5: Based on RAN1 agreements, SSB adaptation triggered by NW is taken as a preferred solution

Observation 6: For the applicability of PRACH adaptation, UEs in idle/inactive mode and connected mode UEs are both supported

Observation 7: For the adaptation of SSB in time-domain, it is possible to support the scenario of UEs in idle/inactive mode

Observation 8: Based on RAN1 agreements, there are two applicable cases for intra-cell deployment. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE

Observation 9: For the case of serving cell with both legacy and Rel-19 NES-capable UE assumption, occasion/resources collision issues may arise if the additional adaptation resources overlap with the legacy resources

Observation 10: For adaptation of SSB in time domain, different adaptation mechanisms may present different RRM requirement impacts and different level of impact

Observation 11: From RAN4 side, no RA procedure requirements in idle mode were defined in the current Spec. TS 38.133

Observation 12: For adaptation of SSB in time domain in NES, the following requirements may be impacted:

- Section 6: RRC_CONNECTED state mobility. 6.1.1 Handover; 6.2 RRC Connection Mobility Control;

- Section 8: Signalling characteristics. 8.1.2 Requirements for SSB based radio link monitoring; 8.3 SCell Activation and Deactivation Delay; 8.5.2 Requirements for SSB based beam failure detection; 8.5.5 Requirements for SSB based candidate beam detection; 8.9 NR-DC: PSCell Addition and Release Delay; 8.10 Active TCI state switching delay; 8.11 PSCell Change;

- Section 9: Measurement Procedure. 9.2 NR intra-frequency measurements (including cell identification, measurement);9.3 NR inter-frequency measurements (including cell identification, measurement); 9.5 L1-RSRP measurements for Reporting; 9.8 L1-SINR measurements for Reporting; 9.13 L1-RSRP measurements for a cell with different PCI from serving cell [inter-cell, mTRP]

Observation 13: For adaptation of PRACH in time domain in NES, the requirements shown in Table 1 may be impacted

Proposal 1: For on-demand SSB, prioritize the discussion in RAN4 based on Scenario #2 and Scenario #2A with case #1 and case #2.

Proposal 2: Since RAN1 agree to support UEs in connected mode, for adaptation mechanisms of SSB in time-domain, the RRM requirements for the scenarios of Rel-19 NES-capable UEs in connected mode can be discussed at current stage

Proposal 3: RAN4 to wait for RAN1 progress on which scenario (s): PCell and/or SCell(s) are supported for adaptation mechanisms of SSB in time-domain

Proposal 4: If Rel-19 NES-capable UE’s SCell (s) are supported for adaptation of SSB in time-domain, suggest to wait and combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

Proposal 5: For adaptation of common signal/channel transmissions, RAN4 to discuss:

- Which deployment assumption should be considered to define the new requirements (if any)

  - Intra-cell and/or inter-cell deployment

- For the deployment assumption, which applicable case should be considered to define the new requirements (if any)

  - Serving cell with only Rel-19 NES-capable UE and/or serving cell with both legacy and Rel-19 NES-capable UE

Proposal 6: For adaptation of SSB in time domain, RAN4 to wait for RAN1 progress on whether the scenarios of UEs in idle/inactive can be supported

Proposal 7: For on-demand SSB, RAN4 need to clarify if L3 measurements based on on-demand SSB is needed or not from RAN4 perspective, and discuss the impacts on L1 measurements.

Proposal 8: For on-demand SSB, related requirements for SCell activation and link recovery should be updated accordingly.

Proposal 9: In connected mode, the impact on the following RRM requirements can be studied on adaptation of SSB in time domain with high priority under the applicable scenario

- L1 measurements including L1-RSRP/BFD/CBD/L1-SINR

- L3 measurement including intra-frequency and inter-frequency

- SCell activation and deactivation delay

Proposal 10: On adaptation of SSB in time domain for NES, how to define RRM requirements (if any) should depend on RAN1 progress on SSB adaptation mechanism discussions.

Proposal 11: On adaptation of PRACH in time domain for NES, how to define RRM requirements (if any) in connected mode should depend on RAN1 progress on:

- Collision/Overlap between additional PRACH resources for NES-capable UEs and PRACH resources for legacy UEs.

- Adaptation at PRACH association period

- SSB-RO mapping for the additional PRACH resource

5 References

[1] RP-234061, “New WID: Enhancements of network energy savings for NR”, Ericsson

[2] RP-241650, “Revised WID: Enhancements of network energy savings for NR”, RAN#104 meeting, Ericsson, Apple

[3] RP-241182, “Status Report for WI – Enhancements of network energy saving for NR”, RAN#104 meeting, Ericsson, Apple