3GPP TSG-RAN WG4 Meeting #112	R4-2412866

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Satellite Trajectory Motion for NTN test cases

Agenda item:	8.4.4

Document for:	Discussion

Introduction

In RAN #101, RAN plenary has approved a simplification of the test framework for NGSO, amidst concerns of the time to complete the work item in RAN5 by the end of the Release 17. As a consequence, the specification work in RAN4 has concluded that the test cases for NGSO scenarios would be tested against a fixed (non-varying) propagation delay and fixed doppler delay.

As the speed of satellite in NGSO scenarios is a very relevant aspect of the NTN scenario that has not been covered by similar testing before and considering that NTN is a growing and relevant market, RAN has approved the WID RP-24181 (NR_IoT_NTN_req_test_enh) to develop more meaningful and relevant test cases for this feature considering the motion trajectory of the satellite.

The following objectives were established for this WI:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

Discussion

The objectives for this WI in RRM include the impact of the NGSO trajectory motion in the transmit timing test cases. The WI also discusses on the need to check whether it is possible to implement the new test configuration without the need to change the core requirement.

On maintaining the current core requirements

In fact, the RRM requirements were initially developed considering the satellite trajectory and the performance test cases were first developed with satellite trajectory motion in mind, where a propagation model (Eckstein Hechler) was adopted. The adaptation to a simplified test case actually caused the performance requirement to be adapted with the removal of a margin of the transmit timing accuracy when the simplified test case is adopted. Therefore, there is no need to adapt the core requirements by introducing the satellite trajectory motion in the requirements.

Confirm that the core requirements for transmit timing do not need to be modified if a trajectory satellite motion is adopted in NTN test case configuration.

On the selection of satellite trajectory motion

There are two main options to introduce the trajectory satellite motion:

- Option 1: Utilize a set of pre-defined (in specification annex) vector of NGSO satellite ephemeris information collected from real samples to be selected by TE before the test case. The UE position in this case is also deterministic such that a meaningful UE position is selected for each vector of ephemeris information.

- Option 2: Adopt a motion trajectory model (e.g. Eckstein-Hechler) and provide general guidelines for testing equipment to design UE position and trajectory information (e.g. ensure a range for the elevation angle).  For that see Annex.

Select between one of the following options for the satellite motion trajectory in the test case:

  - Option 1: Utilize a set of pre-defined (in specification annex) vector of NGSO satellite ephemeris information collected from real samples to be selected by TE before the test case. The UE position in this case is also deterministic such that a meaningful UE position is selected for each vector of ephemeris information.

  - Option 2: Adopt a motion trajectory model (e.g. Eckstein-Hechler) and provide general guidelines for testing equipment to design UE position and trajectory information (e.g. ensure a range for the elevation angle).

On the satellite trajectory motion configuration for transmit timing test cases

There are infinite valid orbits around Earth that could be used for testing purposes. If different TEs implement different orbital parameters, it might be difficult to guarantee that all UEs are tested under the same conditions and in certain cases it might get more difficult to specify the test metrics. Therefore, it is necessary to introduce some guidance for the performance configuration.

For the configuration of the satellite assistance information, it is important to define meaningful configuration.

it is not meaningful for the transmit timing cases that the elevation angle between the UE and the serving cell is set close to 90 degrees for NGSO scenarios, as there will be almost no variation on the UE transmit timing.

For mobility cases, it is not meaningful that the serving cell is configured such that the elevation angle between the satellite and the UE is close to 90 degrees (best propagation case scenario). In special, for the distance-based triggers.

In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE.

  - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator.

  - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

On the satellite trajectory motion configuration for frequency test case

RAN4 has to decide whether it is more meaningful to test for the condition where the doppler vary the fastest (close to 90 degrees or where the absolute doppler deviation is the highest, with smaller elevation angles.

For the frequency error test cases, decide between the options below for the satellite trajectory reference configuration

  - Option 1: Select a region where the doppler vary the fastest (close to 0 degrees of elevation)

  - Option 2: Select a region where the absolute doppler deviation is the largest (smaller elevation angles)

On the satellite trajectory channel model

We note that currently in NTN performance requirements for demodulation, that a pre-compensation is applied of 0.1 ppm for all performance requirements with a constant doppler applied to a TDL channel model either according to this ppm or by approaching the maximum doppler that would be expected to be achieved; however we note that the time-variability of that doppler is not constant and follows an s-curve profile, as can be seen in [2] and extracted in figure 1. below.

Figure 1  Doppler S-Curves for LEO (NGSO) orbits with varying maximum elevations.

The doppler observed in NGSO orbits is not static and follows a time-varying S-curve pattern.

RAN4 shall discuss the introduction of a time-varying doppler for NGSO cases.

The doppler is also significantly affected within a beam from, depending on the location on earth, whereby the position of the UE  influences the delay between a scatterer and the receiver. As can be seen in figure 2. below

Figure 2: Delay distribution within a 100km x 100km beam directly below a satellite (i = 0° and ψ = 0° in the middle). Delays for points within the beam (top left), as seen by the User Equipment in the middle with ΔtSPREAD = 0.174ms (top right), as seen by the UE shifted by half of the radius with ΔtSPREAD = 0.255ms (bottom left), and as seen by the UE on the border of the beam ΔtSPREAD = 0.333ms (bottom right). Altitude = 600km
The doppler is dependent on the UE location within a beam.

This variation would be difficult to agree an approach within RAN4, therefore we propose RAN4 should make an assumption that the UE remains static in the middle of the beam.

RAN4 shall make an assumption that the UE remains static in the middle of the beam on earth.

The proposed changes should be based upon an existing NTN TDL channel and included in a similar format to the way HST tests are derived with time-varying doppler for demodulation performance requirements.

RAN4 shall take a similar approach to HST channel modelling and combine time varying doppler with a NTN TDL channel for performance requirements derivation.

Conclusion

The paper has analyzed the new NGSO test setup to be developed to include the effects of the satellite motion in the reference configuration and the following observations and conclusions were made:

Proposal 1: Confirm that the core requirements for transmit timing do not need to be modified if a trajectory satellite motion is adopted in NTN test case configuration.

Proposal 2: Select between one of the following options for the satellite motion trajectory in the test case:

a.	Option 1: Utilize a set of pre-defined (in specification annex) vector of NGSO satellite ephemeris information collected from real samples to be selected by TE before the test case. The UE position in this case is also deterministic such that a meaningfu UE position is selected for each vector of ephemeris information.

b.	Option 2: Adopt a motion trajectory model (e.g. Eckstein-Hechler) and provide general guidelines for testing equipment to design UE position and trajectory information (e.g. ensure a range for the elevation angle).

Observation 1: it is not meaningful for the transmit timing cases that the elevation angle between the UE and the serving cell is set close to 90 degrees for NGSO scenarios, as there will be almost no variation on the UE transmit timing.

Observation 2: For mobility cases, it is not meaningful that the serving cell is configured such that the elevation angle between the satellite and the UE is close to 90 degrees (best propagation case scenario). In special, for the distance-based triggers.

Proposal 3: In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE.

a.	This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator.

b.	The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

Proposal 4: For the frequency error test cases, decide between the options below for the satellite trajectory reference configuration

a.	Option 1: Select a region where the doppler vary the fastest (close to 0 degrees of elevation)

b.	Option 2: Select a region where the absolute doppler deviation is the largest (smaller elevation angles)

Observation 3: The doppler observed in NGSO orbits is not static and follows a time-varying S-curve pattern.

Proposal 5: RAN4 shall discuss the introduction of a time-varying doppler for NGSO cases.

Proposal 6: RAN4 shall make an assumption that the UE remains static in the middle of the beam on earth.

Proposal 7: RAN4 shall take a similar approach to HST channel modelling and combine time varying doppler with a NTN TDL channel for performance requirements derivation.

- Annex

The reference propagator must be such that the DP does not cause the reference TA to deviate significantly from the UL timing expected by the gNB during the reception of UL signals. It is fair to assume that the best way to measure that is if the “reference measurement” is as close as the gNB timing as possible. The dilution of precision was investigated during the SI phase and investigated in  [5] from which the table below is extracted:

|  |  |  | Maximum errors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Without fitting | Model | Duration of prediction (minute) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
|  | Kepler | 3 | 261 | 1.74 | 2.64 | 17.60 |
|  | Eckstein Hechler (J6) | 21 | 303 | 2.02 | 0.48 | 3.20 |
|  | 2x2 num | 22,5 | 301.4 | 2.01 | 0.42 | 2.80 |
|  | 6x6 num | 27 | 302.5 | 2.02 | 0.44 | 2.93 |
|  |  |  | Maximum errors |  |  |  |
|  | Model | Duration of prediction (second) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
| Without fitting | Kepler | 45 | 43.4 | 0.29 | 0.689 | 45.93 |
|  | Eckstein Hechler (J6) | 300 | 43.4 | 0.29 | 0.093 | 6.20 |
|  | 2x2 num | 300 | 44.3 | 0.30 | 0.1 | 6.67 |
|  | 6x6 num | 350 | 44.2 | 0.29 | 0.084 | 5.60 |
| With fitting | TLE | 370 | 44,1 | 0.29 | 0.1 | 6.67 |
|  | Eckstein Hechler (J6) | 390 | 44,2 | 0.29 | 0.083 | 5.53 |
|  | 2x2 num | 375 | 44 | 0.29 | 0.092 | 6.13 |
|  | 6x6 num | 395 | 44,2 | 0.29 | 0.079 | 5.27 |

Based on this reference, we prefer to adopt a precise model, such as the Eckstein-Hechler as the reference model. Presuming the epoch time represents the most up-to-date information broadcasted by the network, at epoch time the reference satellite assistance information is that broadcasted in SIB19. The interval between epoch times is at most 10.24 s, according to the current definition provided in 3GPP TS38.331.