3GPP TSG-RAN WG4 Meeting #112	R4-2412867

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On RRM scope of work for less than 5 MHz

Agenda item:	8.8.3.4

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this contribution we discuss some aspects related to the work of specifying less than 5 MHz operation for NTN.

Discussion

In the WI description, RAN has provided clear guidelines to leverage the work already done for less than 5MHz channel bandwidth in Rel-18. One of the main aspect related to the less than 5 MHz operation is the operation with punctured SSB, where the effect SSB length is reduced from 20 PRBs to 12 PRBs and the information contained in PBCH is punctured across several PRBs.

Because of this puncturing, the time for acquisition of an unknown cell has been expanded in the less than 5 MHz work item, and this has impacted the time to detection, for example:

##### 6.1.1.2.2 Interruption time [TS 38.133] The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: […] T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

This problem is likely to be observed in NTN operation below 5 MHz. Therefore, RAN4 should address this issue.

Decide whether the time to detect an unknown cell is increased when a punctured SSB is used in less than 5 MHz.

Another issue is related to RLM/BFD due to reduced bandwidth, caused by the punctured SSBs

Investigate whether RLM/BFD thresholds need to be updated for operation with less than 5 MHz in NTN.

Conclusion

The paper has analyzed the initial scope of the work for adopting the less than 5 MHz for the NTN UEs in FR1 and the following proposals were made.

Proposal 1: Decide whether the time to detect an unknown cell is increased when a punctured SSB is used in less than 5 MHz.

Proposal 2: Investigate whether RLM/BFD thresholds need to be updated for operation with less than 5 MHz in NTN.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- Annex

The reference propagator must be such that the DP does not cause the reference TA to deviate significantly from the UL timing expected by the gNB during the reception of UL signals. It is fair to assume that the best way to measure that is if the “reference measurement” is as close as the gNB timing as possible. The dilution of precision was investigated during the SI phase and investigated in  [5] from which the table below is extracted:

|  |  |  | Maximum errors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Without fitting | Model | Duration of prediction (minute) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
|  | Kepler | 3 | 261 | 1.74 | 2.64 | 17.60 |
|  | Eckstein Hechler (J6) | 21 | 303 | 2.02 | 0.48 | 3.20 |
|  | 2x2 num | 22,5 | 301.4 | 2.01 | 0.42 | 2.80 |
|  | 6x6 num | 27 | 302.5 | 2.02 | 0.44 | 2.93 |
|  |  |  | Maximum errors |  |  |  |
|  | Model | Duration of prediction (second) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
| Without fitting | Kepler | 45 | 43.4 | 0.29 | 0.689 | 45.93 |
|  | Eckstein Hechler (J6) | 300 | 43.4 | 0.29 | 0.093 | 6.20 |
|  | 2x2 num | 300 | 44.3 | 0.30 | 0.1 | 6.67 |
|  | 6x6 num | 350 | 44.2 | 0.29 | 0.084 | 5.60 |
| With fitting | TLE | 370 | 44,1 | 0.29 | 0.1 | 6.67 |
|  | Eckstein Hechler (J6) | 390 | 44,2 | 0.29 | 0.083 | 5.53 |
|  | 2x2 num | 375 | 44 | 0.29 | 0.092 | 6.13 |
|  | 6x6 num | 395 | 44,2 | 0.29 | 0.079 | 5.27 |

Based on this reference, we prefer to adopt a precise model, such as the Eckstein-Hechler as the reference model. Presuming the epoch time represents the most up-to-date information broadcasted by the network, at epoch time the reference satellite assistance information is that broadcasted in SIB19. The interval between epoch times is at most 10.24 s, according to the current definition provided in 3GPP TS38.331.