3GPP TSG-RAN WG4 Meeting #112	R4-2412874

Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0530 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_ENDC_RF_FR1_enh2-Perf] CR on demodulation requirements and reference channels for 8Rx CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some errors for reference channels referred in 8Rx CA demodulation requirements cases. Remove square brackets of 8Rx CA demodulation requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - update requirements in Clause 5.2A.4.1 A.3.2.1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference channels of 8Rx CA cases are wrong and will get wrong throughputs accordingly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.4.1 A.3.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

5.2A.4	8RX requirements

5.2A.4.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.4.1-7 or Table 5.2A.4.1-8 (dependent on Baseline or Simplified SU-MIMO 8Rx Receiver), the requirements are based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.4.1-1 ~ Table 5.2A.4.1-6, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.4.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.9 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |

Table 5.2A.4.1-2: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |

Table 5.2A.4.1-3: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-20.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 10 | R.PDSCH.1-3.8 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 15 | R.PDSCH.1-20.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.1-20.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 25 | R.PDSCH.1-20.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 30 | R.PDSCH.1-21.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 35 | R.PDSCH.1-21.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 40 | R.PDSCH.1-21.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.9 |
| 45 | R.PDSCH.1-21.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 50 | R.PDSCH.1-21.5 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.5 |

Table 5.2A.4.1-4: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.3 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.5 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |

Table 5.2A.4.1-5: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |

Table 5.2A.4.1-6: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-34.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 10 | R.PDSCH.2-34.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 15 | R.PDSCH.2-34.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.2-34.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 25 | R.PDSCH.2-34.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 30 | R.PDSCH.2-34.6 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 40 | R.PDSCH.2-3.8 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.1 |
| 50 | R.PDSCH.2-35.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.0 |
| 60 | R.PDSCH.2-35.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 80 | R.PDSCH.2-35.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 90 | R.PDSCH.2-35.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 100 | R.PDSCH.2- 35.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.8 |

Table 5.2A.4.1-7: Minimum performance for multiple CA configurations, Baseline Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-1, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-1, 5.2A.4.1-3, 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-4 shall be applied for both CCs. Note 3: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-3 and 5.2A.4.1-6 shall be applied for both CCs. |  |  |

Table 5.2A.4.1-8: Minimum performance for multiple CA configurations, Simplified Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-2, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-5, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-2, 5.2A.4.1-3, 5.2A.4.1-5, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-2 and 5.2A.4.1-5 shall be applied for both CCs. Note 3: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-3 and 5.2A.4.1-6 shall be applied for both CCs. |  |  |

<< Unchanged sections omitted >>

A.3.2.1	FDD

A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<< Unchanged sections omitted >>

Table A.3.2.1.1-22: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-22.1 FDD | R.PDSCH.1-22.2 FDD | R.PDSCH.1-22.3 FDD | R.PDSCH.1-22.4 FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.50 | 0.50 | 0.50 | 0.50 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 19968 | 63528 | 83976 | 106576 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 3 | 8 | 10 | 13 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 37800 | 119448 | 160272 | 201096 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 29600 | 125136 | 167904 | 210672 |
| Max. Throughput averaged over 2 frames | Mbps | 18.970 | 60.352 | 79.777 | 101.247 |

<End of changes>