3GPP TSG-RAN WG4 Meeting #112	R4-2412877

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Discussion and initial simulation results on MIMO layer evaluation for 6Rx UE

Agenda item:	8.1.1.3.2

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements, especially for handheld UEs. In the RAN#104 meeting, the WID has been updated [2]. The latest objectives of 6Rx are as below:

Objective of Core part WI 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. Objective of Performance part WI 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - Support 4 MIMO layers at least, and follow the conclusion in core part for whether to support 6 MIMO layers - Specify the SDR requirements to support 6Rx

From the objective descriptions of 6Rx, 4 MIMO layers should be supported at least, but 6 MIMO layers can be supported once the gain and feasibility are verified. Generally, simulation/validation assumptions alignment should be achieved before the 6 MIMO layers performance gain validation of 6Rx receiver. However, since this is the third meeting for this topic (WF[3] as below) and simulation assumptions are not aligned yet, we provide our further analysis and initial simulation results for 6Rx receiver based on general demodulation performance cases in this contribution.

- Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE Issue 3-1-1: Tightening BS EVM requirement Way Forward: RAN4 to further discuss if the BS EVM requirement needs to be tightened when discussing the feasibility of 6-Layer MIMO for 6Rx UEs. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-1-3: Whether to use same requirement for handheld UE and FWA Way Forward: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices. - Sub-topic 3-2: 6-layer Support Issue 3-2-1: 6-layer Support Agreement: - Decision on support of 6 MIMO layers for handheld is deferred until performance level differences in comparison with 4 MIMO layers are known based on evaluations considering realistic antenna correlation assumptions and deployment scenarios Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature. Issue 3-2-3: Release independence of 6-layer Support Way Forward: Follow RAN4 Chair guidance and defer any release independence discussions until the requirements are stable.

- Discussion and simulation results

Therotially, rank is used to describe the characteristics of the channel quantitatively for spatial mulitiplexing, which is the number of uncorrelation/independent virtual channels. For the scenarios with the same number of Rx antennas, it’s no doubt that the performance of 6 MIMO layers could outperform 4 MIMO layers if the channel uncorrelation promised. However, once correlation between antennas are introduced, the performance of 6 MIMO layers can decrease compared with 4 MIMO layers. Hence we setup three cases to verfiy the gains for 6Rx antennas and higher ranks: 4Rx with 4 layers, 6Rx with 4 layers and 6Rx with 6 layers.

Simulation assumptions

The test parameters are as Table 2-1.

Table 2-1 test parameters for 6Rx

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| CBW and SCS |  |  | 40MHz with 30kHz |
| MCS |  |  | MCS4/MCS13/MCS19 |
| Propagation condition |  |  | TDLA30-10 |
| Antenna configuration |  |  | 8x4, 8x6 |
| Correlation configuration |  |  | XP Low, XP medium |
| cases |  |  | 8T4R-4L, 8T6R-4L, 8T6R-6L |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DMRS for PDSCH mapping type A |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | Antenna ports indexes |  | {1000-1003} for Rank 4 tests {1000-1005} for Rank 6 tests |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Considering 6Rx is targeting at support of handheld UE for NR FR1 single carrier scenario, more Rx antennas will be introduced into UE’s limited space, hence the cross polarized (XP/X-pol) antennas should be typical application for 6Rx antennas. The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB and cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at UE. Before the introduction of realistic antenna correlation assumptions and deployment scenarios, simulation results alignments based on current cross polarized (XP/X-pol) antennas correlation modelling could be a reasonable selection.

When 6Rx antennas are introduced, there will be 3 antennas with the same polarization in one direction at UE side, then the spatial correlation matrices at UE side could be described as:

RUE=1β14ββ14*1β14β*β14*1

For the low correlation scenario, both correlation factors α of gNB side and β of UE side are zero, which means ideal channel conditions with no correlation at all.

For the medium correlation scenario, both correlation factors α of gNB side and β of UE side are described in 38.101-4 Table B.2.3.2.2-1 as below, in which the value of UE side correlation factor β is 0.6.

Table B.2.3.2.2-1: The α and β parameters for cross-polarized MIMO correlation matrices Correlation Model; 1; 2; . Medium Correlation; 0.3; 0.3; 0.6; 0.2. High Correlation; 0.9; 0.9; 0.9; 0.3. Note 1: Value of α1 applies when more than one pair of cross-polarized antenna elements in first dimension at gNB side. Note 2: Value of α2 applies when more than one pair of cross-polarized antenna elements in second dimension at gNB side. Note 3: Value of β applies when more than one pair of cross-polarized antenna elements at UE side..

At the same time, TDL channel model has been standardization for demodulation performance evaluation. We propose to use current modelling of cross polarized antennas as the MIMO correlation matrices and TDLA30-10 as the propagation condition.

Proposal 1: For 6Rx performance evaluation, use TDLA30-10 as the propagation condition and cross polarized (XP/X-pol) antennas correlation modelling with low and medium correlation as the correlation configuration for the starting point.

For the MCS level, separate indexes should be covered for different modulation orders, therefore, use MCS4/13/19 as the MCS levels for performance evaluation as the beginning.

Proposal 2: For 6Rx performance evaluation, use MCS4, MCS13, and MCS19 with different modulation orders as the beginning.

Initial simulation results

Based on the test parameters in Table 2-1, our initial simulation results are summarized in Figure 2-1 - Figure 2-3 for MCS4, MCS13 and MCS19 separately.

Figure 2-1. Performance comparison for 6Rx with MCS4

Figure 2-2. Performance comparison for 6Rx with MCS13

Figure 2-3. Performance comparison for 6Rx with MCS19

In terms of 6Rx cases having the same 4 MIMO layers with 4 Rx cases, from our initial simualtion results for 6Rx receivers, it is obvious that 6Rx cases could outperform 4Rx cases with 4 MIMO layers due to more receive diversity gain, for the ideal channel conditions with no correlation cases at least, and have slightly better performance under the medium channel correlation cases for all cases with MCS4, MCS13, MCS19.

However, for 6Rx with 6 MIMO layers cases, performance gain could be observed only at higher SNR ranges compared with 6Rx with 4 MIMO layers cases, for ideal channel conditions with no correlation cases. There are rather performance loss for medium correlation cases with MCS13 and MCS19 cases.

Observations from our simulation results could be summarized as:

Observation 1: For 4 MIMO layers cases, 6Rx could outperform 4Rx antenna cases for the ideal channel conditions with no correlation cases at least, due to the diversity gain of more receiver antennas.

Observation 2: For 6Rx antenna cases, 6 MIMO layers cases could outperform 4 MIMO layers cases only at higher SNR value range using ideal channel conditions with no correlation cases.

Observation 3: For 6Rx antenna cases with XP medium correlation with MCS13 and MCS19, the performance of 6 MIMO layers cases is even worse than 4 MIMO layers cases.

Observation 4: For 6Rx antenna cases with XP medium correlation, the maximum throughput is hard to be achieved for 6 MIMO layers cases with MCS13 and MCS19.

From above observations, it seems it’s hard to get the benefit using 6Rx with 6 MIMO layers since the antenna correlation introduced in XP medium correlation deployment. Furthermore, even if we assume that some enhancements can be achieved in UE antenna deployment, so that lower correlation factors can be implemented for 6Rx, the situation has not changed much. Herein we take a lower correlation factor 0.1 as an example for XP medium correlation, the performance is described in Figure 2-4.

Observation 5: For 6Rx antenna cases with XP medium correlation, even the correlation factor downgrades from 0.6 to 0.1, the performance of 6 MIMO layers case is still worse than 4 MIMO layers case.

Figure 2-4. Performance comparison for 6Rx with enhanced medium correlation

Initial simulation results analysis

According to our initial simulation results and observations above, 6 MIMO layers could only benefit at higher SNR value range using ideal channel conditions with no correlation cases. Even if we assume 0.1 UE antenna correlation could be achieved instead of 0.6 for XP medium correlation, the performance of 6 MIMO layers case is still worse than 4 MIMO layers case. In addition, for the practical deployment, we can easily assume that it is hard to promise 0.1 UE antenna correlation factor as discussed in [4]. As a consequence of the initial performance evaluation, for handheld UE with 6Rx receiver antennas, we propose not to introduce requirements for 6 MIMO layers.

Proposal 3: For handheld UEs with 6Rx antennas, considering correlation will be introduced in realistic deployment, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our initial simulation results.

- Conclusion

In this contribution, we provide initial simulation assumptions, simulation results analysis, and views on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

Proposal 1: For 6Rx performance evaluation, use TDLA30-10 as the propagation condition and cross polarized (XP/X-pol) antennas correlation modelling with low and medium correlation as the correlation configuration for the starting point.

Proposal 2: For 6Rx performance evaluation, use MCS4, MCS13, and MCS19 with different modulation orders as the beginning.

Observation 1: For 4 MIMO layers cases, 6Rx could outperform 4Rx antenna cases for the ideal channel conditions with no correlation cases at least, due to the diversity gain of more receiver antennas.

Observation 2: For 6Rx antenna cases, 6 MIMO layers cases could outperform 4 MIMO layers cases only at higher SNR value range using ideal channel conditions with no correlation cases.

Observation 3: For 6Rx antenna cases with XP medium correlation with MCS13 and MCS19, the performance of 6 MIMO layers cases is even worse than 4 MIMO layers cases.

Observation 4: For 6Rx antenna cases with XP medium correlation, the maximum throughput is hard to be achieved for 6 MIMO layers cases with MCS13 and MCS19.

Observation 5: For 6Rx antenna cases with XP medium correlation, even the correlation factor downgrades from 0.6 to 0.1, the performance of 6 MIMO layers case is still worse than 4 MIMO layers case.

Proposal 3: For handheld UEs with 6Rx antennas, considering correlation will be introduced in realistic deployment, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our initial simulation results.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104 meeting, Huawei

[3] R4-2410751, WF on requirements for 6Rx, RAN4#111 meeting, AT&T

[4] R4-2409459, Views on MIMO layer for 6Rx UE, RAN4#111 meeting, Samsung