3GPP TSG-RAN WG4 Meeting #112	R4-2412878

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	8.16.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved, in which additional scenarios for “UE performance requirements for 8Rx” is an important objective for the enhancement of UE demodulation performance parts. The latest objectives of UE demodulation performance enhancements as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

Generally, the enhancement focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario. Since this is the first meeting for this topic, we provide our analysis and views on the test scopes and simulation assumptions in this contribution.

- Discussion

Inter-cell interference with MMSE-IRC receiver scenario

PDSCH demodulation requirements

For the inter-cell interference with MMSE-IRC receiver scenario, PDSCH demodulation test requirements has been standardized for 2Rx and 4Rx UE antennas, which are using 2x2 and 2x4 antenna configurations seperately. For 8Rx UE antennas, it’s a good starting point to use current interference model for PDSCH requirements. However, since current requirements are only based on rank 1 for the serving cell, it’s better to check if the enhancement is still focusing on rank 1. At the same time, considering 64QAM and 256QAM are more typical than 16QAM in NR, if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel may need more discussion.

Observation 1: For 2Rx and 4Rx antennas scenario, 2x2 and 2x4 antenna configurations with rank1 are used by the serving cell separately.

Observation 2: 16QAM based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel are defined in the interference model for PDSCH requirements.

Proposal 1: Clarify the antenna configurations for UE performance requirements for 8Rx in the inter-cell interference with MMSE-IRC receiver scenario, 2x8 or more configuration options.

Proposal 2: Clarify the rank value of the serving cell for UE performance requirements for 8Rx in the inter-cell interference with MMSE-IRC receiver scenario, rank1 or more configuration options.

Proposal 3: Clarify the modulation order based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel, whether 16QAM is still suitable or not.

Proposal 4: Reuse Rel-17 interference model and simulation assumptions for 2/4Rx UE as a starting point for 8Rx PDSCH demodulation requirements.

CQI reporting requirements

For the CQI reporting requirements of inter-cell interference with MMSE-IRC receiver, test requirements has been standardized for 2Rx and 4Rx UE antennas, which are using 2x2 and 2x4 antenna configurations seperately and could verify the CQI processing with inter-cell interference enough. For 8Rx UE antennas, it seems no CQI related processing difference with 4Rx cases, hence, we think it’s necessary to clarify if CQI reporting requirements are needed for 8Rx UE antennas with inter-cell interference scenario.

Proposal 5: Clarify if CQI reporting requirements are needed for 8Rx UE antennas with inter-cell interference with MMSE-IRC receiver scenario.

Intra-cell inter-user interference with MMSE-IRC receiver

For the intra-cell interference with MMSE-IRC receiver scenario, PDSCH demodulation test requirements has been standardized for 2Rx and 4Rx UE antennas with multiple UE pairing, which are using 2x2 and 4x4 antenna configurations seperately.

For 8Rx UE antennas, it’s a good starting point to use current test assumptions for MU-MIMO PDSCH demodulation requirements. For the ranks in 2Rx antennas deployment, 1+ 1 for the target UE and co-scheduled UE, while for the ranks in 4Rx antennas deployment, 2+ 2 for the target UE and co-scheduled UE. The number of Tx antennas and ranks should be specified for the simulation campaign for 8Rx with intra-cell inter-user interference.

Proposal 6: Clarify the antenna configurations for UE performance requirements for 8Rx in the intra-cell inter-user interference with MMSE-IRC receiver scenario, 2x8, 8x8 or more configuration options.

Proposal 7: Clarify the rank value of the target UE and co-scheduled UE(s) for UE performance requirements for 8Rx in the intra-cell inter-user interference with MMSE-IRC receiver scenario, rank1 or more configuration options.

Meanwhile, similar as the MMSE-ITC receiver, Enhanced receiver Type 2 also could benefit on the more Rx antennas for intra-cell inter-user interference scenario. Therefore, we need to clarify whether PDSCH demodulation requirements are needed or not for intra-cell inter-user interference with Enhanced receiver Type 2.

Proposal 8: Clarify whether PDSCH demodulation requirements are needed or not for intra-cell inter-user interference with Enhanced receiver Type 2.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Observation 1: For 2Rx and 4Rx antennas scenario, 2x2 and 2x4 antenna configurations with rank1 are used by the serving cell separately.

Observation 2: 16QAM based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel are defined in the interference model for PDSCH requirements.

Proposal 1: Clarify the antenna configurations for UE performance requirements for 8Rx in the inter-cell interference with MMSE-IRC receiver scenario, 2x8 or more configuration options.

Proposal 2: Clarify the rank value of the serving cell for UE performance requirements for 8Rx in the inter-cell interference with MMSE-IRC receiver scenario, rank1 or more configuration options.

Proposal 3: Clarify the modulation order based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel, whether 16QAM is still suitable or not.

Proposal 4: Reuse Rel-17 interference model and simulation assumptions for 2/4Rx UE as a starting point for 8Rx PDSCH demodulation requirements.

Proposal 5: Clarify if CQI reporting requirements are needed for 8Rx UE antennas with inter-cell interference with MMSE-IRC receiver scenario.

Proposal 6: Clarify the antenna configurations for UE performance requirements for 8Rx in the intra-cell inter-user interference with MMSE-IRC receiver scenario, 2x8, 8x8 or more configuration options.

Proposal 7: Clarify the rank value of the target UE and co-scheduled UE(s) for UE performance requirements for 8Rx in the intra-cell inter-user interference with MMSE-IRC receiver scenario, rank1 or more configuration options.

Proposal 8: Clarify whether PDSCH demodulation requirements are needed or not for intra-cell inter-user interference with Enhanced receiver Type 2.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO