3GPP TSG-RAN4 Meeting #112	 R4-2412879

# Maastricht, Netherlands,19 – 23 Aug, 2024

Source: 	Huawei

Title: 	draft TP to TR 38.769 for Co-existence evaluation assumptions

Agenda Item:	8.20. 1

Document for:	Approval

Introduction

At the RAN#102 meeting, the new study item on solutions for Ambient IoT was approved [1] and in RAN#103 the SID was revised [2]. The Skeleton for TR 38.769 [3] was endorsed in RAN1#116. Last meeting, TP to TR skeleton was endorsed. [4]

In this contribution, we provide the draft TP to TR 38.769 for Co-existence evaluation assumptions.

Reference

[1] RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR，Huawei，RAN#102, Dec., 2023

[2] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

[3] R1-2401795, Skeleton for TR 38.769 “Study on Solutions for Ambient IoT (Internet of Things)” v0.0.1, Huawei (TR editor), RAN 1#116, Feb. ,2024

[4] R4-2410596, draft TP to TR38.769 skeleton for RF part.docx, Huawei, RAN4#111, May 2024

Text Proposal to TR 38.769

------------Start of the Change-------------

- 6.5	Coexistence of Ambient IoT and NR/LTE

Editor’s note: Corresponds to the first RAN4 objective in the SID.

# 6.5.1 Regulation consideration

# 6.5.2 Co-existence scenario

# 6.5.3 Co-existence evaluation assumptions

General parameters are as follows:

Table 6.5.3-1. General parameters

| General Parameter | D1T1&D2T2 |
| --- | --- |
| Carrier frequency | 900 MHz (Band 8) |
| BW for NR | 10MHz with 15KHz SCS |
| Channel BW for A-IOT R2D | 180kHz，for 15kHz SCS |
| Channel BW for A-IOT D2R | 15kHz |
| Waveform（R2D） | OOK waveform generated by OFDM modulator |
| Waveform（CW） | Unmodulated single tone |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |

Layout for D1T1 are as follows:

Table 6.5.3-2. Layout for D1T1

| Parameter | Assumptions for D1T1 |
| --- | --- |
| Scenario | InF-DH |
| Hall size | 120×60 m |
| Room height | 10 m |
| Sectorization | None |
| Pathloss model | NLOS and LOS in TR38.901 |
| BS deployment / Intermediate UE dropping | For D1T1-A2: 18 BSs on a square lattice with spacing D, located D/2 from the walls. Readers transmit round-robin as the baseline L=120m x W=60m; D=20m - BS height = 8 m For D1T1-A1: -FFS on layout: one node as transmission and CW, the neighbour node as reception |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IOT device under one reader at one drop MCL (Minimum Coupling Loss) between device and reader is 45dB. |
| NR BS deployment (outdoor), i.e. scenario 1-1 and 1-2 | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory center is set as 100m. |
| NR BS Inter-site distance | 750 |
| Minimum NR BS – NR UE distance (2D) | 35 m |
| NR UE (D1T1) dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 50% indoor, 50% outdoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 |

Layout for D2T2 is as follows：

Table 6.5.3-3. Layout for D2T2

| Parameter | Assumptions for D2T2 |
| --- | --- |
| Scenario | InH-office |
| Hall size | 120×50 m |
| Room height | 3m |
| Sectorization | None |
| Pathloss model | LOS and NLOS in TR38.901 |
| BS deployment / Intermediate UE dropping | - L=120m×W=50m; - Intermediate UE height = 1.5 m For D2T2-A2: The intermediate UEs selected from the fixed positions. Number of intermediate UE for simulation: - Option 1: 2 UE at one drop - Option 2: 12 UE at one drop For D2T2-A1: FFS on layout |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IOT device under one reader at one drop Minimum distance between reader and device is 1m |
| NR BS deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory centre is set as 100m. |
| NR UE dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 80% indoor, 20% outdoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 |

For D1T1，A-IoT micro BS parameters are as follows:

Table 6.5.3-4. A-IoT micro BS parameters

| A-IoT micro BS parameters | D1T1 | note |
| --- | --- | --- |
| A-IoT micro-BS total Tx power | 33dBm or （optional）38 dBm |  |
| A-IoT micro-BS Antenna height | 8m |  |
| A-IoT micro-BS receiver Noise Figure（dB） | 10 |  |
| A-IoT micro-BS antenna gain (dBi) | 6 |  |
| A-IoT micro-BS ACLR (dB) | [ACLR1:40dB，ACLR2:50dB] | Reference to TS 36.104 NB-IoT standalone as a start point |

Indoor A-IoT BS antenna pattern is as follows:

Antenna Array Geometry：

- equals to omni-directional anenna pattern in GCG in horizontal

Table 6.5.3-5. A-IoT BS antenna pattern

| Parameter | Assumption |
| --- | --- |
| Antenna pattern (horizontal) | AHϕ=-min12ϕϕ3dB2,Am, = 90°, Am = 15 dB |
| Antenna pattern (vertical) | AVθ=-min12θ-90°θ3dB2,SLAv, = 90°, SLAv = 15 dB |
| Combining method in 3D antenna pattern |  |
| BS antenna gain (dBi) (including feeder loss) | 6 |

For D2T2，A-IoT intermediate UE parameters are proposed as follows:

Table 7-6. intermediate UE parameters

| intermediate UE parameters | D2T2 | note |
| --- | --- | --- |
| intermediate UE total Tx power（dBm） | 23 |  |
| intermediate UE Antenna height(m) | 1.5 |  |
| gain of antenna intermediate UE (dBi) | 0 |  |
| intermediate UE receiver Noise Figure（dB） | 9 |  |

Carrier wave node parameters are as follows:

Table 6.5.3-7. Carrier wave parameters

| NR macro-BS Parameter | Recommended value for D1T1 | Recommended value for D2T2 | note |
| --- | --- | --- | --- |
| CW Tx power (dBm) | 23dBm for UL spectrum, FFS 26dBm 33dBm(M), 38dBm (O) for DL spectrum Note: only applicable for device 1/2a | Inter-mediate UE Tx power is assumed. |  |
| CW Tx antenna gain (dBi) | the value equals to UE Tx ant gain, or BS Tx ant gain Note: only applicable for device 1/2a |  |  |
| CW cancellation (dB) | For [monostatic backscatter], FFS - [140dB for BS] - [120dB for UE] For [bistatic backscatter] Assuming CW has no impact to the receiver sensitivity loss. |  |  |

Device parameters are as follows:

Table 6.5.3-8. Devices parameters

| A-IoT device parameters | Device 1 | Device 2a | Device 2b | Note |
| --- | --- | --- | --- | --- |
| A-IoT device maximum Tx power (dBm) | -10 | -10 | [-10/-20] | For device 1/2a, the device backscattering power = min {-10 dBm, power of the CW signal arriving at the device after space attenuation + backcatter loss/gain} |
| Device antenna height（m） | 1.5 | 1.5 | 1.5 |  |
| A-IoT device antenna gain (dBi) | 0 | 0 | 0 |  |
| A-IoT device backscatter loss (dB) | OOK: -6 dB PSK: 0 dB | [FFS] | N/A |  |
| A-IoT device 2a backscatter amplifier gain(dB) | N/A | 10(M),15(O) | N/A |  |
| A-IoT Device receiver sensitivity (dBm) | -36 | -46 | [FFS] |  |
| A-IoT device noise figure (dB) | 24 | [FFS] | [FFS] |  |
| ACLR | 9 |  |  |  |
| BB LPF | 3rd/5th-order RC filter with cutoff frequency at half of R2D transmission bandwidth. |  |  |  |

NR outdoor macro-BS parameters are as follows:

Table 7-9. NR macro-BS parameters

| NR macro-BS Parameter | Values for co-existence simulation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |

Referring to AAS study TR 37.840 sub-clause 5.4.4, a passive antenna is currently modeled as a single radiating source according to definition in TR 36.814, Table A.2.1.1-2. The following Macro BS antenna pattern is used for A-IOT coexistence study.

Antenna Array Geometry：

- BS point at fixed beam direction

  - vertical: θtilt + 90°

  - horizontal: 0, 120, 240 °

Table 7-10. NR macro-BS antenna pattern

| Parameter | Assumption |
| --- | --- |
| Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) | AHϕ=-min12ϕϕ3dB2,Am = 65 degrees, Am = 25 dB |  |
| Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns) | AVθ=-min12θ-90°-θetiltθ3dB2,SLAv = 10 degrees, SLAv = 25 dB, = 9 degrees |  |
| Combining method in 3D antenna pattern |  |  |
| BS antenna gain (dBi) (including feeder loss) | 15 |  |

NR UE parameters are as follows:

Table 7-11. NR UE parameters

| NR UE Parameter | Recommended value | note |
| --- | --- | --- |
| NR UE Tx power (dBm) | 23 (maximum) | -40 to 23 for calibration |
| NR UE Antenna gain (dBi) | 0 |  |
| NR UE ACLR（dB） | 30 | For power class 3 NR UE |
| NR UE Noise Figure（dB） | 9 |  |
| Height of UE antenna (m) | 1.5 |  |
| Antenna configuration | Omni direction antenna |  |

Use the pathloss model in TR38.901 7.4.3.1 [4] for simulation：

For penetration loss，use the High-loss model in Table 7.4.3-2 in TR38.901：

Table 7-12. Pathloss model

| NR UE Parameter | PLb | PLin |
| --- | --- | --- |
| outdoor macro gNB to indoor device/indoor reader | UMa | 0.5*d2Din |
| outdoor legacy UE to indoor device/D2T2 reader | UMi | 0.5*d2Din |
| outdoor legacy UE to indoor D1T1 reader | UMi | 0.5*d2Din |
| indoor legacy UE <-> indoor device | indoor office | NA |
| indoor legacy UE <-> indoor reader | D1T1: InF-DH D2T2: indoor office | NA |

------------End of the Change-------------