3GPP TSG-RAN WG4 Meeting #112	R4-2412880

# Maastricht, Netherlands,19 – 23 Aug, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on A-IoT deployment scenarios and spectrum usage

Agenda Item:	8.20.2.1

Document for:	Approval

Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. Based on the agreed way forwards of the last two meetings [2,3], this contribution further discusses the remaining open issues.

Deployment scenario

Following bullets are captured in the approved WF [2, 3]:

Issue 2-1-1: deployment scenarios for D1T1

Option 1-1: Legacy NR gNB are outdoor macro gNB while A-IoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors.

Option 1-2: Legacy NR gNB are outdoor macro gNB while A-IoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB.

Option 2-1: Legacy NR gNB are co-located with A-IoT reader and CW. All of NR and A-IoT BS/UE/Reader/Device/CW are indoors. A-IoT reader /CW and Legacy gNB share same hardware

Option 2-2: Legacy NR gNB are co-located with A-IoT reader and CW. All of NR and A-IoT BS/UE/Reader/Device/CW are indoors. A-IoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Agreement in RAN4#110bis:

- RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2.

Agreement in RAN4#111:

- Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and A-IOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

  - FFS on co-site scenario (option 2-1 and 2-2)

For CW inside topology of both D1T1 and D2T2, it is common for the A-IoT reader and CW to share hardware in the CW inside topology of both D1T1 and D2T2.

Observation 1: For CW inside topology of both D1T1 and D2T2, A-IoT reader and CW are hardware sharing.

Concerning CW outside topology D1T1-B, the A-IoT Readers are mounted on the ceiling in a square lattice with a spacing of D (D=20m as assumption). To make installation more convenient, the CW node can be co-located with the neighbouring A-IoT Reader. This simplification is practical and will not impact the simulation outcomes.

Figure 2-1. D1T1 for Device 1/2a

Proposal 1: For outside topology D1T1-B, assume that CW node is co-located with the neighbouring A-IoT Reader.

Proposal 1 can benefit from simplification of the co-existence simulation if the assumption is acceptable. This means we won't have to simulate all cases, including D1T1-A1, D1T1-A2, and D1T1-B. More details will be provided in the evaluation cases section.

During the previous discussion, some companies are considering evaluate option 2-1 and option 2-2. These options involve deploying NR and A-IoT BS hardware sharing or co-locating them on the same site.

However, there are some challenges to consider. Assuming a 10MHz RF transmission bandwidth and 180kHz BB LPF, simulation shows that a PSD difference of 24dB is required to avoid affecting envelope detection demodulation. It’s is very challenge for implementation.

Figure 2-2. Simulation of Required PSD power boosting for Device 1

Furthermore, option 2-1 and option 2-2, may result in problematic interference issues. In this scenario, there may be a significant sensitivity difference between A-IoT devices (specifically Device 1 and Device 2a) and n8 NR UEs. For example, Ambient IoT Device 1 receiver reference sensitivity is about -36dBm, while n8 NR UE reference sensitivity is -97dBm. Interference between indoor NR UEs and indoor A-IoT BS may be problematic due to mutual blocking. When the interference source is the A-IoT micro-BS transmitter, and the victim is the indoor NR UE receiver, the interference signal from A-IoT BS transmitter can reach levels about -36dBm.However, the blocking level for the NR UE receiver is -56dBm, which means that some NR UE receivers may be blocked by the A-IoT micro-BS transmitting signal. Conversely, a few NR UE receivers that receive strong NR signals and are not blocked by the A-IoT may interfere with the A-IoT micro-BS receiver.

Observation 2: A PSD difference of 24dB is required for Option 2-1, which poses a significant challenge for implementation.

Observation 3: Interference between indoor NR UEs and indoor A-IoT BS for Option 2-1 and Option 2-2 may be problematic due to a significant sensitivity difference between A-IoT devices (specifically Device 1 and Device 2a) and NR UEs.

Proposal 2: Deprioritize to Option 2-1 and Option 2-2, due to very high PSD difference and interference.

Spectrum usages

R2D spectrum

Issue 2-2-1: Spectrum usage for R2D in D1T1

Agreement in RAN4#111:

- Use FDD DL as starting point for co-existence evaluation for R2D in D1T1

- FFS on FDD UL spectrum.

It was agreed that use FDD DL as starting point for R2D in D1T1. FFS on FDD UL spectrum.

According to the SID (RP-240826), Ambient IoT uses FR1 licensed spectrum in FDD, this implies that base stations transmit on the downlink spectrum and receive on the uplink spectrum, while UE transmits on the uplink spectrum and receives on the FDD downlink spectrum. R2D spectrum usage shall follow the FDD usage to get more smooth coexistence with existing NR.

In D1T1, the A-IoT BS transmits data to the A-IoT device, and the A-IoT device receives data from the A-IoT BS, therefore R2D shall use FDD downlink spectrum.

On the other hand, R2D transmit in FDD uplink spectrum in D1T1 cannot meet the spectrum regulation.

Proposal 3: R2D shall transmit in FDD downlink spectrum in D1T1.

CW2D spectrum

For CW spectrum usage, following are captured in the WF [1].

Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

· Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

· Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

· Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Agreement in RAN4#110bis:

- For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation

  - CW transmits in either UL or DL spectrum

  - FFS on inside topology and outside topology.

Agreement in RAN4#111:

- Use inside topology as starting point for co-existence evaluation (case 1-1, case 1-2) for calibration.

- Further discuss the difference of outside topology (case 1-4) from co-existence study perspective.

When CW uses the same spectrum, the pass loss difference caused by the CW node in both inside and outside topologies is random and does not impact the coexistence simulation outcome.

Proposal 4: When CW uses the same spectrum, CW inside and outside topology do not impact coexistence simulation result.

CW inside the topology denotes that CW is transmitted from the BS for D1T1. Case 1-1 meets the spectrum regulation, whereas Case 1-2, where CW is transmitted in the UL spectrum by the BS, does not appear to comply with regulation. Therefore, Case 1-1 is a more reasonable option than Case 1-2.

For Case 1-4, CW is transmitted in the uplink spectrum. Then D2R backscattering is transmitted in the same carrier as CW. This allows reusing BS hardware design for the uplink receiver, as it receives backscattered signals in the same spectrum.

Since D2R backscattering is transmitted in the same carrier as CW, R2D use the same spectrum as CW.

Proposal 5: In D1T1, use Case 1-1 and Case 1-4 as CW2D spectrum usage for the formal simulation:

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

The following is from the WF [3]:

Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied.

· Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

· Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

· Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Agreement in RAN4#110bis:

- For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

  - Use UL spectrum as the starting point for co-existence evaluation.

    - It won’t preclude the use of DL for backscattering transmission.

    - FFS on the minimum distance between the intermediate UE and A-IoT device

Agreement in RAN4#111:

- Use case 2-2 as starting point for co-existence evaluation for calibration.

- FFS on case 2-3

- Further discuss the difference of outside topology (case2-4) from co-existence study perspective.

Similar with D1T1, for D2T2, the pass loss caused by the CW node in both inside (case 2-2) and outside (case 2-4) topologies is random and does not impact the coexistence simulation outcome.

CW inside the topology indicates that CW is transmitted from the intermediate UE. Case 2-2 aligns with current spectrum regulations for the intermediate UE to transmit CW in uplink spectrum.

Considering that the carrier-wave is transmitted in uplink spectrum for Case 2-2, then D2R is also transmitted in uplink spectrum, it is reasonable and beneficial to reuse the uplink receiver of intermediate UE for both cases where the carrier-wave node is inside and outside topology in D2T2. It is recommended to transmit CW in uplink spectrum, that is Case 2-4.

Proposal 6: In D2T2, choose Case 2-2 and Case2-4 as CW2D spectrum usage:

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

D2R spectrum

It was agreed that for device 1 and device 2a, D2R use the same spectrum as CW.

For Device 2b, it generates its UL transmission internally. In D1T1-C, Reader is BS, so R2D use FDD downlink spectrum, and it is reasonable for D2R using uplink spectrum to transmit. In D2T2-C, D2R transmits in uplink spectrum will be good for a uniform device design for various cases.

Proposal 7: D2R transmits in the uplink spectrum for device 2b.

Evaluation methodology

About evaluation methodology, following bullets are copied from WF [1].

Issue 2-4-1: Evaluation methodology

Agreement in RAN4#110bis:

- Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803

- Depending on the discussion on deployment scenarios, for some cases, calculation for the worst interference link may be enough.

- FFS on whether RAN4 needs to perform link level simulation

Agreement in RAN4#111:

- Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803

- FFS on whether RAN4 needs to perform link level simulation

The open issue regarding that RAN4 requires an SINR value as a performance metric for system level simulation.

Based on our simulation，input SNR for a RF envelope detector is required to be around 7.5dB, given that the RF BW=10MHz.

Figure 4-1. Minimum input SNR for Device 1

Proposal 8: For RF BW 10MHz, R2D SINR is 7.5dB for device 1.

Performance metric for A-IoT

Following bullets are captured in the approved WF [2,3].

Issue 2-4-2: Performance metric for A-IOT

Agreement:

- For NR system, use 5% throughput loss as performance metric as legacy.

- For A-IOT system, including reader, device, intermediate UE, further discuss the performance metric:

  - Option 1: [10%] BLER, [Rx power]

  - Option 2: SINR degradation

  - Other options are not precluded

Agreement in RAN4#111:

- Use SINR for calibration purpose

- FFS on performance metric for co-existence evaluation and requirements definition.

It is agreed that for legacy NR system, use 5% throughput loss as performance metric.

Due to the unique use cases of the A-IoT, such as indoor inventory and indoor command, conventional throughput evaluation metric of legacy NR is not applicable. For A-IoT system SINR is agreed upon as the performance metric, it can also be used for formal simulations.

It is noted that the CW node is a component of the A-IoT system, and its interference is accounted for in the baseline reference.

Proposal 9: The SINR includes CW interference is used as the baseline reference before any degradation.

For D2R，metric is proposed as following：

Readers is micro-BS in D1T1 and intermediate UE in D2T2.

It is recommended [1dB] SINR degradation as D2R performance metric for A-IoT BS and [1dB] for A-IoT intermediate UE.

Proposal 10: CDF SINR degradation can serve as performance metric for A-IoT D2R. It is recommended 1dB for A-IoT BS and 1dB for A-IoT intermediate UE.

Proposal 11: For A-IoT device with receiver based on RF envelope detector, it can correctly demodulate wanted signals only when the following conditions are met:

1) SINR>= SINR threshold [7.5dB] for 10MHz RF BW;

2) the wanted signal level>=demodulation sensitivity threshold [ -36dBm for Device 1, and -46dBm for Device 2a]

Following bullets are captured in the approved WF [2,3].

Issue 2-4-2: SINR definition for D2R

Agreement in RAN4#111:

Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2

FFS on how to consider CW cancellation capability in formal simulation

In RAN1, D2R transmission bandwidth is as following:

[2a1]-Alt1 (M):

DSB

X kHz is considered for D2R transmission bandwidth.

The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz

[2a1]-Alt2:

SSB

X kHz is considered for D2R transmission bandwidth.

The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz.

For device 2b only, FFS for device 2a.

X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies

SINR definition for D2R is as following:

SINR_D2R= Signal Power in transmission bandwidth / (Noise+ Interference Power) in transmission bandwidth

For D2R, A-IoT BS is coherent receiver, transmission bandwidth is equal with the interference bandwidth.

While the D2R transmission bandwidth discussion in RAN1 is still on-going, it is known that a narrower bandwidth is used for poor coverage. The 15 kHz D2R transmission bandwidth can be used as the co-existence simulation baseline.

The NF （noise figure）of A-IoT BS is assumed with 10 dB as NR micro-BS, so the noise is negligible when compared to the interference. Thus, SINR_D2R for A-IoT BS can be simplified as SIR.

Proposal 12：The 15 kHz D2R transmission bandwidth can be used as the co-existence simulation baseline.

Proposal 13：SINR definition for D2R is as following:

SINR_D2R= Signal Power in transmission bandwidth / (Noise+ Interference Power) in transmission bandwidth,

Simplified SINR definition for D2R is as following:

SIR_D2R= Signal Power in [15kHz] transmission bandwidth / (Interference Power) in [15kHz] transmission bandwidth

Following bullets are captured in the approved WF [2,3].

Issue 2-4-2: SINR definition for R2D

Agreement in RAN4#111:

SINR for R2D for calibration purposes

- signal power of device to the noise and interference within 10MHz

  - Assume interference NR BW is 10MHz

- FFS on BB LPF

Proposal 14：SINR definition for R2D is as following:

SINR_R2D= Signal Power in transmission bandwidth / (Noise + Interference Power) in transmission bandwidth

In RAN1, R2D transmission bandwidth is as following:

| R2D specific parameters |  |
| --- | --- |
| Transmission bandwidth | 180 kHz as baseline. Other larger values are not precluded. |
| ED bandwidth | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |
| BB LPF | [X]-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. Companies to report X = {3, 5}. |

The receiver block diagram for device 1 is shown in Figure 1 below.

Figure 5-1: Receiver Block Diagram for Device 1

For device 1/2a, the antenna receives both wanted signals and broadband interference, which are then transmitted to the RF ED after passing through the broadband matching network and potentially the RF BPF. The signals remain broad bandwidth. The RF ED operates in a non-linear manner following the square law.

For R2D, A-IoT device is ED receiver, device transmission bandwidth [180kHz], while the interference RF bandwidth used in calibration is 10M.

SINR definition for R2D:

SINR_R2D= Signal Power in [180kHz] transmission bandwidth / (Interference Power) in [10M/] RF bandwidth)

Proposal 15：SINR definition for R2D:

SINR_R2D= Signal Power in [180kHz] transmission bandwidth / (Interference Power) in [10M] RF bandwidth)

Evaluation cases

Previously,20 cases for Device 1/2a were picked up and agreed for calibration as listed in Table X. Please note that the columns labeled as Note has been added for the purpose of discussion.

Table 6-1: Receiver Block Diagram for Device 1

| Deployment scenario and topology |  |  | Evaluation case No. | Note | (Aggressor -> Victim) | spectrum |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-legacy UE only outdoor | D1T1 option1-1， | 1 |  | device -> NR UL | R2D: DL CW2D and D2R: UL |
|  |  |  | 2 |  | NR UL -> reader |  |
|  |  |  | 3 |  | reader -> NR DL |  |
|  |  |  | 4 |  | NR DL -> device |  |
|  |  |  | 5 |  | device -> NR DL | R2D: DL CW2D and D2R: DL |
|  |  |  | 6 |  | NR DL -> reader |  |
| 1-2 | D1T1-legacy UE indoor | D1T1 option1-2 | 7 | Redundant, same with case 1 | device -> NR UL | R2D: DL CW2D and D2R: UL |
|  |  |  | 8 |  | NR UL -> reader |  |
|  |  |  | 9 |  | reader -> NR DL |  |
|  |  |  | 10 | Redundant, Same with case 4 | NR DL -> device |  |
|  |  |  | 11 |  | device -> NR DL | R2D: DL CW2D and D2R: DL |
|  |  |  | 12 | Redundant, Same with case 6 | NR DL -> reader |  |
| 2-3 | D2T2-A2-legacy UE only outdoor | D2T2 option1-1 | 13 |  | device -> NR UL | R2D: UL CW2D and D2R: UL |
|  |  |  | 14 |  | NR UL -> reader |  |
|  |  |  | 15 |  | reader -> NR UL |  |
|  |  |  | 16 |  | NR UL -> device |  |
| 2-4 | D2T2-A2-legacy UE indoor | D2T2 option1-2 | 17 | Redundant, Same with case 13 | device -> NR UL | R2D: UL CW2D and D2R: UL |
|  |  |  | 18 |  | NR UL -> reader |  |
|  |  |  | 19 | Redundant, Same with case 15 | reader -> NR UL |  |
|  |  |  | 20 |  | NR UL -> device |  |

Case 7, 10, 12, 17, and 19 are actually same cases with 1, 4, 6, 13, and 15, respectively. Therefore, they are redundant and can be removed from the formal simulation.

Proposal 16：Remove redundant case 7, 10, 12, 17, and 19 from Table 6-1, since they share the same result with case 1, 4, 6, 13, and 15, respectively.

For outside topology D1T1-B, assume that CW node is co-located with the neighbouring A-IoT Reader.

From simulation, the D1T1-A1, D1T1-A2, and D1T1-B (or D2T2-A1, D2T2-A2 and D2T2-B) do not affect the interference simulation result because the aggressor and victim location is randomly selected. This means that we don't need to perform simulation all of D1T1-A1, D1T1-A2, and D1T1-B (or D2T2-A1, D2T2-A2 and D2T2-B) case. The simulation does not need to distinguish -A1, -A2 and -B. Evaluation one case can applies to all -A1, -A2 and -B.

Proposal 17：No need to distinguish -A1, -A2 and -B for simulation. One case can be applied to all D1T1-A1, D1T1-A2 and D1T1-B (or D2T2-A1, D2T2-A2 and D2T2-B).

Simulation is not needed for interference from CW to NR/A-IOT. The reasons are as follows: The remaining interference after CW cancellation/suppression can be calculated, and the suppressed CW is used as the system's floor noise. CW deployment reuses Reader sites, and the interference path is similar to that of Reader. The CW output power does not exceed the Reader output power, and since CW is a single tone, interference from CW is no greater than that from Reader.

Proposal 18：Simulation is not needed for interference from CW to NR/A-IOT.

Evaluation parameters

Based on calibration parameters, we make necessary adjustments and propose following parameters & assumptions for Deployment scenario 1 with Topology 1.

Issue 4-2-1: General parameters

General parameters are proposed to be as follows:

Table 7-1. General parameters

| General Parameter | D1T1&D2T2 |
| --- | --- |
| Carrier frequency | 900 MHz (Band 8) |
| BW for NR | 10MHz with 15KHz SCS |
| Channel BW for A-IOT R2D | 180kHz，for 15kHz SCS |
| Channel BW for A-IOT D2R | 15kHz |
| Waveform（R2D） | OOK waveform generated by OFDM modulator |
| Waveform（CW） | Unmodulated single tone |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |

Issue 4-2-2: Layout for D1T1

Based on our simulation，when Readers are concurrently transmitting, the baseline SINR is poor because the co-channel interference is strong. The inter-system interference is weaker than the co-channel interference, resulting in overlapping SINR curves with and without interference, making the impact invisible. Although the SINR does not deteriorate, the overall SINR is less than 15 dB due to the strong co-channel interference.

However, when readers transmit Round Robin, the SINR is greater than 20 dB even in the presence of inter-frequency interference, meeting the requirements.

Therefore, it is recommended to use Readers Round Robin transmission as the coexistence simulation baseline, and concurrent simulation can be performed when the interference is small. From the perspective of coexistence, the Round Robin transmission scenario is more sensitive to inter-system interference and is more likely to be interfered than that of Readers concurrent transmission.

Although Round Robin inventory is inefficient, it can be optimized through implementation. For example，if the indoor factory area is large enough, concurrent transmission can be implemented through spatial isolation.

For D1T1, Readers transmit round-robin as the baseline, and concurrent transmission is not considered in the co-existence study due to mutual interference issue. Concurrent transmission of Readers is up to implementation.

Proposal 19: For D1T1, Readers transmit round-robin as the baseline, and concurrent transmission is not considered in the co-existence study. Concurrent transmission of Readers is up to implementation.

It is recommended to use following parameters for co-existence simulation, i.e. scenario 1-1 and 1-2.

Table 7-2. Layout for D1T1

| Parameter | Assumptions for D1T1 |
| --- | --- |
| Scenario | InF-DH |
| Hall size | 120×60 m |
| Room height | 10 m |
| Sectorization | None |
| Pathloss model | NLOS and LOS in TR38.901 |
| BS deployment / Intermediate UE dropping | For D1T1-A2: 18 BSs on a square lattice with spacing D, located D/2 from the walls. Readers transmit round-robin as the baseline L=120m x W=60m; D=20m - BS height = 8 m For D1T1-A1: -FFS on layout: one node as transmission and CW, the neighbour node as reception |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IOT device under one reader at one drop MCL (Minimum Coupling Loss) between device and reader is 45dB. |
| NR BS deployment (outdoor), i.e. scenario 1-1 and 1-2 | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory center is set as 100m. |
| NR BS Inter-site distance | 750 |
| Minimum NR BS – NR UE distance (2D) | 35 m |
| NR UE (D1T1) dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 50% indoor, 50% outdoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 |

Issue 4-2-3: Layout for D2T2

Agreement in RAN4#111:

Use InH-office as baseline for D2T2 co-existence evaluation.

Use following parameters for co-existence simulation, i.e. scenario 1-1 and 1-2.

Table 7-3. Layout for D2T2

| Parameter | Assumptions for D2T2 |
| --- | --- |
| Scenario | InH-office |
| Hall size | 120 ×50 m |
| Room height | 3m |
| Sectorization | None |
| Pathloss model | LOS and NLOS in TR38.901 |
| BS deployment / Intermediate UE dropping | - L=120m x W=50m; - Intermediate UE height = 1.5 m For D2T2-A2: The intermediate UEs selected from the fixed positions. Number of intermediate UE for calibration: - Option 1: 2 UE at one drop - Option 2: 12 UE at one drop For D2T2-A1: FFS on layout |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IOT device under one reader at one drop Minimum distance between reader and device is 1m |
| NR BS deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory centre is set as 100m. |
| NR UE dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 20% indoor, 80% outdoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 |

Issue 4-3-1: A-IoT micro-BS parameters for D1T1

For D1T1，A-IoT micro BS parameters are proposed as follows:

Table 7-4. A-IoT micro BS parameters

| A-IoT micro BS parameters | D1T1 | note |
| --- | --- | --- |
| A-IoT micro-BS total Tx power | 33dBm or （optional）38 dBm |  |
| A-IoT micro-BS Antenna height | 8m |  |
| A-IoT micro-BS receiver Noise Figure（dB） | 10 |  |
| A-IoT micro-BS antenna gain (dBi) | 6 |  |
| A-IoT micro-BS ACLR (dB) | [ACLR1:40dB，ACLR2:50dB] | Reference to TS 36.104 NB-IoT standalone as a start point |

Indoor A-IoT BS antenna pattern:

Antenna Array Geometry：

- equals to omni-directional antenna pattern in GCG in horizontal

Table 7-5. A-IoT BS antenna pattern

| Parameter | Assumption |
| --- | --- |
| Antenna pattern (horizontal) | AHϕ=-min12ϕϕ3dB2,Am, = 90°, Am = 15 dB |
| Antenna pattern (vertical) | AVθ=-min12θ-90°θ3dB2,SLAv, = 90°, SLAv = 15 dB |
| Combining method in 3D antenna pattern |  |
| BS antenna gain (dBi) (including feeder loss) | 6 |

Issue 4-3-2: Intermediate UE parameters for D2T2

For D2T2，A-IoT intermediate UE parameters are proposed as follows:

Table 7-6. intermediate UE parameters

| intermediate UE parameters | D2T2 | note |
| --- | --- | --- |
| intermediate UE total Tx power（dBm） | 23 |  |
| intermediate UE Antenna height(m) | 1.5 |  |
| gain of antenna intermediate UE (dBi) | 0 |  |
| intermediate UE receiver Noise Figure（dB） | 9 |  |

Carrier wave node parameters are proposed as follows:

Table 7-7. Carrier wave parameters

| NR macro-BS Parameter | Recommended value for D1T1 | Recommended value for D2T2 | note |
| --- | --- | --- | --- |
| CW Tx power (dBm) | 23dBm for UL spectrum, FFS 26dBm 33dBm(M), 38dBm (O) for DL spectrum Note: only applicable for device 1/2a | Inter-mediate UE Tx power is assumed. |  |
| CW Tx antenna gain (dBi) | the value equals to UE Tx ant gain, or BS Tx ant gain Note: only applicable for device 1/2a |  |  |
| CW cancellation (dB) | For [monostatic backscatter], FFS - [140dB for BS] - [120dB for UE] For [bistatic backscatter] Assuming CW has no impact to the receiver sensitivity loss. |  |  |

Issue 4-3-4: A-IOT device parameters

Agreement in RAN4#111:

Use Device 1 with following parameters for co-existence simulation.

| A-IoT device parameters | Device 1 Values for co-existence simulation |
| --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 |
| A-IoT device reflection （backscatter）loss (dB) Note: due to, e.g., impedance mismatch | OOK: -6 dB |
| A-IoT device power gain of reflection amplifier (dB) | N/A |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | -36 |
| A-IoT device noise figure (dB) | 24 |
| Guard band | 0PRB |

Agreement in RAN4#111:

Assume no frequency selectivity for co-existence evaluation for co-existence simulation for device 1 and 2a.

Both of Device 1 and Device 2a base on backscattered UL transmissions. Device 2a has DL and/or UL amplification to enhance receiver sensitivity and increase backscattered signal power, which sets it apart from Device 1 that lacks an amplifier.

For device 1 and 2a, the power of backscattered signals depends on the power of the received external carrier-wave and the distance between carrier-wave node and Ambient IoT devices. It is reasonable to assume backscattered signals of both of device 1 and 2a are less than -10dBm.

Device 2b generates its UL transmission internally, independent of backscatter. Considering the peak power consumption of few 100 µW and the power added efficiency (PAE) of power amplifier, the maximum transmit power is expected to be no higher than -10 dBm. We can use [-10/-20] dBm to analysis.

Device parameters are proposed as follows:

Table 7-8. Devices parameters

| A-IoT device parameters | Device 1 | Device 2a | Device 2b | Note |
| --- | --- | --- | --- | --- |
| A-IoT device maximum Tx power (dBm) | -10 | -10 | [-10/-20] | For device 1/2a, the device backscattering power = min {-10 dBm, power of the CW signal arriving at the device after space attenuation + backcatter loss/gain} |
| Device antenna height（m） | 1.5 | 1.5 | 1.5 |  |
| A-IoT device antenna gain (dBi) | 0 | 0 | 0 |  |
| A-IoT device backscatter loss (dB) | OOK: -6 dB PSK: 0 dB | [FFS] | N/A |  |
| A-IoT device 2a backscatter amplifier gain(dB) | N/A | 10(M),15(O) | N/A |  |
| A-IoT Device receiver sensitivity (dBm) | -36 | -46 | [FFS] |  |
| A-IoT device noise figure (dB) | 24 | [FFS] | [FFS] |  |
| ACLR | 9 |  |  |  |
| BB LPF | 3rd/5th-order RC filter with cutoff frequency at half of R2D transmission bandwidth. | 3rd/5th-order RC filter with cutoff frequency at half of R2D transmission bandwidth. |  |  |

Issue 4-4-1: NR macro BS parameters

NR outdoor macro-BS parameters are proposed as follows:

Table 7-9. NR macro-BS parameters

| NR macro-BS Parameter | Values for co-existence simulation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |

Referring to AAS study TR 37.840 sub-clause 5.4.4, a passive antenna is currently modeled as a single radiating source according to definition in TR 36.814, Table A.2.1.1-2. The following Macro BS antenna pattern is proposed for A-IOT coexistence study.

Antenna Array Geometry：

- BS point at fixed beam direction

  - vertical: θtilt + 90°

  - horizontal: 0, 120, 240 °

Table 7-10. NR macro-BS antenna pattern

| Parameter | Assumption |
| --- | --- |
| Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) | AHϕ=-min12ϕϕ3dB2,Am = 65 degrees, Am = 25 dB |  |
| Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns) | AVθ=-min12θ-90°-θetiltθ3dB2,SLAv = 10 degrees, SLAv = 25 dB, = 9 degrees |  |
| Combining method in 3D antenna pattern |  |  |
| BS antenna gain (dBi) (including feeder loss) | 15 |  |

Issue 4-4-2: NR UE parameters

NR UE parameters are proposed as follows:

Table 7-11. NR UE parameters

| NR UE Parameter | Recommended value | note |
| --- | --- | --- |
| NR UE Tx power (dBm) | 23 (maximum) | -40 to 23 for calibration |
| NR UE Antenna gain (dBi) | 0 |  |
| NR UE ACLR（dB） | 30 | For power class 3 NR UE |
| NR UE Noise Figure（dB） | 9 |  |
| Height of UE antenna (m) | 1.5 |  |
| Antenna configuration | Omni direction antenna |  |

Use the pathloss model in TR38.901 7.4.3.1 [4] for simulation：

For penetration loss，use the High-loss model in Table 7.4.3-2 in [4]：

Table 7-12. Pathloss model

| NR UE Parameter | PLb | PLin |
| --- | --- | --- |
| outdoor macro gNB to indoor device/indoor reader | UMa | 0.5*d2Din |
| outdoor legacy UE to indoor device/D2T2 reader | UMi | 0.5*d2Din |
| outdoor legacy UE to indoor D1T1 reader | UMi | 0.5*d2Din |
| indoor legacy UE <-> indoor device | indoor office | NA |
| indoor legacy UE <-> indoor reader | D1T1: InF-DH D2T2: indoor office | NA |

Proposal 20: For Ambient IoT, adopt the assumptions and parameters for co-existence analysis as Table 7-1~ 7-12.

Conclusion

Based on the analysis in this contribution, we have the following observations and proposals:

Deployment scenarios:

Observation 1: For CW inside topology of both D1T1 and D2T2, A-IoT reader and CW are hardware sharing.

Proposal 1: For outside topology D1T1-B, assume that CW node is co-located with the neighbouring A-IoT Reader.

Observation 2: A PSD difference of 24dB is required for Option 2-1, which poses a significant challenge for implementation.

Observation 3: Interference between indoor NR UEs and indoor A-IoT BS for Option 2-1 and Option 2-2 may be problematic due to a significant sensitivity difference between A-IoT devices (specifically Device 1 and Device 2a) and NR UEs.

Proposal 2: Deprioritize to Option 2-1 and Option 2-2, due to very high PSD difference and interference.

Spectrum usages

Proposal 3: R2D shall transmit in FDD downlink spectrum in D1T1.

Proposal 4: When CW uses the same spectrum, CW inside and outside topology do not impact coexistence simulation result.

Proposal 5: In D1T1, use Case 1-1 and Case 1-4 as CW2D spectrum usage for the formal simulation:

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal 6: In D2T2, choose Case 2-2 and Case2-4 as CW2D spectrum usage:

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal 7: D2R transmits in the uplink spectrum for device 2b.

Evaluation methodology

Proposal 8: For RF BW 10MHz, R2D SINR is 7.5dB for device 1.

Performance metric for A-IoT

Proposal 9: The SINR includes CW interference is used as the baseline reference before any degradation.

Proposal 10: CDF SINR degradation can serve as performance metric for A-IoT D2R. It is recommended 1dB for A-IoT BS and 1dB for A-IoT intermediate UE.

Proposal 11: For A-IoT device with receiver based on RF envelope detector, it can correctly demodulate wanted signals only when the following conditions are met:

1) SINR>= SINR threshold [7.5dB] for 10MHz RF BW;

2) the wanted signal level>=demodulation sensitivity threshold [ -36dBm for Device 1, and -46dBm for Device 2a]

Proposal 12：The 15 kHz D2R transmission bandwidth can be used as the co-existence simulation baseline.

Proposal 13：SINR definition for D2R is as following:

SINR_D2R= Signal Power in transmission bandwidth / (Noise+ Interference Power) in transmission bandwidth,

Simplified SINR definition for D2R is as following:

SIR_D2R= Signal Power in [15kHz] transmission bandwidth / (Interference Power) in [15kHz] transmission bandwidth

Proposal 14：SINR definition for R2D is as following:

SINR_R2D= Signal Power in transmission bandwidth / (Noise+Interference Power) in transmission bandwidth

Proposal 15：SINR definition for R2D:

SINR_R2D= Signal Power in [180kHz] transmission bandwidth / (Interference Power) in [10M] RF bandwidth)

Evaluation cases

Proposal 16：Remove redundant case 7, 10, 12, 17, and 19 from Table 6-1, since they share the same result with case 1, 4, 6, 13, and 15, respectively.

Proposal 17：No need to distinguish -A1, -A2 and -B for simulation. One case can be applied to all D1T1-A1, D1T1-A2 and D1T1-B (or D2T2-A1, D2T2-A2 and D2T2-B).

Proposal 18：Simulation is not needed for interference from CW to NR/A-IOT.

Evaluation parameters

Proposal 19: For D1T1, Readers transmit round-robin as the baseline, and concurrent transmission is not considered in the co-existence study. Concurrent transmission of Readers is up to implementation.

Proposal 20: For Ambient IoT, adopt the assumptions and parameters for co-existence analysis as Table 7-1~ 7-12.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

[2] R4-2406714, WF on Ambient IoT in NR, CMCC, RAN4#110bis, April, 2024

[3] R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC, RAN4#111, May, 2024

[4] 3GPP TR38.901, Study on channel model for frequencies from 0.5 to 100 GHz,V18.0.0,2024-03