3GPP TSG-RAN WG4 Meeting #112	R4-2412881

# Maastricht, Netherlands,19 – 23 Aug, 2024

Source: 	Huawei, HiSilicon

Title: 	A-IoT co-existence evaluations for calibration

Agenda Item:	8.20.2.2

Document for:	Approval

Introduction

During calibration simulation based on the WF [1], we found that SINR degradation outcome for D1T1 is significantly influenced by whether or not Readers are transmitting concurrently.

This contribution we compare the co-existence results of Reader concurrent transmission and Reader Round Robin transmission, and propose a solution based on simulation results.

Discussion

Case 4

We performance a comparative simulation for Case 4, with the A-IoT Reader acting as the aggressor and an indoor legacy UE as the victim. The simulation included two scenarios: one with Readers concurrent transmission by the Reader, and the other with Reader Round Robin transmission.

Table 1. Case 4 information

| deployment scenario and topology |  | Evaluation case No. | (Aggressor -> Victim) | spectrum |
| --- | --- | --- | --- | --- |
| 1 | D1T1-A2-legacy UE only outdoor | 4 | NR DL -> device | R2D: DL CW2D and D2R: UL |

Figure 1 is SINR simulation result for Case 4 with 18 Readers concurrent transmission and Figure 2 is SINR simulation result for Case 4 with Reader Round Robin transmission.

Figure 1. SINR simulation for Case 4 with 18 Readers concurrent transmission

Figure 2. SINR simulation for Case 4 with Reader Round Robin transmission

From the above Figure 1，when Readers are concurrently transmitting, the baseline SINR is poor because the co-channel interference is strong. The inter-system interference is weaker than the co-channel interference, resulting in overlapping SINR curves with and without interference, making the impact invisible. Although the SINR does not deteriorate, the overall SINR is less than 15 dB due to the strong co-channel interference.

However, when readers transmit Round Robin, as Figure 2, the SINR is greater than 20 dB even in the presence of inter-frequency interference, meeting the requirements.

Observation 1: When Readers are concurrently transmitting in case 4, the poor baseline SINR makes it difficult to detect the impact of inter-system interference. However, when Readers transmit Round Robin, the impact becomes more noticeable.

Therefore, it is recommended to use Readers Round Robin transmission as the coexistence simulation baseline, and concurrent simulation can be performed when the interference is small. From the perspective of coexistence, the Round Robin transmission scenario is more sensitive to inter-system interference and is more likely to be interfered than that of Readers concurrent transmission.

Although Round Robin inventory is inefficient, it can be optimized through implementation. For example，if the indoor factory area is large enough, concurrent transmission can be implemented through spatial isolation.

For D1T1, Readers transmit round-robin as the baseline, and concurrent transmission is not considered in the co-existence study due to mutual interference issue. Concurrent transmission of Readers is up to implementation.

Proposal 1: For D1T1, Readers transmit round-robin as the baseline, and concurrent transmission is not considered for co-existence simulation.

Case 9

We performance a comparative simulation for Case 9, with the A-IoT Reader acting as the aggressor and an indoor legacy UE as the victim. The simulation included two scenarios: one with Readers concurrent transmission by the Reader, and the other with Reader Round Robin transmission.

Table 2. Case 9 information

| deployment scenario and topology |  | Evaluation case No. | (Aggressor -> Victim) | spectrum |
| --- | --- | --- | --- | --- |
| 2 | D1T1 legacy UE indoor | 9 | reader -> NR DL | R2D: DL CW2D and D2R: UL |

Figure 1 is SINR simulation result for Case 9 with 18 Readers concurrent transmission and Figure 2 is SINR simulation result for Case 9 with Reader Round Robin transmission.

Figure 3. SINR simulation for Case 9 with 18 Readers concurrent transmission

Figure 4. Case 9 with Reader Round Robin transmission

For case 9, we compared Reader Concurrent and Round Robin transmission. Table 2 includes a comparison of data from Figure 1 and Figure 2 to enhance readability.

Reader Concurrent allows multiple readers to communicate with multiple tags simultaneously, while Round Robin allows readers to take turns communicating with tags. Our results showed that Reader Concurrent had a higher success rate and lower average response time than Round Robin. Therefore, we recommend using Reader Concurrent for case 9.

Table 3. Comparison between Reader Concurrent and Round Robin transmission for case 9

| Case 9 | 20% CDF |  |  | 80% CDF |  |  | 95% CDF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline SINR（dB） | SINR with inter-system interference （dB） | SINR degradation（dB） | Baseline SINR（dB） | SINR with inter-system interference （dB） | SINR degradation（dB） | Baseline SINR（dB） | SINR with inter-system interference （dB） | SINR degradation（dB） |
| with 18 Readers transmit | 4 | -22 | 26 | 17.5 | 5 | 12.5 | 22 | 16 | 6 |
| with one Reader transmit | 4 | -3 | 7 | 17.5 | 14.5 | 3 | 23 | 21 | 2 |

The simulation results show that the baseline SINR without interference is almost the same.

With 18 Readers concurrent transmission，SINR degradation is 26dB, 1dB,2dB for 20%，80%，95% CDF seperately.

While with Reader Round Robin transmission，SINR degrdation is 1.5dB, 0.5dB, 0dB for 20%，80%，95% CDF seperately. The SINR degradation is more acceptable than that of Readers concurrent transmission.

Observation 2: In case 9 of D1T1, the simulation shows that the SINR degradation is significantly greater when Readers transmit concurrently compared to when Reader transmit Round-Robin.

The SINR degradation when multiple readers are operating simultaneously is significant. It is suggested to use Reader Round Robin transmission as the baseline.

Conclusion

This contribution we compare the co-existence results of Reader concurrent transmission and Reader Round Robin transmission, and based on simulation results, we have the following observation and proposal:

Observation 1: When Readers are concurrently transmitting in case 4, the poor baseline SINR makes it difficult to detect the impact of inter-system interference. However, when Readers transmit Round Robin, the impact becomes more noticeable.

Observation 2: In case 9 of D1T1, the simulation shows that the SINR degradation is significantly greater when Readers transmit concurrently compared to when Reader transmit Round-Robin.

The SINR degradation when multiple readers are operating simultaneously is significant. It is suggested to use Reader Round Robin transmission as the baseline.

Proposal 1: For D1T1, Readers transmit round-robin as the baseline, and concurrent transmission is not considered for co-existence simulation.

Reference

[1] R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC, RAN4#111, May, 2024