3GPP TSG-RAN WG4 Meeting #112 	R4-2412899

Maastricht, Netherlands, August 19 – 23, 2024

Source:	Nokia

Title:	Expected EIRP mask testing

Agenda item:       8.11.4 BS conformance testing	[NR_BS_RF_req_evo-Perf]

Document for:	Approval

- Introduction

TSG RAN#103 meeting approved new Rel-19 work item “NR base station (BS) RF requirement evolution for FR1/FR2 and testing” [1], where first objective is related to expected EIRP mask for upper 6 GHz.

WRC-23 has identified the frequency range 6425 – 7125 MHz which is 3GPP band n104 for IMT as in the RR Footnotes 5.457D, 5.457E and 5.457F. The IMT identification comes with some technical conditions which includes the limits on the expected equivalent isotropic radiated power (e.i.r.p.) spectral density of IMT base stations for protecting Earth-to-space fixed satellite services (FSS) as contained in ITU-R Resolution 220 (WRC-23) [2].

Performance part of expected EIRP mask objective of work item [1] is as follow:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is December 2024.

Last RAN4#111 meeting discussions were concluded in agreed WF [3]. In this contribution we continue discussion for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We provide also text proposal (TP) with technical background to TR 38.908 (Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz).

In addition, we provide TP for the new Manufacturer’s declarations and test procedure for BS conformance specification TS 38.141-2.

- Discussion

In WF [3] following open issues are collected for further discussions:

# Open issue for U6GHz EIRP mask and OTA testing Issue 2-8 Expected EIRP sampling grid for average EIRP - Recommended for further discussion: - Need further discussions Issue 2-9 Other related with conformance testing declaration and RF channels - Recommended for further discussion: - The following issues to be considered: - the number of mechanical down-tilt and its corresponding angular coverage range; - For each angular coverage range, the number of beams for conformance testing and its corresponding weighting factor; - To balance the number for beams for conformance testing/complexity and test accuracy; - The impacts of potential factors (measurement sampling grid for summation error etc) on EIRP accuracy. - in conformance testing, consideration of converting coordinate system to 3GPP coordinate system, to align with other test cases - Other issues are not precluded. Issue 2-10 Other related with RF channels - Recommended for further discussion: - Need further discussions Issue 2-11 Other related with confidence intervals - Recommended for further discussion: - Need further discussions

  - Details on the averaging processes in the expected e.i.r.p. requirement

In this section we provide some technical background for concept of the expected e.i.r.p. requirement averaging processes that is also proposed as TP to Technical Report at the end of this contribution in section 5.

    - Angle definition

The angles in the spherical coordinate system used in this section is as shown in Figure 1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 1: Angle definition

    - Averaging over horizontal and vertical angles

One of the average calculations for the expected e.i.r.p. is averaging the e.i.r.p. over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θL and θH as shown in Figure 2.

Figure 2: Spherical strip (blue shade)

The average e.i.r.p. for each elevation bin is defined as:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ        ,                1

The e.i.r.p. EIRPθ,φ will be measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

      - Approximating the double integral in the e.i.r.p. averaging over an elevation angle range

The average e.i.r.p. evaluated over a spherical strip involves taking e.i.r.p. measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure 3 is an illustration of the measured e.i.r.p. points and the area of the spherical strip for one of the elevation bin. Note that the measured e.i.r.p. points are over three discrete elevation angles with equal number of points in azimuth.

Figure 3: Measured e.i.r.p. points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 4 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 4: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ                      (2)

By substituting the following:

dφ≈∆φ=2π÷M

dθ≈∆θ=π2÷N

the overall equation can be written as:

EIRPθL , θH=π2MNsinθH-sinθLn=nLnHm=1MEIRPθn,φm cosθn    ,          (3)

where

θn	is the midpoint between the elevation intervals

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θL , θH bounding range

nH	is the highest elevation point within the θL , θH bounding range

m	is the azimuth point within the -π , π bounding range

Note: the equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

      - Example illustration of average e.i.r.p. for an elevation angular range

This example uses the ‘elevation angular range’ of 30° ≤ θ < 60° (bin number 6) as defined at WRC-23 for ITU-R Resolution 220:

| Elevation angular range [θL ≤ θ < θH] (Degrees) | Bin number |
| --- | --- |
| 0<<5 | 1 |
| 5<<10 | 2 |
| 10<<15 | 3 |
| 15<<20 | 4 |
| 20<<30 | 5 |
| 30<<60 | 6 |
| 60<<90 | 7 |

Table 1: The elevation angular ranges defined in ITU-R Resolution 220

- Let’s say we have a 10° interval for both azimuth and elevation. This will result to M = 36 (i.e. 360°/10°), and N = 9 (i.e. 90°/10°) points.

- Let’s consider the spherical strip between θL = 30° and θH = 60° (bin number 6)

- This will result to:

  - θn = 35°, 45° and 55°

  - φm = -175°, -165°, -155°, …, 155°, 165° and 175°

  - nL = 35°

  - nH = 55°

  - m = 1, 2, 3, …, 35, 36.

- Note: equation is in radians

- Figure 5 is the illustration of this Example

Figure 5: Illustration of the Example

It is worth noting that this Example shows that we have 3 vertical sets of e.i.r.p. points (i.e. θn = 35°, 45° and 55°) for 1 of the elevation angle range, i.e. bin number 6.

      - Rearranging the average e.i.r.p. equation

The elevation angular range over the entire upper hemisphere (defined for the expected e.i.r.p.) is of different size/bin as shown in Figure 6. Therefore, there is an opportunity to define different ‘angular step size’ for the different elevation bin.

Figure 6: Elevation angular range

The elevation step size (in radians), se, for the upper hemisphere is given by

sθ=π2N                                                                             (4)

This means equation (3) can also be written as

EIRPθL , θH=sθMsinθH-sinθLn=1θH-θLsθm=1MEIRPθn,φm cosθn    ,                       (5)

Similarly, the azimuthal step size (in radians), sφ, around the base station on the horizontal plane is given by

sφ=2πM                                                                              (6)

Therefore, the overall equation can be written as

EIRPθL , θH=sθ×sφ2πsinθH-sinθLn=1θH-θLsθm=12πsφEIRPθn,φm cosθn    ,                      (7)

Since the EIRPθL , θH calculates the average e.i.r.p. only for the elevation bin bounded by θL and θH strip, all the parameters in equation (7) are only relevant for that elevation bin. Therefore, each elevation bin can have different elevation and azimuthal step sizes instead of the general step size M (for azimuth) and N (for elevation) applied to all elevation bins as seen in equation (3).

Observation 1: The average e.i.r.p. calculation of a single beam for the upper hemisphere can be re-written in terms of elevation and azimuthal step sizes.

Observation 2: The alternative equation (with elevation and azimuthal step sizes) removes the dependency of the variable N which is the number of equally-spaced elevation points over the entire upper hemisphere. Therefore, this alternative equation allows flexibility in determining different elevation and azimuthal step sizes for the different elevation angular range.

Proposal 1: RAN4 to consider equation (7) with the use of “elevation step size” and “azimuthal step size”.

    - Averaging over beamforming directions for the expected e.i.r.p. requirement

The expected e.i.r.p. (EEIRP) considers averaging over all the beamforming directions αn,βn where these beams have a uniform angular distribution within the steering range of the IMT base station as shown in Figure 7.

Figure 7: Uniform angular distribution of the beamforming directions

For a given vertical angle θa and horizontal angle φb with a sampling of K beamforming directions γk k=1…K, the equation can be written as:

EEIRPθa,φb=k=1KwkEIRPθa,φb ; γk                                     (8)

where wk refers to the weight for the kth beamforming direction, i.e., the fraction of the steering range represented by the kth beamforming direction. The power measured is the sum of e.i.r.p. in both polarizations. For a set of uniform equispaced beams in both elevation and azimuth with K beamforming directions, the weight for each kth beamforming direction, wk, is therefore 1/K.

The following example illustrates one of the ways to associated the weights for different number of beamforming directions, and how the expected e.i.r.p. is calculated for a specific angle set θa,φb.

      - Example calculation of averaging over beamforming directions with weights

This example has 9 representative beamforming directions (red dots) over the entire electrical steering range. The weight for each beamforming direction in this example is purely a representation of the fractional area covered by the angular boundaries. It can be regarded that each of the fractional area represents the probability of the beam electrically steering within these angles.

Figure 8 (a) shows the electrical steering range, and the beamforming directions with its weight. We can define the weight for each beamforming direction to be a simple representation of the fractional area covered by the angular boundaries (green dashed lines in Figure 8 (b), and each of the fractional area represents the probability of the beam electrically steering within these boundaries.

(a)	(b)

Figure 8: (a) Electrical steering range area, and (b) Beamforming directions and its associated weight

Defining

θ1=θmin                                         ,                   (9)

θ2=θmin+θmax-θmin2        ,                   (10)

θ3=θmax                                        ,                    (11)

φ1=φmin                                       ,                    12

φ2=φmin+φmax-φmin2    ,                    13

φ3=φmax                                     ,                    14

Therefore, the weights for the different beamforming directions (points) are summarized in Table 2.

| Point | Beamforming direction | Weight, wk | θ (°) | ϕ (°) |
| --- | --- | --- | --- | --- |
| 1 | γ1 | 1/16 | θ3 | ϕ1 |
| 2 | γ2 | 2/16 | θ2 | ϕ1 |
| 3 | γ3 | 1/16 | θ1 | ϕ1 |
| 4 | γ4 | 2/16 | θ3 | ϕ2 |
| 5 | γ5 | 4/16 | θ2 | ϕ2 |
| 6 | γ6 | 2/16 | θ1 | ϕ2 |
| 7 | γ7 | 1/16 | θ3 | ϕ3 |
| 8 | γ8 | 2/16 | θ2 | ϕ3 |
| 9 | γ9 | 1/16 | θ1 | ϕ3 |

Table 2: Weights for the beamforming directions (points) in Example 1

These beams are multiplied with the associated weights before being summed to form the average e.i.r.p.

EEIRPθa,φb=116EIRPθa,φb ; γ1+216EIRPθa,φb ; γ2+112EIRPθa,φb ; γ3           +216EIRPθa,φb ; γ4+416EIRPθa,φb ; γ5+216EIRPθa,φb ; γ6        +116EIRPθa,φb ; γ7+216EIRPθa,φb ; γ8+116EIRPθa,φb ; γ9            ,                        (15)

The weights do not change for different minimum and maximum values of the electrical steering angles (i.e. θmin, θmax, φmin and φmax) as the weights are the fractions of the total area covered.

Observation 3: The weights in the EEIRP calculation can be represented as the fractional area of the electrical steering range.

Observation 4: Once the number of beamforming directions have been determined and the method to define the angular boundaries, the weight for each beamforming direction will not change for different minimum and maximum values of the electrical steering angles (i.e. θmin, θmax, φmin and φmax).

Proposal 2: RAN4 to consider defining the angular boundaries in the most practical way (e.g. in the middle of two horizontal/vertical beamforming directions).

  - Expected e.i.r.p. (EEIRP) calculation

The expected e.i.r.p. (EEIRP) is the mean power from the averaging of two different processes highlighted in section 2.1.2 and 2.1.3 where it averages the measurement points over the defined elevation angular range around the base station, and averages over the defined beamforming directions. This expected e.i.r.p. calculation for both averaging processes are required to be sufficiently accurate, therefore, it is important to define appropriate angular step sizes (for averaging over the elevation angular range) and sufficient number of beamforming directions.

Number of stored datapoints=Number of elevation points ×Number of azimuthal points×Number of beamforming directions

Therefore, minimising any of the elevation/azimuthal points and/or the number of beamforming directions will significantly reduce the amount of data storage (due to the multiplicative effect) for the EEIRP calculation.

Observation 5: The EEIRP calculation involves 2 averaging processes and the total number of data points that need to be stored is the multiplication of the measurement points for a single beam multiplied by the number of beamforming directions. This could result in significant data storage requirement.

Proposal 3: RAN4 to consider the number of measurement points when defining the calculation methodology.

    - Angular step size

Using equation (7), different angular step sizes can be considered for different elevation bin. This section analyses how different angular step sizes impact the accuracy of the EEIRP calculation.

First, the parameters defined for the studies leading to the discussions at WRC-23 are used as an example to illustrate the impact of different step size to the overall EEIRP. This is considered as the reference AAS beam and the relevant parameters are defined in Table 3.

|  | Parameter | Value |
| --- | --- | --- |
| 1 | Element gain | 5.5 dBi |
| 2 | Horizontal 3 dB beamwidth | 90° |
| 3 | Vertical 3 dB beamwidth | 90° |
| 4 | Front-to-back ratio (horizontal & vertical) | 30° for both |
| 5 | Antenna polarisation | Linear ±45° |
| 6 | Array configuration (Row x Column) | 16 x 8 |
| 7 | Element spacing (horizontal & vertical) | 0.5λ for both |
| 8 | Electrical tilt range (etilt) | 0° to 20° (in 1° step) |
| 9 | Electrical scan range (escan) | -60° to 60° (in 1° step) |

Table 3: AAS parameters used for analysis

The resulting AAS beam in the elevation and azimuth cut is shown in Figure 9, and the horizontal 3 dB beamwidth is approximately 12.6° whilst the vertical 3 dB beamwidth is approximately 6.3°.

Figure 9: Reference AAS beam

There are 7 elevation angular ranges (bins) with different bin size for the EEIRP limit (in the ITU-R Resolution 220), ranging from 5° to 30° as shown in Table 4.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size (°) | Bin number |
| --- | --- | --- |
| 0<<5 | 5 | 1 |
| 5<<10 | 5 | 2 |
| 10<<15 | 5 | 3 |
| 15<<20 | 5 | 4 |
| 20<<30 | 10 | 5 |
| 30<<60 | 30 | 6 |
| 60<<90 | 30 | 7 |

Table 4: The elevation angular ranges defined in ITU-R Resolution 220 and the bin sizes

As the bin size for the first 4 elevation ranges are 5°, the maximum step size is therefore 5° for these elevation ranges. Similarly for bin 5 (20° ≤ θ < 30°), the maximum step size is 10°. This 10° step size is also set for bin 6 and 7 in this analysis.

Next, the analysis creates three sets of step sizes (Set A, Set B and Set C), and compared against a reference case where a step size of 0.25° is used for both elevation and azimuth. The beamforming directions are equally spaced with 1° resolution; with horizontal electrical scan (escan) of ±60° and elevation electrical tilt (etilt) of 0° to 20°. The result is shown in Table 5.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size | Set | Elevation step size | Azimuthal step size | Number of points per elevation bin | Number of azimuth points | Difference from reference (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0< θ <5 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.10 |
|  |  | C | 5° | 10° | 1 | 36 | 0.51 |
| 5< θ <10 | 5° | A | 1° | 10° | 5 | 36 | 0.02 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.11 |
|  |  | C | 5° | 10° | 1 | 36 | 0.20 |
| 10< θ <15 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.07 |
|  |  | C | 5° | 10° | 1 | 36 | 0.45 |
| 15< θ <20 | 5° | A | 1° | 10° | 5 | 36 | 0.00 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.02 |
|  |  | C | 5° | 10° | 1 | 36 | 0.10 |
| 20< θ <30 | 10° | A | 2° | 10° | 5 | 36 | 0.01 |
|  |  | B | 5° | 10° | 2 | 36 | 0.04 |
|  |  | C | 10° | 10° | 1 | 36 | 0.15 |
| 30< θ <60 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.01 |
|  |  | C | 10° | 10° | 3 | 36 | 0.26 |
| 60< θ <90 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.02 |
|  |  | C | 10° | 10° | 3 | 36 | 0.09 |
| Total number of measurement points |  | A | 1764 |  |  |  |  |
|  |  | B | 792 |  |  |  |  |
|  |  | C | 396 |  |  |  |  |
| Total number of points (with 0.25° step size for both elevation & azimuth) = 129 600 |  |  |  |  |  |  |  |

Table 5: Simulation result comparing the impact of different step size to the reference case

Observation 6: The ability to define separate elevation and azimuthal step size for the different elevation bin could reduce the number of measurement points whilst not impacting the averaging accuracy.

    - Narrower AAS beam

The next analysis looks at the impact of narrower AAS beam by changing only the array configuration to 32 x 8 (row x column) while keeping the other parameters in Table 3 the same. This will create a narrower beam in the elevation plane as shown in Figure 10 where the horizontal 3 dB beamwidth is still approximately 12.6° whilst the vertical 3 dB beamwidth is reduced to approximately 3.2°.

Figure 10: AAS beam with array configuration of 32 x 8 (row x column)

The result is shown in Table 6, and it illustrates that the error is more significant when the elevation step size is larger than the vertical 3 dB beamwidth for some of the elevation bins.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size | Set | Elevation step size | Azimuthal step size | Number of points per elevation bin | Number of azimuth points | Difference from reference (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0< θ <5 | 5° | A | 1° | 10° | 5 | 36 | 0.04 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.35 |
|  |  | C | 5° | 10° | 1 | 36 | 3.50 |
| 5< θ <10 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.15 |
|  |  | C | 5° | 10° | 1 | 36 | 0.47 |
| 10< θ <15 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.09 |
|  |  | C | 5° | 10° | 1 | 36 | 0.26 |
| 15< θ <20 | 5° | A | 1° | 10° | 5 | 36 | 0.00 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.08 |
|  |  | C | 5° | 10° | 1 | 36 | 0.40 |
| 20< θ <30 | 10° | A | 2° | 10° | 5 | 36 | 0.02 |
|  |  | B | 5° | 10° | 2 | 36 | 0.12 |
|  |  | C | 10° | 10° | 1 | 36 | 0.31 |
| 30< θ <60 | 30° | A | 2.5° | 10° | 12 | 36 | 0.01 |
|  |  | B | 5° | 10° | 6 | 36 | 0.17 |
|  |  | C | 10° | 10° | 3 | 36 | 0.10 |
| 60< θ <90 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.00 |
|  |  | C | 10° | 10° | 3 | 36 | 0.11 |
| Total number of measurement points |  | A | 1764 |  |  |  |  |
|  |  | B | 792 |  |  |  |  |
|  |  | C | 396 |  |  |  |  |
| Total number of points (with 0.25° step size for both elevation & azimuth) = 129 600 |  |  |  |  |  |  |  |

Table 6: Simulation result for AAS beam array 32 x 8 (row x column)

Observation 7: The maximum elevation step size for a particular bin should not be bigger than the elevation bin size nor the 3 dB beamwidth of the beam.

Observation 8: The lowest elevation bin is impacted the most from the elevation step size.

Proposal 4: RAN4 to consider defining the maximum elevation step size for a particular elevation bin to be smaller or equal to the elevation bin size or the 3 dB beamwidth of the beam.

  - Number of frequencies

Number of frequencies for OTA spatial emission conformance testing should be performed with appropriate frequencies selected. Many tests in BS conformance specifications are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the BS. These are denoted as RF channels B (bottom), M (middle) and T (top).

Due to nature of OTA spatial emission requirement as a protection for co-existing with FSS UL services it seems reasonable to set two extremes i.e. bottom (B) and top (T). That will assure respective test coverage taking into account reasonable test effort.

Observation 9: Number of frequencies to measure in operating band will impact test effort.

Proposal 5: RAN4 to consider the bottom (B) and top (T) channels as frequencies to measure when defining the test methodology.

  - Test procedure to TS 38.141-2

In this clause provide details of text proposal for new manufacturer declaration and test description into TS 38.141-2 based on considerations from previous sub-clauses.

Text proposal for new Manufacturer’s declarations for TS 38.141-2:

## 4.6	Manufacturer's declarations

The following BS manufacturer's declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3], clause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.xx | OTA spatial emission beam | The vendor may declare separate set or a subset of beams in D.3 with the narrowest intended beamwidths in the elevation and azimuth domains (NBeWθ and NBeWϕ) which are intended for the EEIRP above the horizon in the ITU requirements, to be used during the tests. | x | x | n/a |

Text proposal for OTA spatial emission test procedure for TS 38.141-2:

<Start of TP to TS 38.141-2>

## 6.9	OTA spatial emission

### 6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 6.9.2 Protection of FSS UL

This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

#### 6.9.2.1	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], clause 9.9.2.1.

#### 6.9.2.2	Test purpose

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the values specified in table 9.9.2.1-1 in TS 38.104 [2].

EEIRP is defined as the average value of the EIRP, with the averaging being performed over azimuth angles from −180° to +180 ° and over the specified elevation angle range [θL ≤ θ < θH] in table 9.9.2-1 in TS 38.104 [2].

With the BS generating beam peak directions within the OTA peak directions set.

#### 6.9.2.4	Method of test

##### 6.9.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: B and T; see clause 4.9.1.

Directions to be tested: OTA spatial emission beam directions (D.XX).

Beams to be tested: Declared beam set elevation and azimuth step sizes should be smaller than or equal to the narrowest intended beamwidth in the elevation and/or azimuth domains, respectively:

Sθ≤NBeWθ, Sφ≤NBeWφ

Where Sθ and Sφ are the elevation and azimuth step sizes, and NBeWθand NBeWφ are the narrowest intended beamwidth in the elevation and azimuth domains, respectively.

Aggregated BS channel bandwidth positions to be tested for contiguous carrier aggregation: MBW Channel CA; see clause 4.9.1.

##### 6.9.2.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam of the BS according to the declared OTA spatial emission beam (D.XX) direction

5)	For a BS declared to be capable of single carrier operation, start transmission according to the applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 for BS type 1-O and BS type 1-H in clause 4.9.2 at manufacturers declared rated carrier EIRP (Prated,c,EIRP, D.11).

For a BS declared to be capable of contiguous carrier aggregation operation, set the base station to transmit according to NR-FR1-TM1.1 for BS type 1-O an BS type 1-H in clause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7.2.3.1 and 4.8.

6)	Measure the spectrum emission of the transmitted signal using at least the number of measurement points according to Sθ and Sφ.

7)	Compute EEIRP as the weighted average of the EIRPθL , θH over B tested beamforming directions as follows:

EEIRPθL , θH =b=1Bwb×EIRPθL , θH

Where wb is the weight applied to beamforming direction b and:

b=1Bwb=1

For equally spaced beamforming directions:

wb=1B,b=1..B

Where EIRPθL , θH should be calculated from the EIRP measurements as follows:

EIRPθL , θH=Sθ×Sφ2πsinθH-sinθLn=1θH-θLSθm=12πSφEIRPθn,φm cosθn

Where θH and θL are shown in Figure 9.9.2.1-1 in TS 38.104 [2].

8)	Measure the radiated power for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular OTA spatial emission beam as EIRP = EIRPp1 + EIRPp2.

9) 	Repeat step 4-8 for all OTA spatial emission beam directions declared.

In addition, for multi-band RIB(s), the following steps shall apply:

10) For multi-band RIBs and single band tests, repeat the steps 6) - 9) above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 6.7.2.5	Test requirement

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the values specified in table 9.9.2.1-1 in TS 38.104 [2].

<End of TP to TS 38.141-2>

- Conclusion

In this contribution we continue discussion for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We provide also text proposal (TP) with technical background to TR 38.908 (Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz).

In addition, we provide TP for the new Manufacturer’s declarations and test procedure for BS conformance specification TS 38.141-1.

We have made following observations and proposals:

Observation 1: The average e.i.r.p. calculation of a single beam for the upper hemisphere can be re-written in terms of elevation and azimuthal step sizes.

Observation 2: The alternative equation (with elevation and azimuthal step sizes) removes the dependency of the variable N which is the number of equally-spaced elevation points over the entire upper hemisphere. Therefore, this alternative equation allows flexibility in determining different elevation and azimuthal step sizes for the different elevation angular range.

Proposal 1: RAN4 to consider equation (7) with the use of “elevation step size” and “azimuthal step size”.

Observation 3: The weights in the EEIRP calculation can be represented as the fractional area of the electrical steering range.

Observation 4: Once the number of beamforming directions have been determined and the method to define the angular boundaries, the weight for each beamforming direction will not change for different minimum and maximum values of the electrical steering angles (i.e. θmin, θmax, φmin and φmax).

Proposal 2: RAN4 to consider defining the angular boundaries in the most practical way (e.g. in the middle of two horizontal/vertical beamforming directions).

Observation 5: The EEIRP calculation involves 2 averaging processes and the total number of data points that need to be stored is the multiplication of the measurement points for a single beam multiplied by the number of beamforming directions. This could result in significant data storage requirement.

Proposal 3: RAN4 to consider the number of measurement points when defining the calculation methodology.

Observation 6: The ability to define separate elevation and azimuthal step size for the different elevation bin could reduce the number of measurement points whilst not impacting the averaging accuracy.

Observation 7: The maximum elevation step size for a particular bin should not be bigger than the elevation bin size nor the 3 dB beamwidth of the beam.

Observation 8: The lowest elevation bin is impacted the most from the elevation step size.

Proposal 4: RAN4 to consider defining the maximum elevation step size for a particular elevation bin to be smaller or equal to the elevation bin size or the 3 dB beamwidth of the beam.

Observation 9: Number of frequencies to measure in operating band will impact test effort.

Proposal 5: RAN4 to consider the bottom (B) and top (T) channels as frequencies to measure when defining the test methodology.

- 4. References

- RP-241284, Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, ZTE Corporation

- ITU WRC-23, Final Acts, Resolution 220

- R4-2409955, Way Forward for [111][311] NR_BS_RF, ZTE Corporation

# 5. Text proposal to TR 38.908

<<Start of TP to TR 38.908>

# 4	Background

The ITU-R Radio Regulation has introduced the expected e.i.r.p. technical condition for the protection of the FSS uplink (Earth-to-space) satellite receiver from the aggregate effect of IMT base stations. This is introduced through Resolution 220 (previously known as Resolution COM4/7) [x] and the relevant excerpt is found in the Annex. The expected e.i.r.p. values defined in [x] are as presented in Table 1.

Table 4-1. Expected e.i.r.p. values for vertical angle range defined in ITU Resolution 220 [x]

| Vertical angle range θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (See NOTES 1, 2 and 3) |
| --- | --- |
| 0° ≤ θ < 5° | 27 |
| 5° ≤ θ < 10° | 23 |
| 10°≤ θ < 15° | 19 |
| 15°≤ θ < 20° | 18 |
| 20°≤ θ <30° | 16 |
| 30°≤ θ < 60° | 15 |
| 60°≤ θ ≤ 90° | 15 |
| NOTE 1: The expected e.i.r.p. is defined as the average value of the e.i.r.p., with the averaging being performed. ‒ over horizontal angles from −180º to +180º, with the IMT base station beamforming in a specific direction within its horizontal and vertical steering range, ‒ over different beamforming directions within the IMT base station horizontal and vertical steering range, and ‒ over the specified vertical angle range θL ≤ θ < θH. NOTE 2: An IMT base station shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed). NOTE 3: See the Annex to this Resolution for additional details on how the expected e.i.r.p. can be calculated for this frequency band. |  |

The key requirements of ITU Resolution 220 [2] are:

- compliance with values of expected e.i.r.p above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to Resolution 220 [x]

This expected e.i.r.p. condition proposed for the Radio Regulation is only for the top half of the hemisphere, i.e. above the horizon, and 360° (or 2π) around the azimuth. The angles for the ITU-R (Radio Regulation) are defined as described on Figure 6.1-1.

<next part of TP>

6 Conformance testing requirement and procedures

- 6.1 Angle definition

The angles in the spherical coordinate system used in this section is as shown in Figure 5.1-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 5.1-1: Angle definition

- 6.1.1Averaging over horizontal and vertical angles

One of the average calculations for the expected e.i.r.p. is averaging the e.i.r.p. over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θL and θH as shown in Figure 6.1.1-1.

Figure 6.1.1-1: Spherical strip (blue shade)

The average e.i.r.p. for each elevation bin is defined as:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ        ,                1

The e.i.r.p. EIRPθ,φ will be measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

      - Approximating the double integral in the e.i.r.p. averaging over an elevation angle range

The average e.i.r.p. evaluated over a spherical strip involves taking e.i.r.p. measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure 5.1.1.1-1 is an illustration of the measured e.i.r.p. points and the area of the spherical strip for one of the elevation bin. Note that the measured e.i.r.p. points are over three discrete elevation angles with equal number of points in azimuth.

Figure 6.1.1.1-1: Measured e.i.r.p. points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 6.1.1.1-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 6.1.1.1-2: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ                      (2)

By substituting the following:

dφ≈∆φ=2π÷M

dθ≈∆θ=π2÷N

the overall equation can be written as:

EIRPθL , θH=π2MNsinθH-sinθLn=nLnHm=1MEIRPθn,φm cosθn    ,          (3)

where

θn	is the midpoint between the elevation intervals

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θL , θH bounding range

nH	is the highest elevation point within the θL , θH bounding range

m	is the azimuth point within the -π , π bounding range

Note: the equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

      - Example illustration of average e.i.r.p. for an elevation angular range

This example uses the ‘elevation angular range’ of 30° ≤ θ < 60° (bin number 6) as defined at WRC-23 for ITU-R Resolution 220 [x]:

| Elevation angular range [θL ≤ θ < θH] (Degrees) | Bin number |
| --- | --- |
| 0<<5 | 1 |
| 5<<10 | 2 |
| 10<<15 | 3 |
| 15<<20 | 4 |
| 20<<30 | 5 |
| 30<<60 | 6 |
| 60<<90 | 7 |

Table 6.1.1.2-1: The elevation angular ranges defined in ITU-R Resolution 220 [x]

- Let’s say we have a 10° interval for both azimuth and elevation. This will result to M = 36 (i.e. 360°/10°), and N = 9 (i.e. 90°/10°) points.

- Let’s consider the spherical strip between θL = 30° and θH = 60° (bin number 6)

- This will result to:

  - θn = 35°, 45° and 55°

  - φm = -175°, -165°, -155°, …, 155°, 165° and 175°

  - nL = 35°

  - nH = 55°

  - m = 1, 2, 3, …, 35, 36.

- Note: equation is in radians

- Figure 6.1.1.2-1 is the illustration of this Example

Figure .1.1.2-1: Illustration of the Example

It is worth noting that this Example shows that we have 3 vertical sets of e.i.r.p. points (i.e. θn = 35°, 45° and 55°) for 1 of the elevation angle range, i.e. bin number 6.

      - Rearranging the average e.i.r.p. equation

The elevation angular range over the entire upper hemisphere (defined for the expected e.i.r.p.) is of different size/bin as shown in Figure 6.1.1.3-1. Therefore, there is an opportunity to define different ‘angular step size’ for the different elevation bin.

Figure 6.1.1.3-1: Elevation angular range

The elevation step size (in radians), se, for the upper hemisphere is given by

sθ=π2N                                                                             (4)

This means equation (3) can also be written as

EIRPθL , θH=sθMsinθH-sinθLn=1θH-θLsθm=1MEIRPθn,φm cosθn    ,                       (5)

Similarly, the azimuthal step size (in radians), sφ, around the base station on the horizontal plane is given by

sφ=2πM                                                                              (6)

Therefore, the overall equation can be written as

EIRPθL , θH=sθ×sφ2πsinθH-sinθLn=1θH-θLsθm=12πsφEIRPθn,φm cosθn    ,                      (7)

Since the EIRPθL , θH calculates the average e.i.r.p. only for the elevation bin bounded by θL and θH strip, all the parameters in equation (7) are only relevant for that elevation bin. Therefore, each elevation bin can have different elevation and azimuthal step sizes instead of the general step size M (for azimuth) and N (for elevation) applied to all elevation bins as seen in equation (3).

    - Averaging over beamforming directions for the expected e.i.r.p. requirement

The expected e.i.r.p. (EEIRP) considers averaging over all the beamforming directions αn,βn where these beams have a uniform angular distribution within the steering range of the IMT base station as shown in Figure 6.1.2-1.

Figure 6.1.2-1: Uniform angular distribution of the beamforming directions

For a given vertical angle θa and horizontal angle φb with a sampling of K beamforming directions γk k=1…K, the equation can be written as:

EEIRPθa,φb=k=1KwkEIRPθa,φb ; γk                                     (8)

where wk refers to the weight for the kth beamforming direction, i.e., the fraction of the steering range represented by the kth beamforming direction. The power measured is the sum of e.i.r.p. in both polarizations. For a set of uniform equispaced beams in both elevation and azimuth with K beamforming directions, the weight for each kth beamforming direction, wk, is therefore 1/K.

The following example illustrates one of the ways to associated the weights for different number of beamforming directions, and how the expected e.i.r.p. is calculated for a specific angle set θa,φb.

      - Example calculation of averaging over beamforming directions with weights

This example has 9 representative beamforming directions (red dots) over the entire electrical steering range. The weight for each beamforming direction in this example is purely a representation of the fractional area covered by the angular boundaries. It can be regarded that each of the fractional area represents the probability of the beam electrically steering within these angles.

Figure 6.1.2.1-1 (a) shows the electrical steering range, and the beamforming directions with its weight. We can define the weight for each beamforming direction to be a simple representation of the fractional area covered by the angular boundaries (green dashed lines in Figure 6.1.2.1-2 (b), and each of the fractional area represents the probability of the beam electrically steering within these boundaries.

(a)	(b)

Figure 6.1.2.1-1: (a) Electrical steering range area, and (b) Beamforming directions and its associated weight

Defining

θ1=θmin                                         ,                   (9)

θ2=θmin+θmax-θmin2        ,                   (10)

θ3=θmax                                        ,                    (11)

φ1=φmin                                       ,                    12

φ2=φmin+φmax-φmin2    ,                    13

φ3=φmax                                     ,                    14

Therefore, the weights for the different beamforming directions (points) are summarized in Table 6.1.2.1-1.

| Point | Beamforming direction | Weight, wk | θ (°) | ϕ (°) |
| --- | --- | --- | --- | --- |
| 1 | γ1 | 1/16 | θ3 | ϕ1 |
| 2 | γ2 | 2/16 | θ2 | ϕ1 |
| 3 | γ3 | 1/16 | θ1 | ϕ1 |
| 4 | γ4 | 2/16 | θ3 | ϕ2 |
| 5 | γ5 | 4/16 | θ2 | ϕ2 |
| 6 | γ6 | 2/16 | θ1 | ϕ2 |
| 7 | γ7 | 1/16 | θ3 | ϕ3 |
| 8 | γ8 | 2/16 | θ2 | ϕ3 |
| 9 | γ9 | 1/16 | θ1 | ϕ3 |

Table 6.1.2.1-1: Weights for the beamforming directions (points) in Example 1

These beams are multiplied with the associated weights before being summed to form the average e.i.r.p.

EEIRPθa,φb=116EIRPθa,φb ; γ1+216EIRPθa,φb ; γ2+112EIRPθa,φb ; γ3           +216EIRPθa,φb ; γ4+416EIRPθa,φb ; γ5+216EIRPθa,φb ; γ6        +116EIRPθa,φb ; γ7+216EIRPθa,φb ; γ8+116EIRPθa,φb ; γ9            ,                        (15)

The weights do not change for different minimum and maximum values of the electrical steering angles (i.e. θmin, θmax, φmin and φmax) as the weights are the fractions of the total area covered.

  - 6.2 Expected e.i.r.p. (EEIRP) calculation

The expected e.i.r.p. (EEIRP) is the mean power from the averaging of two different processes highlighted in section 6.1.1 and 6.1.2 where it averages the measurement points over the defined elevation angular range around the base station, and averages over the defined beamforming directions. This expected e.i.r.p. calculation for both averaging processes are required to be sufficiently accurate, therefore, it is important to define appropriate angular step sizes (for averaging over the elevation angular range) and sufficient number of beamforming directions.

Number of stored datapoints=Number of elevation points ×Number of azimuthal points×Number of beamforming directions

Therefore, minimising any of the elevation/azimuthal points and/or the number of beamforming directions will significantly reduce the amount of data storage (due to the multiplicative effect) for the EEIRP calculation.

    - 6.2.1 Angular step size

Using equation (7), different angular step sizes can be considered for different elevation bin. This section analyses how different angular step sizes impact the accuracy of the EEIRP calculation.

First, the parameters defined for the studies leading to the discussions at WRC-23 are used as an example to illustrate the impact of different step size to the overall EEIRP. This is considered as the reference AAS beam and the relevant parameters are defined in Table 3.

|  | Parameter | Value |
| --- | --- | --- |
| 1 | Element gain | 5.5 dBi |
| 2 | Horizontal 3 dB beamwidth | 90° |
| 3 | Vertical 3 dB beamwidth | 90° |
| 4 | Front-to-back ratio (horizontal & vertical) | 30° for both |
| 5 | Antenna polarisation | Linear ±45° |
| 6 | Array configuration (Row x Column) | 16 x 8 |
| 7 | Element spacing (horizontal & vertical) | 0.5λ for both |
| 8 | Electrical tilt range (etilt) | 0° to 20° (in 1° step) |
| 9 | Electrical scan range (escan) | -60° to 60° (in 1° step) |

Table 6.2.1-1: AAS parameters used for analysis

The resulting AAS beam in the elevation and azimuth cut is shown in Figure 9, and the horizontal 3 dB beamwidth is approximately 12.6° whilst the vertical 3 dB beamwidth is approximately 6.3°.

Figure 6.2.1-1: Reference AAS beam

There are 7 elevation angular ranges (bins) with different bin size for the EEIRP limit (in the ITU-R Resolution 220), ranging from 5° to 30° as shown in Table 6.2.1.-2.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size (°) | Bin number |
| --- | --- | --- |
| 0<<5 | 5 | 1 |
| 5<<10 | 5 | 2 |
| 10<<15 | 5 | 3 |
| 15<<20 | 5 | 4 |
| 20<<30 | 10 | 5 |
| 30<<60 | 30 | 6 |
| 60<<90 | 30 | 7 |

Table 6.2.1-2: The elevation angular ranges defined in ITU-R Resolution 220 and the bin sizes

As the bin size for the first 4 elevation ranges are 5°, the maximum step size is therefore 5° for these elevation ranges. Similarly for bin 5 (20° ≤ θ < 30°), the maximum step size is 10°. This 10° step size is also set for bin 6 and 7 in this analysis.

Next, the analysis creates three sets of step sizes (Set A, Set B and Set C), and compared against a reference case where a step size of 0.25° is used for both elevation and azimuth. The beamforming directions are equally spaced with 1° resolution; with horizontal electrical scan (escan) of ±60° and elevation electrical tilt (etilt) of 0° to 20°. The result is shown in Table 6.2.1-3.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size | Set | Elevation step size | Azimuthal step size | Number of points per elevation bin | Number of azimuth points | Difference from reference (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0< θ <5 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.10 |
|  |  | C | 5° | 10° | 1 | 36 | 0.51 |
| 5< θ <10 | 5° | A | 1° | 10° | 5 | 36 | 0.02 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.11 |
|  |  | C | 5° | 10° | 1 | 36 | 0.20 |
| 10< θ <15 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.07 |
|  |  | C | 5° | 10° | 1 | 36 | 0.45 |
| 15< θ <20 | 5° | A | 1° | 10° | 5 | 36 | 0.00 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.02 |
|  |  | C | 5° | 10° | 1 | 36 | 0.10 |
| 20< θ <30 | 10° | A | 2° | 10° | 5 | 36 | 0.01 |
|  |  | B | 5° | 10° | 2 | 36 | 0.04 |
|  |  | C | 10° | 10° | 1 | 36 | 0.15 |
| 30< θ <60 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.01 |
|  |  | C | 10° | 10° | 3 | 36 | 0.26 |
| 60< θ <90 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.02 |
|  |  | C | 10° | 10° | 3 | 36 | 0.09 |
| Total number of measurement points |  | A | 1764 |  |  |  |  |
|  |  | B | 792 |  |  |  |  |
|  |  | C | 396 |  |  |  |  |
| Total number of points (with 0.25° step size for both elevation & azimuth) = 129 600 |  |  |  |  |  |  |  |

Table 6.2.1-3: Simulation result comparing the impact of different step size to the reference case

    - 6.2.2 Narrower AAS beam

The next analysis looks at the impact of narrower AAS beam by changing only the array configuration to 32 x 8 (row x column) while keeping the other parameters in Table 3 the same. This will create a narrower beam in the elevation plane as shown in Figure 10 where the horizontal 3 dB beamwidth is still approximately 12.6° whilst the vertical 3 dB beamwidth is reduced to approximately 3.2°.

Figure 6.2.2-1: AAS beam with array configuration of 32 x 8 (row x column)

The result is shown in Table 6, and it illustrates that the error is more significant when the elevation step size is larger than the vertical 3 dB beamwidth for some of the elevation bins.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size | Set | Elevation step size | Azimuthal step size | Number of points per elevation bin | Number of azimuth points | Difference from reference (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0< θ <5 | 5° | A | 1° | 10° | 5 | 36 | 0.04 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.35 |
|  |  | C | 5° | 10° | 1 | 36 | 3.50 |
| 5< θ <10 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.15 |
|  |  | C | 5° | 10° | 1 | 36 | 0.47 |
| 10< θ <15 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.09 |
|  |  | C | 5° | 10° | 1 | 36 | 0.26 |
| 15< θ <20 | 5° | A | 1° | 10° | 5 | 36 | 0.00 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.08 |
|  |  | C | 5° | 10° | 1 | 36 | 0.40 |
| 20< θ <30 | 10° | A | 2° | 10° | 5 | 36 | 0.02 |
|  |  | B | 5° | 10° | 2 | 36 | 0.12 |
|  |  | C | 10° | 10° | 1 | 36 | 0.31 |
| 30< θ <60 | 30° | A | 2.5° | 10° | 12 | 36 | 0.01 |
|  |  | B | 5° | 10° | 6 | 36 | 0.17 |
|  |  | C | 10° | 10° | 3 | 36 | 0.10 |
| 60< θ <90 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.00 |
|  |  | C | 10° | 10° | 3 | 36 | 0.11 |
| Total number of measurement points |  | A | 1764 |  |  |  |  |
|  |  | B | 792 |  |  |  |  |
|  |  | C | 396 |  |  |  |  |
| Total number of points (with 0.25° step size for both elevation & azimuth) = 129 600 |  |  |  |  |  |  |  |

Table 6.2.2-2: Simulation result for AAS beam array 32 x 8 (row x column)

<<End of TP to TR 38.908>