3GPP TSG-RAN WG4 Meeting #112 	R4-2412902

Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.114 | CR | 0016 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.114 with updates and corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is CR to repeater core specification TS 38.106. This CR introduces aligment for repeater terminology which is described in [R4-2412900]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alignment of terminology for repeaters Rel-17 and NCR Rel-18. There are also introduced new clauses 6.1.1A and 6.2.1A that are introducing correct titles. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminology in repeater core and test specifications will not be align. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.1, 4.4, 4.5, 6.1, new clause 6.1.1A, 6.1.1, new clause 6.2.1A, 8.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

ancillary equipment: electrical or electronic equipment, that is intended to be used with a receiver or transmitter

NOTE:	It is considered as an ancillary equipment if:

the equipment is intended for use with a receiver or transmitter to provide additional operational and/or control features to the radio equipment, (e.g. to extend control to another position or location); and

the equipment cannot be used on a stand alone basis to provide user functions independently of a receiver or transmitter; and

the receiver or transmitter, to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub-unit of the main equipment essential to the main equipment basic functions).

antenna port: for EMC purposes, port for connection of an antenna used for intentional transmission and/or reception of radiated RF energy, equivalent to an RF antenna connector.

channel bandwidth: the RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

continuous phenomena: electromagnetic disturbance, the effects of which on a particular device or equipment cannot be resolved into a succession of distinct effects (IEC 60050-161 [20]).

exclusion band: frequency range(s) not subject to test or assessment.

multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

NR repeater: the repeater type 1-C or repeater type 2-O

NCR-MT: the NCR-MT is defined as a function entity to communicate with a gNB via a Control link (C-link) to enable exchange of control information (e.g. side control information at least for the control of NCR-Fwd). The C-link is based on NR Uu interface.

NCR-Fwd: the NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB.

Figure 3.1-1: Conceptual model of network-controlled repeater

NCR type 1-C: network-controlled repeater including both NCR-MT and NCR-Fwd operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors.

Figure 3.1-2: NCR type 1-C downlink and uplink interface

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate antenna connectors

NCR type 1-H: network-controlled repeater including both NCR-MT and NCR-Fwd operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

Figure 3.1-3: NCR type 1-H downlink and uplink interface

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate TAB connectors.

NCR type 2-O: network-controlled repeater including both NCR-MT and NCR-Fwd operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

Figure 3.1-4: NCR type 2-O downlink and uplink interface

NOTE 1:	the NCR-MT and NCR-Fwd may have the same or separate RIB.

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

passband edge: Frequency at the edge of the passband.

port: A particular interface, of the specified equipment (apparatus), with the electromagnetic environment. For example, any connection point on an equipment intended for connection of cables to or from that equipment is considered as a port (see Figure 3.1-1).

Figure 3.1-5: Examples of ports

repeater type 1-C: Repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

repeater type 2-O: Repeater operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply.

signal/control port: port intended for the interconnection of components of an EUT, or between an EUT and associated equipment and used in accordance with relevant functional specifications (for example for the maximum length of cable connected to it).

single-band connector: Repeater type 1-C antenna connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector

spatial exclusion zone: range of angles where no tests of radiated immunity are made for repeater type 2-O (i.e. half sphere around the EUT's radiating direction).

telecommunication port: ports which are intended to be connected to telecommunication networks (e.g. public  switched telecommunication networks, integrated services digital networks), local area networks (e.g. Ethernet, Token Ring) and similar networks.

NOTE:	Telecommunication port is called "wired network port" in CISPR 32 [5] and ETSI EN 301 489-1 [21].

transient phenomena: pertaining to or designating a phenomena or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [20]).

<Next changes>

## 4.1	General

Requirements throughout the EMC specifications are in some cases defined separately for different frequency ranges (FR). The frequency ranges FR1 and FR2 are defined in clause 5.1 of TS 38.106 [2]. NR Repeater and NCR are designed to operate in FR1 and FR2-1.

The equipment shall be tested in normal test environment defined in the corresponding NR Repeater conformance testing specification TS 38.115-1 [3] for NR Repeater type 1-C, NCR type 1-C and NCR type 1-H or TS 38.115-2 [4] for NR Repeater type 2-O and NCR type 2-O. The test conditions shall be recorded in the test report.

For Repeater capable of multi-band operation, the requirements in the present document apply for each supported operating band unless otherwise stated. Operating bands shall be activated according to the test configuration in clause 4.5. Tests shall be performed relating to each type of port and all operating bands shall be assessed during the tests.

The manufacturer shall declare the supported operating band(s) according to the list of NR repeater and NCR operating bands defined in TS 38.106 [2].

NOTE 1: NR operating bands for NR repeater type 1-C, NCR type 1-C and NCR type 1-H, are declared by the manufacturer according to the declaration D.2 specified in TS 38.115-1 [3], table 4.6-1.

NOTE 2: NR operating bands for NR repeater type 2-O and NCR type 2-O, are declared by the manufacturer according to the declaration D.4 specified in TS 38.115-2 [4], table 4.6-1.

<Next changes>

## 4.4	Exclusion bands

The exclusion band for NR repeater and NCR is the frequency range over which no tests of radiated immunity are made in UL or DL.

The exclusion band for DL is defined as:

FDL,low – ΔfOBUE < f < FDL,high + ΔfOBUE

Where values of FDL,low and FDL,high are defined for each operating band in TS 38.106 [2], clause 5.2.

The exclusion band for UL is defined as:

FUL,low – ΔfOBUE < f < FUL,high + ΔfOBUE

Where values of FUL,low and FUL,high are defined for each operating band in TS 38.106 [2], clause 5.2.

For NR repeater and NCR capable of multi-band operation, the total exclusion band is a combination of the exclusion bands for each operating band supported by NR repeater or NCR.

The ΔfOBUE values are defined in table 4.4-1 for both DL and UL.

Table 4.4-1: ΔfOBUE offset values

|  | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| NCR type 1-H | FDL,high – FDL,low < 100 MHz, or FUL,high – FUL,low < 100 MHz | 10 |
|  | 100 MHz FDL,high – FDL,low 900 MHz, or 100 MHz FUL,high – FUL,low 900 | 40 |
| Repeater type 1-C, | FDL,high – FDL,low < 200 MHz, or FUL,high – FUL,low < 200 MHz | 10 |
| NCR type 1-C | 200 MHz FDL,high – FDL,low 900 MHz, or 200 MHz FUL,high – FUL,low 900 MHz | 40 |

NOTE:	As the radiated immunity testing is defined in the frequency range 80 MHz to 6 GHz, there is no exclusion band defined for FR2.

## 4.5	NR repeaters and NCR test configurations

The present clause defines the NR repeaters test configurations that shall be used for demonstrating conformance. A single NR repeater carrier shall be used for testing of single-carrier capable NR repeaters.

The signal's channel bandwidth and subcarrier spacing used to build NR repeater Test Configurations shall be selected according to table 4.7.2-1 in TS 38.115-1 [3] clause 4.7 for NR repeaters type 1-C, NCR type 1-C and NCR type 1-H and table 4.7.2.1-1 in TS 38.115-2 [4] clause 4.7 for NR repeaters type 2-O and NCR type 2-O. The passband frequency range declared per operating band in TS 38.115-1 [3] clause 4.6, and TS 38.115-2 [4] clause 4.6 shall be used.

For other NR repeaters and NCR, the test configurations in tables 4.5-1, 4.5-2, 4.5-3 and 4.5.4 shall be used. The NR repeaters and NCR test configurations (RTCx) are defined in TS 38.115-1 [3], clause 4.7 for NR repeaters type 1-C, NCR type 1-C and NCR type 1-H and in TS 38.115-2 [4], clause 4.7 for NR repeaters type 2-O, NCR type 2-O.

Table 4.5-1: Test configurations for NR repeaters type 1-C

| Repeater test case | Repeater capable of single or multiple passbands in a single band |  |  | Repeater capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband | Common connector | Separate connectors |
| Emission tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 2), RTC4 (Note 2) |
| Immunity tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 2), RTC4 (Note 2) |
| NOTE 1: RTC1 and/or RTC2 shall be applied in each supported operating band. NOTE 2: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |  |  |  |

Table 4.5-2: Test configurations for NR repeaters type 2-O

| Repeater test case | Repeater capable of single or multiple passbands in a single band |  |  |
| --- | --- | --- | --- |
|  | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband |
| Emission tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Immunity tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 |

Table 4.5-3: Test configurations for NCR type 1-C, NCR type 1-H

| Repeater test case | Repeater capable of single or multiple passbands in a single band |  |  | Repeater capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband | Common connector | Separate connectors |
| Emission tests | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 | NCRTC1/2 (Note 1), NCRTC4 | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 3) |
| Immunity tests | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 | NCRTC1/2 (Note 1), NCRTC4 | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 3) |
| Note 1: NCRTC1 and/or NCRTC2 shall be applied in each supported operating band. Note 2: NCRTC4 may be applied for Inter passband gap only. Note 3: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |  |  |  |

Table 4.5-4: Test configurations for NCR type 2-O

| Repeater test case | Repeater capable of single or multiple passbands in a single band |  |  |
| --- | --- | --- | --- |
|  | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband |
| Emission tests | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Immunity tests | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |

<Next changes>

# 6	Performance criteria

## 6.1	Performance criteria for continuous phenomena

### 6.1.1A	Performance criteria for continuous phenomena for NR repeaters and NCR-Fwd

The power accuracy of the EUT shall be measured throughout the period of exposure of the phenomenon.

For repeater type 1-C, NCR type 1-C, and NCR type 1-H, the measured output power Pmax,p,AC during the test shall not change from the rated passband output power Prated,p,AC measured before the test by more than ± 1 dB.

For repeater type 2-O and NCR type 2-O, the maximum passband TRP output power Pmax,p,TRP during the test shall not change from the rated passband TRP output power Prated,p,TRP measured before the test by more than ± 1 dB.

At the conclusion of the test the EUT shall operate as intended with no loss of user control functions or stored data.

### 6.1.1	Performance criteria for continuous phenomena for NCR-MT

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in table 6.1.1-1 and table 6.1.1-2. If the test is not performed using one of these bearers (for example, none of them are supported by the NCR-MT), the characteristics of the bearer used shall be recorded in the test report.

The throughput in table 6.1.1-1 and table 6.1.1-2 is stated relative to the maximum throughput of the FRC.

The NCR-MT uplink and downlink paths shall each meet the performance criteria defined in table 6.1.1-1 and table 6.1.1-2 during the test. If the uplink and downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in table 6.1.1-1 for FR1 WA NCR-MT and table 6.1.1-2 for FR2 NCR-MT (i.e. throughput > 90 % instead of throughput > 95 %). After each test case NCR-MT shall operate as intended with no loss of user control function, stored data and the communication link to both UE and donor test equipment shall be maintained.

For LA NCR-MT the performance criteria shall be that the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A in TS 38.101-1 [3] or TS 38.101-2 [4] for FR1 and FR2 respectively, with parameters specified in clause 7.3.2 in TS 38.101-1 [3] or TS 38.101-2 [4] during the test sequence.

Table 6.1.1-1: FR1 performance criteria for continuous phenomena for WA NCR-MT

| NR channel bandwidth (MHz) as defined in TS 38.106 section 5.4.2 [2] for NCR-MT | Sub-carrier spacing (kHz) | Bearer information data rate for NCR-MT | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 10, 15 | 30 | G-FR1-A1-22 in annex A.1 in TS 38.106 [2] | Throughput > 95 %, no loss of service |
| 10, 15 | 60 | G-FR1-A1-23 in annex A.1 in TS 38.106 [2] |  |
| 20 to 100 | 30 | G-FR1-A1-25 in annex A.1 in TS 38.106 [2] |  |
| 20 to 100 | 60 | G-FR1-A1-26 in annex A.1 in TS 38.106 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1.1-2: FR2-1 performance criteria for continuous phenomena for NCR-MT

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate for NCR-MT | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-21 in annex A.1 in TS 38.106 [2] | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-A1-22 in annex A.1 in TS 38.106 [2] |  |
| 100, 200, 400 | 120 | G-FR2-A1-23 in annex A.1 in TS 38.106 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

## 6.2	Performance criteria for transient phenomena

### 6.2.1A	Performance criteria for transient phenomena for NR repeaters and NCR-Fwd

The power accuracy of the EUT shall be measured before the test and after each exposure.

For repeater type 1-C, NCR-Fwd type 1-C and NCR-Fwd type 1-H, the measured output power Pmax,p,AC after each exposure and after the total test shall not change from the rated passband output power Prated,p,AC measured before the test by more than ± 1 dB.

For repeater type 2-O and NCR-Fwd type 2-O, the maximum passband TRP output power Pmax,p,TRP after each exposure and after the total test shall not change from the rated passband TRP output power Prated,p,TRP measured before the test by more than ± 1 dB.

At the conclusion of the total test comprising the series of individual exposures, the EUT shall operate as intended with no loss of user control functions or stored data.

### 6.2.1	Performance criteria for transient phenomena for NCR-MT

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in table 6.2.1-1 and table 6.2.1-2. If the test is not performed using one of these bearers (for example, none of them are supported by the NCR-MT), the characteristics of the bearer used shall be recorded in the test report.

The NCR-MT uplink and downlink paths shall each meet the performance criteria defined in table 6.1.1-1 and table 6.1.1-2 during the test. If the uplink and downlink paths are evaluated as a one loop, then the criteria is two times the throughput reduction shown in table 6.1.1-1 for FR1 WA NCR-MT and table 6.1.1-2 for FR2 NCR-MT (i.e. throughput > 90 % instead of throughput > 95 %). After each test case NCR-MT shall operate as intended with no loss of user control function, stored data and the communication link to both UE and donor test equipments shall be maintained.

For LA NCR-MT the performance criteria shall be that the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A in TS 38.101-1 [3] or TS 38.101-2 [4] for FR1 and FR2 respectively, with parameters specified in clause 7.3.2 in TS 38.101-1 [3] or TS 38.101-2 [4] during the test sequence.

<Next changes>

## 8.2	Radiated emission

### 8.2.1	Radiated emission, Repeater and NCR

This test is applicable to repeater type 1-C, NCR type 1-C and NCR type 1-H. This test shall be performed on a representative configuration of repeater or NCR.

For repeater type 2-O and NCR type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.106 [2], conforming to the test requirement in TS 38.115-2 [4].

#### 8.2.1.1	Definition

This test assesses the ability of repeater or NCR to limit unwanted emission from the enclosure port.

#### 8.2.1.2	Test method

a)	A test site fulfilling the requirements of ITU-R SM.329 [19] shall be used. The repeater or NCR shall be placed on a non-conducting support and shall be operated from a power source via a RF filter to avoid radiation from the power leads. One of the following two alternative measurement methods shall be used:

1)	Field strength method measurement

The test method shall be in accordance with CISPR 32 5. The field strength measurements shall be performed on a test site that is validated according to the methods and requirements of CISPR 16-1-4 [25].

Unless otherwise stated, measurements are conducted at 3 m or 10 m on an open area test site (OATS) or semi anechoic chamber (SAC) for frequencies up to 1 GHz, or at 3 m on a free space open area test site (FSOATS) or fully-anechoic room (FAR) for frequencies above 1 GHz. Unless otherwise stated, all measurements are done with RMS detector and with the -3 dB bandwidth of the measuring filter equal to the reference bandwidth in table 8.2.1.3-1.

NOTE 1:	Test site validation methods for radiated emissions tests are defined in CISPR 16-1-4 [25], clause 6 and 7. Examples of test site validation methods are listed below:

-	30 - 1000 MHz frequency range: Normalized Site Attenuation (NSA), Reference Site Method (RSM).

-	1 - 18 GHz frequency range: SVSWR standard test procedure, SVSWR reciprocal test procedure.

2)	Substitution method measurement (also called a substitution method)

Mean power of any spurious components shall be detected by the test antenna and measuring receiver (e.g. a spectrum analyser). At each frequency at which a component is detected, the repeater or NCR shall be rotated and the height of the test antenna adjusted to obtain maximum response, and the effective radiated power (e.r.p.) of that component determined by a substitution measurement. The measurement shall be repeated with the test antenna in the orthogonal polarization plane. Unless otherwise stated, all measurements are done as mean power (RMS).

NOTE 2:	Effective radiated power (e.r.p.) refers to the radiation of a half wave tuned dipole instead of an isotropic antenna. There is a constant difference of 2.15 dB between e.i.r.p. and e.r.p, as defined in ITU-R SM.329 annex 1 [19].

e.r.p. (dBm)  EIRP (dBm)  2.15

b)	The repeater or NCR shall transmit with maximum power declared by the manufacturer with all transmitters active. Set the repeater or NCR to transmit a signal as stated in clause 4.5.

c)	The received power shall be measured over the frequency range from 30 MHz to FDL,low - ΔfOBUE and from FDL,high + ΔfOBUE up to 12750 MHz. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R SM.329 [19]. The video bandwidth shall be approximately three times the resolution bandwidth. If this video bandwidth is not available on the measuring receiver, it shall be the maximum available and at least 1 MHz. Unless otherwise stated, all measurements are done as mean power (RMS).

#### 8.2.1.3	Limits

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R Recommendations SM.329 [19] and SM.1539-1 [26].

The repeater type 1-C, NCR-Fwd type 1-C and NCR-Fwd type 1-H shall meet the limits below:

Table 8.2.1.3-1: Limits for radiated emissions from repeater type 1-C, NCR-Fwd type 1-C, NCR-Fwd type 1-H

| Frequency range | e.r.p. (dBm) | Field strength at 3 m (dBµV/m) (NOTE 4) | Field strength at 10 m (dBµV/m) (NOTE 4) | Reference bandwidth | Notes |
| --- | --- | --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 | 65.4 (NOTE 5) | 54.9 (NOTE 5) | 100 kHz | NOTE 7 |
| 1 GHz ≤ f < 12.75 GHz | -30 | 67.4 | Not applicable | 1 MHz | NOTE 7 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 | 67.4 | Not applicable | 1 MHz | NOTE 1,7 |
| 12.75 GHz - 26 GHz | -30 | 67.4 | Not applicable | 1 MHz | NOTE 6,7 |
| FDL,low - ΔfOBUE < f < FDL,high +ΔfOBUE | Not defined | Not defined | Not defined | Not defined | NOTE 2,3,7 |
| NOTE 1: This frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 2: For repeater or NCR capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 3: ΔfOBUE is defined in clause 6.5.3 of TS 38.106 [2]. NOTE 4: The field strength measurements shall be conducted on OATS or SAC for frequencies up to 1 GHz, or on FSOATS or FAR for frequencies above 1 GHz. NOTE 5: Limits for radiated emissions are translated from the e.r.p. limit of -36 dBm into the field strength limit of 61.4 dBµV/m (at 3m) or 50.9 dBµV/m (at 10m), and increased by the site gain value of 4 dB. The value of the site gain is based on ITU-R Recommendations SM.329 [19]. NOTE 6: For NCR when NCR-Fwd and NCR-MT are transmitting simultaneously, the RSE requirements should apply for sum of NCR-MT and NCR-Fwd transmission. NOTE 7: For NCR-MT type 1-C and NCR-MT type 1-H, the radiated emission is covered by radiated spurious emission requirement in clauses 6.5.4.4.2 of TS 38.106 [2]. |  |  |  |  |  |

<End of changes>