3GPP TSG-RAN WG4 Meeting #112	R4-2412905

Maastricht, Netherlands, August 19 – 23, 2024

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS Demodulation Performance Requirements for MMSE-IRC

Agenda item:	8.16.3

Document for:	Discussion

Introduction

One Rel-19 Work Item (WI) to be started in RAN4#112 is NR demodulation performance Phase 5. As documented in the Work Item Description (WID) [1], the justification of the WI is the need for further enhancement of UE and BS demodulation performance to deal with more scenarios of practical network as shown below [1]:

3 Justification In Rel-19, to verify the UE and BS demodulation performance with advanced receiver and apply demodulation requirements to more scenarios of practical network, the following enhancements are identified: - The Rel-18 8Rx performance requirements (introduced under NR_ENDC_ RF_FR1_enh2 WI) do not cover scenarios of co-channel inter-cell and intra-cell inter-user interference. With 8 Rx, larger gain of MMSE-IRC is expected under interference scenarios, due to larger spatial domain freedom compared to 2/4 Rx. Therefore, there are benefits to define UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices under interference scenarios. - BS receiver with MMSE-IRC can efficiently suppress the uplink interference from UEs in neighbouring cells and thus improve uplink coverage and capability. In addition, the widely use of HPUE makes UL co-channel interference from the neighbouring cell UE severer than that in LTE system. There is necessity to introduce BS performance requirements for MMSE-IRC advanced receiver under inter-cell interference.

The justification of the WI above shows that the targeted further enhancements are on handling the interference when using an advanced receiver at the UE for 8Rx CPE/FWA/vehicle/industrial devices in Downlink (DL) scenario and at the BS in Uplink (UL) scenario. Specifically, in this WI, the considered advanced receiver to be used at the UE in DL and the BS in UL is MMSE-IRC receiver.

Based on such a justification above, the objective of the WI for the (demodulation) performance part is defined as below [1]:

4.2 Objective of Performance part WI - Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In the justification and objective of the WI above, there are two independent demodulation performance scenarios which would be treated in two different Agenda Items (AIs) in RAN4 discussions, namely, DL demodulation performance at 8Rx UE and UL demodulation performance at BS.

In this contribution, we share our early views on Rel-19 NR demodulation performance Phase 5 for BS demodulation performance requirements for MMSE-IRC.

Discussion

Signal Model and MMSE-IRC Receiver

TR 36.884 [2] has extensively studied the MMSE-IRC in LTE, in which the outcome was also documented as BS demodulation performance requirements in TS 36.104 [4]. As stated in [1], TR 36.884 [2] could be used as a reference for this WI. In [2], the signal model and the MMSE-IRC algorithm are described as below:

4.1 Signal model For PUSCH, the receiver equations are provided as the reference receiver for evaluation purpose. Assume that is the frequency domain representation of the k-th subcarrier in the -th DFT-OFDM symbol for the -th UE scheduled by its serving BS. is obtained with DFT operation of the time-domain modulating symbol, denoted as , where , with DFT size of . The NRx-dimensional received signal vector of the k-th subcarrier and the l-th DFT-OFDM symbol by the serving BS of the 1st UE is assumed to be expressed as a sum of the signal from the 1st UE, the interference signals (j>1) from other UEs and the white noise , as where , represent the (NRx x NStream) effective channel matrix between the -th UE and the serving BS of the 1st UE, containing the contribution from all receiver branches. is the total number of UEs scheduled by their corresponding BSs in the network. The recovered NStream x 1 signal vector for the 1st UE at its serving BS is detected as follows: where the (NStream x NRx) receiver weight matrix is determined according to the type of PUSCH receiver, as discussed in the following sections. The detected time-domain modulating symbol, denoted as , can be obtained with a IDFT operation for . There will be two types of PUSCH receiver, as MMSE receiver and MMSE-IRC receiver. The equalization is assumed to be performed in the frequency domain. 4.3 MMSE-IRC receiver The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.

The received signal vector r consists of three components: the intended UE signal, the interference signals from the other cells’ UEs and the noise signal. In order to recover the intended UE’s signal vector d1, the MMSE-IRC weight matrix W is applied to the received signal r.

As shown above, the MMSE-IRC weigh matrix W relies on the estimated covariance matrix R of the interference plus noise signals r. It could be observed here that the intercell interference signals, which could also come in the form of other cells’ HPUEs, are all already captured in r, and, hence, in the covariance matrix R.

BS UL MMSE-IRC has been extensively studied in TR 36.884 and specified in TS 36.104 for LTE. A DMRS based MMSE-IRC receiver is assumed.

Intercell interference signals have been considered in TR 36.884 as the source of interference in the received signals. Hence, the estimated covariance matrix R used to compute the MMSE-IRC weight matrix W has already captured the considered intercell interference signals.

With the estimated covariance matrix R as in TR 36.884, no intra-network information exchange shall be forced to compute the MMSE-IRC weight matrix W.

The signal model and MMSE-IRC receiver weight matrix with its corresponding estimated covariance matrix as in TR 36.884 should be used as references to define the FR1 PUSCH requirements in this WI.

There should be no other specific receiver structures to be forced to be implemented and the detailed structure for any (and all) scenarios should be up to implementation.

Similar as in LTE, there is no need to enforce intra-network information exchange, when using the reference receiver.

Number of Rx

As stated in TR 36.884 [2], there is no significant gain of MMSE-IRC to the baseline MMSE receiver in the case of 2RX when the explicit interferer number is increased from 1 to 2. Hence, 2 explicit interferers are modeled for only 4Rx and 8Rx cases.

In 36.766 [3], 8Rx BS is not covered in the evaluation, and one of the reasons is due to a large number of channel faders are needed for potential 8Rx IC (interference cancellation) test. It is noted here that in [3], the study is on intracell interference cancellation, in addition to the intercell interference.

In TR 36.884, with 2 explicit interferers, only 4Rx and 8Rx are considered.

In TR 36.766, only 2Rx and 4Rx are considered.

Based on previous studies documented in TR 36.884 and TR 36.766, RAN4 shall prioritize 4Rx case especially in the case of 2 explicit interferers.

Interference Profile

In TR 36.884 [2], the Dominant Interference Proportion (DIP) obtained from system-level simulation was used to provide interference profile for the link level simulation used to define BS demodulation performance requirement in LTE. Nonetheless, referring to TS 38.101-4 [5] for UE demodulation performance requirement in NR, Interference-to-Noise Ratio (INR) was used instead of DIP.

For interference profile, Dominant Interference Proportion (DIP) was used in LTE BS demodulation performance requirements. Such a DIP as in LTE could be considered as a starting point in this WI,

In NR UE demodulation performance requirements, Interference-to-Noise Ratio (INR) was used.

RAN4 to discuss using INR instead of DIP, as in NR UE demodulation performance requirements.

Focusing only on the UL BS demodulation performance, the key points taken from the justification of the WID [1] are:

- MMSE-IRC is used to efficiently suppress the uplink interference from UEs in neighbouring cells

- HPUE makes UL co-channel interference from the neighbouring cell UE severer than that in LTE system.

Concerning the second point above, there would be a need to accommodate such a justification in the interference profile. The DIPs from TR 36.884 [2] could be used in this WI as references, as the two DIPs already show that one interferer is more dominant than the other. Using DIP from [2] would avoid the need for RAN4 to run system level simulation to have different interference profile due to HPUE. As DIP1 and DIP2 in [2] have different levels of dominance, in which DIP1 is relatively much higher than DIP2, DIP1 could be assumed as interference coming from HPUE while DIP2 from a normal UE.

DIP1 and DIP2 in TR 38.884 have different levels of dominance, which could be interpreted as two different interferers with two different power levels, where the one with the highest power level represents the HPUE. Using such DIP1 and DIP2 will avoid the need to run system level simulation to generate different interference profile with interference coming from a mixed of HPUEs and normal UEs.

RAN4 to use DIP1 and DIP2 in TR 38.884 as references to define corresponding INR1 and INR2, in which DIP1 shall be assumed as the interference from HPUE and DIP2 from a normal UE.

Bandwidth and SCS

In [2] (and, hence, [4]), the BS demodulation performance requirements are defined for all possible bandwidths in LTE. In NR, however, with more options on bandwidth sizes than LTE added by different SCS, defining all possible bandwidth options with all possible SCSs would result in too many test cases. On the other hand, referring to [2] and [4], the performance between different bandwidth sizes is relatively small.

In LTE, the BS demodulation requirements are defined for all possible bandwidth (BW) sizes. Nonetheless, the performance difference of BS MMSE-IRC in those different bandwidth sizes are relatively small.

RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and have all DUTs testable regardless of BW support declaration.

Conclusion

In this contribution, we provide our early views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Observation 1: BS UL MMSE-IRC has been extensively studied in TR 36.884 and specified in TS 36.104 for LTE. A DMRS based MMSE-IRC receiver is assumed.

Observation 2: Intercell interference signals have been considered in TR 36.884 as the source of interference in the received signals. Hence, the estimated covariance matrix R used to compute the MMSE-IRC weight matrix W has already captured the considered intercell interference signals.

Observation 3: With the estimated covariance matrix R as in TR 36.884, no intra-network information exchange shall be forced to compute the MMSE-IRC weight matrix W.

Proposal 1: The signal model and MMSE-IRC receiver weight matrix with its corresponding estimated covariance matrix as in TR 36.884 should be used as references to define the FR1 PUSCH requirements in this WI.

Proposal 2: There should be no other specific receiver structures to be forced to be implemented and the detailed structure for any (and all) scenarios should be up to implementation.

Proposal 3: Similar as in LTE, there is no need to enforce intra-network information exchange, when using the reference receiver.

Observation 4: In TR 36.884, with 2 explicit interferers, only 4Rx and 8Rx are considered.

Observation 5: In TR 36.766, only 2Rx and 4Rx are considered.

Proposal 4: Based on previous studies documented in TR 36.884 and TR 36.766, RAN4 shall prioritize 4Rx case especially in the case of 2 explicit interferers.

Observation 6: For interference profile, Dominant Interference Proportion (DIP) was used in LTE BS demodulation performance requirements. Such a DIP as in LTE could be considered as a starting point in this WI,

Observation 7: In NR UE demodulation performance requirements, Interference-to-Noise Ratio (INR) was used.

Proposal 5: RAN4 to discuss using INR instead of DIP, as in NR UE demodulation performance requirements.

Observation 8: DIP1 and DIP2 in TR 38.884 have different levels of dominance, which could be interpreted as two different interferers with two different power levels, where the one with the highest power level represents the HPUE. Using such DIP1 and DIP2 will avoid the need to run system level simulation to generate different interference profile with interference coming from a mixed of HPUEs and normal UEs.

Proposal 6: RAN4 to use DIP1 and DIP2 in TR 38.884 as references to define corresponding INR1 and INR2, in which DIP1 shall be assumed as the interference from HPUE and DIP2 from a normal UE.

Observation 9: In LTE, the BS demodulation requirements are defined for all possible bandwidth (BW) sizes. Nonetheless, the performance difference of BS MMSE-IRC in those different bandwidth sizes are relatively small.

Proposal 7: RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and have all DUTs testable regardless of BW support declaration.

- References

- RP-241297, Revised WID on NR demodulation performance: Phase 5, 3GPP RAN#104, China Telecom, NTT DOCOMO, Shanghai, China, June 17-20, 2024

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- 3GPP TR 36.766, Study on interference cancellation receiver for LTE BS, v15.0.0, 07-2017

- 3GPP TS 36.104, Base Station (BS) radio transmission and reception, v18.5.0, 03-2024

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, v18.4.0, 06-2024