TSG-RAN Working Group 4 (Radio) meeting #112	R4-2412912

Maastricht, EU, 19th to 23rd August 2024

Source:	Ericsson

Title:	On the topic of BS RF evolution related to co-location requirements

Agenda item:	8.11.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#111 in Fukuoka) the discussion on evolving the BS RF co-location requirements continued. The outcome from the meeting is captured in a way-forward contribution in [1].

Based on the current status of the discussion it can be noticed that challenges with current Rel-15 requirement concept for BS co-location requirements for NR BS type 1-O will require further discussion to outline clear direction for RAN4. In previous discussion several technical issues have been identified for all requirement using the 30 dB BS-to-BS isolation. Especially, with respect to practical test issues previously identified, but also issues related to the relevance to the requirement for frequencies above 2.5 GHz. It should be noticed that aspects related to the isolation value itself also have implications for requirements defined for NR BS types 1-C and 1-H. For FR2 and NR BS type 2-O, no requirements based on 30 dB isolation have been defined.

In NR BS RF core specification TS 38.104 [5] and MSR BS RF core specifications TS 37.104 [6] and TS 37.105 [7] several requirements are using 30 dB isolation in the background for derivation of the requirement level. The following requirements are referred to as co-location requirements in BS RF specifications.

Table 1-1: Co-location requirement overview

| Requirement | Applicability | Description |
| --- | --- | --- |
| Transmitter ON/OFF power level Subclause 6.4, 9.5 | Mandatory | This requirement specifies a maximum limit of the aggressor wanted carrier emission leakage during the receive period falling within victim receiver and maximum allowed transient period. The requirement also includes a maximum transient period between ON and OFF power. The requirement is defined to protect a neighbouring BS within the same network. |
| Transmitter spurious emission Subclause 6.6.5.2.4, 9.7.5.2.5 | Optional by declaration | This requirement specifies the maximum allowed emission falling within victim co-located BS receiver bandwidth where the victim receiver belongs to another network. |
| Transmitter intermodulation Subclause 6.7, 9.8 | Mandatory | This requirement specifies the victim capability to maintain maximum allowed unwanted emission requirement levels when an aggressor interferer signal is injected. The aggressor BS is operating in another co-located network. |
| Receiver out-of-band blocking Subclause 7.5.3, 10.6.2.2 | Optional by declaration | This requirement specifies the ability to maintain sensitivity when an aggressor interferer signal is injected. The aggressor BS is operating within another co-located network. |

In the work related to BS RF evolution in Rel-19, the Work Item Description (WID) [4] include objectives to study possible improvement and resolve test issues.

For an AAS BS with many transceiver branches, the available volume for the RF filter is reduced for higher frequencies. To minimize routing losses, the RF filters (one per each polarization) need to be fitted behind the sub-array. Therefore, it is of great interest to derive RF requirements based on relevant assumptions. If not, RF filters may be over specified, driving cost and complexity for AAS BS operating in the upper region of FR1 and above.

In this contribution we provide:

- Measurement results indicating that the isolation used for requirement derivation may require some further considerations for frequencies in the upper region of FR1.

- Additional technical background information and suggestion on how to progress the work required to evolve the BS RF co-location requirements. The focus in this contribution will be on transmitter spurious emission and receiver out-of-band blocking.

# Discussion

For base stations, co-location requirements have been defined to guarantee operation of several networks in the same geographical area without degrading the network performance.

For this purpose, two fundamental co-location requirements have been defined in the base station RF core specification:

- Transmitter spurious emission within other 3GPP bands.

- Receiver out-of-band blocking from BS operating in another 3GPP band.

For derivation of Transmitter ON/OFF power level requirement and Transmitter intermodulation requirement the technical background is similar and can be re-used. The requirement levels for these requirements have been established from an assumption of BS-to-BS isolation set to 30 dB.

From the beginning of cellular networks this assumption was derived based on the isolation between two passive base stations located on two masts in close vicinity of each other. During the evolution of cellular networks, another scenario was considered for AAS BS in Rel-15.

After Rel-15, operating bands above 2.5 GHz have been introduced for FR1 and AAS with support for higher order MIMO. This means that the isolation between base stations should be revisited by RAN4 to understand what isolation would be relevant for AAS base stations now and also in the future when spectrum between 7 to 24 GHz is considered.

For the RAN4 requirement derivation purpose, the BS-to-BS isolation is defined as the isolation achieved between the aggressor BS RF port to the victim BS RF port. Hence, the isolation includes aggressor antenna gain, propagation loss and victim antenna gain in a far-field context.

  - Transmitter spurious emission

The requirement background comes from the background that multiple networks operating within different 3GPP bands can be co-located in the same geographical area. This means that the aggressor base station transmitter must protect the victim base station receiver to guarantee operation with negligible performance degradation. For this purpose, 3GPP RAN4 defined a requirement for transmitter spurious emission within other 3GPP bands.

The transmitter spurious emission requirement level was originally derived from calculating emission caused by the aggressor base station within the co-located victim base station in the same geographical area receiver to maintain a maximum allowed co-location receiver sensitivity degradation. Originally, a scenario with two equal base station sites close to each other was considered. The isolation was derived based on two fixed beams from passive BS antennas pointing towards each other. The main beam for each base station was directed towards the ground to achieve coverage.

Based on this assumption the BS-to-BS isolation equal to 30 dB was originally derived. The detailed description can be found in TR 25.942, subclause 10.1 [8]. Based on the BS-to-BS isolation and few other assumptions the requirement level of conducted power at the aggressor base station can be established as shown in Figure 2.1-1.

Figure 2.1-1: Illustration of transmitter spurious emission requirement level derivation

The requirement level for maximum allowed spurious emission for a co-located base station have been calculated in logarithmical scale as:

Pem=kT+B+F-M+I1,	(Eq. 2.1-1)

where kT is the thermal noise equal to -174 dBm/Hz, B is the considered receiver bandwidth in dBHz, F is the noise figure in dB, M is the noise margin corresponding to the allowed victim receiver sensitivity degradation and I1 is the BS-to-BS port isolation in dB.

For co-located networks RAN4 have decided that the victim receiver noise floor can increase 0.5 dB due to a co-located network operating in another band in the same geographical area. The impact to the victim receiver total noise of 0.5 dB corresponds to an emission level M = 9 dB below the victim receiver noise floor. For FR1, RAN4 often assumes a BS noise figure of 7 dB and the BS-to-BS port isolation equal to 30 dB. Then the maximum allowed spurious emission power spectral density can be calculated to -96 dBm/100 kHz. This level is used for requirement levels for NR BS type 1-C and 1-H. It’s also used as background for the requirement level derived for NR BS type 1-O.

Observation 1: The requirement level for transmitter spurious emission level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size.

It would be reasonable to assume that the isolation between base stations will depend on frequency. When the requirement was established, operating bands within the frequency range 800 MHz to 2 GHz were taken into account. Now when FR1 cover the frequency range from 410 MHz to 7125 MHz a more detailed approach to determine the requirement level is needed. It would also be reasonable to assume that the BS-to-BS isolation would increase for situations where the aggressor base station operation frequency is far away from the victim base station operating frequency.

When requirements for AAS BS was established in Rel-15 for NR BS type 1-O the requirement concept was extended to move the requirement to the OTA domain. Unfortunately, RAN4 at the time didn’t reach the goal to define a true OTA requirement. Instead, a concept with a co-location reference antenna was developed. The requirement was anchored at the RF connector of the co-location reference antenna.

With the Rel-15 requirements derived in TR 37.843 [9] a co-location reference antenna based on a single column passive antenna is specified. The concept focused on early versions on AAS BS with antenna apertures with similar sizes are passive base station antennas operating below 2.5 GHz. Now the with larger antennas operating at frequencies above 2.5 GHz the details of the current requirement concept need further discussion.

Different reference points for spurious emission are considered:

- Total conducted spurious emission level at the aggressor, Pem

- Total conducted spurious emission level at the co-location reference antenna (CLTA), Pem,CLTA

In Figure 2.1-2, the aggressor BS and victim BS are visualised with all different reference points. With the Rel-15 concept, the co-location reference antenna is used to mimic the victim BS. In the figure below a scenario considering co-location between AAS BS with AAS BS is considered since that is more relevant for higher frequencies within FR1.

# Figure 2.1-2: Overview of requirement background for transmitter spurious emission requirement assuming side-by-side configuration

In Rel-15, the requirement level at the co-location reference antenna in conformance test specification TS 38.141-2 [10] referred to as the Co-Location Test Antenna (CLTA) was determined as:

Pem, CLTA=Pem-I2+X,		(Eq. 2.1-2)

where I2 is the isolation between the AAS BS and the passive BS antenna (used as CLTA), X is a scaling factor equal to 9 dB added to account additional noise due to 8-port MIMO systems already deployed before Rel-15.

Observation 2: For transmitter spurious emission requirement, it can be noticed that the 9 dB additional noise would change the meaning of the original thought behind the requirement. With 9 dB additional noise, the impact on the victim receiver will be significantly larger than 0.5 dB degradation of the sensitivity.

For the evolution of BS RF requirements, it is proposed to set the scaling factor to 0 dB to guarantee the intention with the requirement. This would also follow guidance from several administrations for general spurious emissions.

In Rel-15 RAN4 decided to assume following: I1=I2=30 dB. By combining Eq. 2.1-1 and Eq. 2.1-2 and assuming I1=I2 it can be noticed that the requirement level at the CLTA is independent of the BS-to-BS port isolation. Hence, the requirement level is completely defined by the victim receiver properties.

Observation 3: With the Rel-15 co-location requirement concept the impact of the BS-to-BS isolation cancel out in the transmitter spurious emission requirement level derivation. This would indicate that the requirement level is not dependent on frequency and antenna size, which is not in line with underlying physics.

Observation 4: To evolve BS co-location transmitter spurious emission requirement to properly support frequencies above 2.5 GHz it is now time for RAN4 to study the technical background for BS co-location requirements to properly capture aspects related to frequency dependence, antenna size dependence and beamforming dependence. The relevance of the current requirement concept is highly questionable, and testability is questionable. The current concept for BS type 1-O cannot support frequencies above 2.5 GHz, where large antenna apertures are used.

  - Receiver out-of-band blocking

The requirement background comes from the background that multiple networks operating within different 3GPP bands can be co-located in the same geographical area. This means that the aggressor base station transmitter will generate an out-of-band interferer for the victim base station. For this purpose, 3GPP RAN4 defined a requirement for receiver out-of-band blocking requirement.

The receiver out-of-band blocking requirement interferer level was originally derived from calculating the interferer level at the victim receiver input assuming a certain BS-to-BS isolation value. Originally, a scenario with two equal base station sites close to each other was considered. The isolation was derived based on two fixed beams from passive BS antennas pointing towards each other. The main beam for each base station was directed towards the ground to achieve coverage.

Based on this assumption the BS-to-BS isolation equal to 30 dB was originally derived. The detailed description can be found in TR 25.942, subclause 10.1. Based on the BS-to-BS isolation and few other assumptions the requirement interferer level at the victim base station can be established as shown in Figure 2.2-1.

Figure 2.2-1: Illustration of receiver out-of-band blocking requirement interferer level derivation

The blocking requirement is based on sensitivity, while an interferer signal is injected. For receiver out-of-band blocking the sensitivity shall not be degraded more than 6 dB when the out-of-band interferer signal is injected.

The requirement interferer level can be calculated in logarithmical scale as:

Pi=Ptx-I1,		(Eq. 2.2-1)

where Ptx is the aggressor base station transmit power and I1 is the BS-to-BS port isolation in dB.

Observation 5: The requirement level for receiver out-of-band blocking interferer level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size.

It would be reasonable to assume that the isolation between base stations will depend on frequency. When the requirement was established, operating bands within the frequency range 800 MHz to 2 GHz were defined. Now, when FR1 covers the frequency range from 410 to 7125 MHz, a more detailed approach to determine the requirement level is needed. It would also be reasonable to assume that the BS-to-BS isolation would increase for situations where the aggressor base station operation frequency is far away from the victim base station operating frequency.

When requirements for AAS BS was established in Rel-15 for NR BS type 1-O, the requirement concept was extended to move the requirement to the OTA domain. Unfortunately, RAN4 at the time didn’t reach the goal to define a true OTA requirement. Instead, a concept with a co-location reference antenna was developed. The requirement was anchored at the RF connector of the co-location reference antenna.

With the Rel-15 requirements a co-location reference antenna based on a single column passive antenna is specified. The concept focused on early versions on AAS BSs with antenna apertures with similar sizes as passive base station antennas operating below 2.5 GHz. Now, the with larger antennas operating at frequencies above 2.5 GHz, the details of the current requirement concept need further discussion.

Different reference points for receiver out-of-band blocking were considered:

- Receiver out-of-band blocking interferer level at the victim receiver input port, Pi .

- Receiver out-of-band blocking interferer level at the co-location reference antenna (CLTA), Pi,CLTA.

In Figure 2.2-2, the aggressor BS and victim BS is visualised with different reference points. With the Rel-15 concept, the co-location reference antenna is used to mimic the aggressor BS. In the figure below a scenario considering co-location between AAS BS with AAS BS is considered since that is more relevant for higher frequencies within FR1.

# Figure 2.2-2: Overview of requirement background for receiver out-of-band blocking requirement assuming side-by-side configuration

In Rel-15, the requirement level derived in TR 37.843 at the co-location reference antenna in conformance test specification TS 38.141-2 referred to as the Co-Location Test Antenna (CLTA) was determined as:

Pi, CLTA=Pi+I2,		(Eq. 2.2-2)

where I2 is the isolation between the AAS BS and the passive BS antenna (used as CLTA).

In Rel-15 RAN4 decided to assume: I1=I2=30 dB. By combining Eq. 2.2-1 and Eq. 2.2-2 and assuming I1=I2 it can be noticed that the requirement level at the CLTA is independent of the BS-to-BS port isolation. Hence, the requirement level is completely defined by the aggressor BS output power level.

Observation 6: With the Rel-15 co-location requirement concept the impact of the BS-to-BS isolation cancel out in the requirement interferer level derivation. This would indicate that the requirement level is not dependent on frequency and antenna size, which is not in line with underlying physics.

Observation 7: To evolve BS co-location receiver out-of-band blocking requirement to properly support frequencies above 2.5 GHz it is now time for RAN4 to study the technical background for BS co-location requirements to properly capture aspects related to frequency dependence, antenna size dependence and beamforming dependence. The relevance of the current requirement concept is highly questionable, and testability is questionable. The current concept for BS type 1-O cannot support frequencies above 2.5 GHz, where large antenna apertures are used.

  - Requirement concept evolution

As pointed out in previous contributions [2, 3, 11] the Rel-15 requirement concept comes with some challenges for frequencies above 2.5 GHz. One way to resolve them would be to define requirement levels in such a way that instead of specifying a unique antenna rather consider specifying OTA parameters such as power density or field strength. This would give more flexibility for test laboratories to select proper antennas to be used to meet specified OTA conditions. Similar concept as used for EMC testing.

To enable usage of BS type 1-O, the co-location requirements above 2.5 GHz needs to be evolved. RAN4 need to go back to and model the isolation properly for relevant antenna configurations and frequency ranges. To be able to properly understand how the BS type 1-O requirement should be defined following aspects needs to be considered in order:

- Co-location scenarios: Define a BS co-location scenario used as reference for requirement. The co-location reference scenario will determine the isolation properties and give guidance to operators on how to deploy networks. The Rel-15 scenario considered co-location between an AAS BS and a non-AAS BS using passive antenna. There are many potential candidate co-location deployment scenarios to consider:

  - Original scenario from TR 25.942.

  - Side-by-side scenario adopted for AAS BS in Rel-15 in TR 37.843.

  - On top of each other, typically used when multiple bands are deployed on the same mast at a site.

  - Or another relevant scenario.

- Frequency ranges: Define frequency range to consider. It would be useful for RAN4 to consider 2.5 GHz as reference and 3.5, 7.0 and 14 GHz. For each frequency range relevant antenna geometries for aggressor BS and victim BS needs to be established. The frequency ranges make it possible to differentiate the requirement over frequency.

- Isolation evaluation: Evaluate BS-to-BS isolation using EM-simulation tools. The EM simulations can be simplified by slitting up the simulation for array with only elements without RDN and beamforming applied. Then the RDN and beamforming can be applied using circuit theory as post processing. Since now AAS BS is considered, the isolation dependence due to beamforming needs to be captured. This would require isolation to be considered when the BS beamform towards UEs on the ground. Based on statistics over many beamforming cases the relevant isolation can be established.

- Conducted requirement level: Derive requirement levels for each frequency range and combinations of frequency ranges. The requirement level can be established using Eq. 2.1-1 for transmitter spurious emission and Eq. 2.2-1 for receiver out-of-band blocking, considering the isolation as a function of co-location scenario, frequency, antenna and beamforming of aggressor BS and victim BS.

- OTA requirement level: Investigate OTA parameter candidates, e.g., power density, or field strength, which is suitable to replace CLTA, and specify OTA parameter to be measured. Based on Pem for transmitter spurious emission and Pi for receiver out-of-band blocking derive the level for the specified OTA parameter. In the process additional information such as array configuration may be needed. The expected output would be a 4x4 matrix with requirement values for different combinations of victim frequency and aggressor frequency.

Proposal 1: Based on observed technical issues with current definition of transmitter spurious emission requirement and receiver out-of-band blocking requirement for BS type 1-O adopt the workflow above to enable an evolution for BS RF co-location requirements supporting larger array antenna structures operating at frequencies above 2.5 GHz.

It can also be noticed that if a new concept is adopted for co-location requirements also requirements for BS type 1-C, 1-H may need to be evolved accordingly.

  - Measurement results

To evaluate the isolation between the AAS BS and the CLTA access to the Transceiver Array Boundary (TAB) interface is required. The details for the test object considered in this contribution is listed in Table 2.4-1.

Table 2.4-1: Test object description

| Parameter | Value |
| --- | --- |
| Sub-array configuration | 2x1 |
| Array configuration | 4x8 |
| Transceiver configuration | 64TX/64RX |
| Operating frequency range | Band 41 (2490 to 2690 MHz) |

For this purpose, the radio electronics have been removed and only ¼ of the array antenna is considered for practical reasons. This setup will indicate the isolation between the TAB RF interface and the CLTA, considering no radio electronics and enclosure. It is expected that this setup will produce less isolation than what is expected for a complete product.

To evaluate the BS-to-BS isolation different scenarios (Array-to-Subarray isolation and Array-to-Array isolation) have been considered for two different types of CLTA with different length.

Table 2.4-2: Measurements

| Scenario | Description |
| --- | --- |
| A | Array-to-Subarray isolation, 1.3 m CLTA |
| B | Array-to-Subarray isolation, 0.3 m CLTA |
| C | Array-to-Array isolation, 1.3 m CLTA |
| D | Array-to-Array isolation, 0.3 m CLTA |

The test scenarios are visualised in Figure 2.4-1 and Figure 2.4-2.

# Figure 2.4-1: Test configuration, Scenario A and Scenario B

For test scenario A and B, the isolation is measured between a sub-array and CLTA RF port. The considered sub-array is marked within the orange box in Figure 2.4-1.

# Figure 2.4-2: Test configuration, Scenario C and Scenario D

For test scenario C and scenario D the isolation between composite AAS array antenna and the CLTA RF port is considered. For scenario A and scenario B the isolation is measured for 2625 MHz, 2605 MHz and 2585 MHz within in band 41. To capture out-of-band characteristics, isolation is measured also for 2355 MHz and 1840 MHz. Isolation is measured for co-polarization and x-polarization combinations. For scenario C and scenario D the isolation is measured for 2625 MHz for co-polarization only.

To avoid interference from external sources (other networks) and reflections, all measurements was conducted in a shielded anechoic chamber. The test setup for Scenario A and Scenario B is visualised in Figure 2.4-3.

# Figure 2.4-3: Test setup for Scenario A and Scenario B inside an anechoic chamber

The measurement results for scenario A, scenario B, scenario C and scenario D are listed in Table 2.4-3, Table 2.4-4, Table 2.4-5 and Table 2.4-6.

Table 2.4-3: Measurement results for Scenario A

| Frequency (MHz) | Polarization CLTA (+45 or -45 degrees) | Polarization AAS (+45 or -45 degrees) | Isolation (dB) |
| --- | --- | --- | --- |
| 2625 | +45 | +45 | 38.7 |
| 2605 | +45 | +45 | 39.8 |
| 2585 | +45 | +45 | 41.7 |
| 2355 | +45 | +45 | 42.6 |
| 1840 | +45 | +45 | 53.4 |
| 2625 | -45 | -45 | 38.0 |
| 2605 | -45 | -45 | 39.0 |
| 2585 | -45 | -45 | 40.3 |
| 2355 | -45 | -45 | 44.2 |
| 1840 | -45 | -45 | 53.1 |
| 2625 | -45 | +45 | 49.7 |
| 2605 | -45 | +45 | 54.1 |
| 2585 | -45 | +45 | 60.5 |
| 2355 | -45 | +45 | 51.9 |
| 1840 | -45 | +45 | 55.6 |
| 2625 | +45 | -45 | 45.9 |
| 2605 | +45 | -45 | 44.6 |
| 2585 | +45 | -45 | 44.5 |
| 2355 | +45 | -45 | 45.8 |
| 1840 | +45 | -45 | 57.7 |

It can be observed that for scenario A, isolation in all cases is larger than 30 dB. For out-of-band situations the isolation is 10 to 20 dB higher than what is currently assumed for requirement derivation.

Observation 8: The measured isolation considering two base stations operating within the same band is generally larger than 30 dB.

Observation 9: The measured isolation considering two base stations operating in different bands is very much larger than 30 dB.

Table 2.4-4: Measurement results for Scenario B

| Frequency (MHz) | Polarization CLTA (+45 or -45 degrees) | Polarization AAS (+45 or -45 degrees) | Isolation (dB) |
| --- | --- | --- | --- |
| 2625 | -45 | -45 | 36.3 |
| 2605 | -45 | -45 | 35.6 |
| 2585 | -45 | -45 | 35.3 |
| 2355 | -45 | -45 | 48.0 |
| 1840 | -45 | -45 | 53.7 |

As expected, the isolation reduces due to the shorter CLTA antenna. For this case the power is not spread out as much as for scenario A.

Results related to scenario C and D have previously been presented in [12].

Table 2.4-5: Measurement results for Scenario C

| Frequency (MHz) | AAS beam direction (degrees) | Polarization CLTA (+45 or -45 degrees) | Polarization AAS (+45 or -45 degrees) | Isolation (dB) |
| --- | --- | --- | --- | --- |
| 2625 | 0,0 | -45 | -45 | 58.0 |
| 2625 | 0,60 | -45 | -45 | 46.0 |
| 2625 | 0,-60 | -45 | -45 | 47.0 |

Table 2.4-6: Measurement results for Scenario D

| Frequency (MHz) | AAS beam direction (degrees) | Polarization CLTA (+45 or -45 degrees) | Polarization AAS (+45 or -45 degrees) | Isolation (dB) |
| --- | --- | --- | --- | --- |
| 2625 | 0,0 | -45 | -45 | 58.0 |
| 2625 | 0,60 | -45 | -45 | 47.0 |
| 2625 | 0,-60 | -45 | -45 | 45.0 |

As expected, it can be observed that the isolation considering the complete array increase. It can also be seen that the isolation varies due to beam steering direction.

Observation 10: The measured isolation considering composite AAS beam may vary depending on considered beam direction.

It should be emphasized that these results are presented only as an indication of the actual isolation levels and at this stage no conclusions can be drawn for what the values in the standard should be.

In this contribution we present measurement results for the isolation between an AAS BS array antenna and Co-location Reference Antenna (CLTA). The intention is to bring more light to the discussion on what isolation level to assume for co-location requirements above 2.5 GHz in the upper region of FR1 and above. By tradition RAN4 have assumed a fixed value of 30 dB for all situations for co-location requirement derivation. It’s now clear that this approach is not suitable for AAS BS supporting frequencies above 2.5 GHz. With current approach the requirements will set constraints in such way that implementations will be complex and costly. Instead, requirements should be defined in such way that inherent isolation due to beamforming and larger antenna apertures is included in the requirement background.

# Conclusion

In this contribution we present the background for two BS RF co-location requirements: Transmitter spurious emission and Receiver out-of-band blocking. It can be noticed that it is not obvious how to add support for frequencies above 2.5 GHz, since both frequency dependence and antenna array size is not part of the technical background for co-location requirements.

Based on observed technical challenges we propose a way forward to evolve the BS RF co-location requirements. It is essential to understand the impact on BS-to-BS isolation due to higher frequencies, larger array antenna size and beam forming aspects. It is therefore proposed to study base station co-location isolation for different typical deployment scenarios to better understand what isolation that is relevant to use for co-location requirement in the future. The way forward can be summarized as visualised in Figure 3-1.

# Figure 3-1: Requirements concept evolution

In this contribution we observed following:

Observation 1: The requirement level for transmitter spurious emission level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size.

Observation 2: For transmitter spurious emission requirement, it can be noticed that the 9 dB additional noise would change the meaning of the original thought behind the requirement. With 9 dB additional noise, the impact on the victim receiver will be significantly larger than 0.5 dB degradation of the sensitivity.

Observation 3: With the Rel-15 co-location requirement concept the impact of the BS-to-BS isolation cancel out in the transmitter spurious emission requirement level derivation. This would indicate that the requirement level is not dependent on frequency and antenna size, which is not in line with underlying physics.

Observation 4: To evolve BS co-location transmitter spurious emission requirement to properly support frequencies above 2.5 GHz it is now time for RAN4 to study the technical background for BS co-location requirements to properly capture aspects related to frequency dependence, antenna size dependence and beamforming dependence. The relevance of the current requirement concept is highly questionable, and testability is questionable. The current concept for BS type 1-O cannot support frequencies above 2.5 GHz, where large antenna apertures are used.

Observation 5: The requirement level for receiver out-of-band blocking interferer level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size.

Observation 6: With the Rel-15 co-location requirement concept the impact of the BS-to-BS isolation cancel out in the requirement interferer level derivation. This would indicate that the requirement level is not dependent on frequency and antenna size, which is not in line with underlying physics.

Observation 7: To evolve BS co-location receiver out-of-band blocking requirement to properly support frequencies above 2.5 GHz it is now time for RAN4 to study the technical background for BS co-location requirements to properly capture aspects related to frequency dependence, antenna size dependence and beamforming dependence. The relevance of the current requirement concept is highly questionable, and testability is questionable. The current concept for BS type 1-O cannot support frequencies above 2.5 GHz, where large antenna apertures are used.

Observation 8: The measured isolation considering two base stations operating within the same band is generally larger than 30 dB.

Observation 9: The measured isolation considering two base stations operating in different bands is very much larger than 30 dB.

Observation 10: The measured isolation considering composite AAS beam may vary depending on considered beam direction.

Based on these observations, RAN4 need to carefully study the background for co-location requirements defined in the upper range of FR1 and above.

In this contribution we propose:

Proposal 1: Based on observed technical issues with current definition of transmitter spurious emission requirement and receiver out-of-band blocking requirement for BS type 1-O adopt the workflow above to enable an evolution for BS RF co-location requirements supporting larger array antenna structures operating at frequencies above 2.5 GHz.

# References

[1]	R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE

[2] 	R4-2408086, “Evolution of BS co-location requirements”, Ericsson

[3]	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson

[4]	RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE

[5]	TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

[6]	TS 37.104, “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”, 3GPP

[7]	TS 37.105, “Active Antenna System (AAS) Base Station (BS) transmission and reception”, 3GPP

[8]	TR 25.942, “Radio Frequency (RF) system scenarios”, 3GPP

[9]	TR 37.843, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA); Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements”, 3GPP

[10]	TS 38.141-2, “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”, 3GPP

[11]	R4-2411872, “On general aspects related to BS RF evolution”, Ericsson

[12]	R4-1808875, “On co-location reference antenna definitions including practical measurements”, Ericsson