3GPP TSG-RAN WG4 Meeting #112	R4-2412913

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.19.1

Source:	Ericsson

Title:	SBFD general aspects

Document for:	Approval

# 1	Introduction

In this contribution, we present our views and proposals on general aspects for SBFD capable BS regarding band support for SBFD capable BS, CLI handling impact on RF requirement, specification structure, and SBFD sub-band configuration. Comprehensive analysis of each SBFD BS RF TX/RX requirement and potential new requirements for SBFD capable BS are detailed in separate submissions [1-3].

# 2	Discussion

Band support

#### Issue 1-1-1: SBFD as band specific or general feature to all TDD bands - Agreement: - SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded.

In RAN4#111, whether SBFD should be a feature as band specific or general to all TDD bands was discussed offline, and the agreement was captured in the WF as listed above.

In our opinion, for SBFD slots, guardband play a crucial role in preventing interference between adjacent sub-bands. However, excessively wide guardband can consume valuable bandwidth. Considering the case that the bandwidth is low, after excluding the guard band, the remaining PRBs for both DL and UL sub-bands may become insufficient. Such a situation is not practical in real-world deployments. For example, the bandwidth of n54 is 5 MHz, deploy SBFD in such band would not bring any gain in practice. Therefore, we think a general principle should be applied when deploying SBFD: The channel bandwidth shall be larger than 50 MHz.

Wide guardband can consume valuable bandwidth. When the bandwidth is low, after excluding the guard band, the remaining PRBs for both DL and UL sub-bands may become insufficient. Such a situation is not practical in real-world deployments.

A general principle should be applied when deploying SBFD: The channel bandwidth shall be larger than 50 MHz.

CLI handling

#### Issue 1-1-6: CLI handling impact on RF requirement - Agreement: - The CLI handling impact on RF requirement: - Option 1: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline. - Option 2: FFS CLI handling impact on RF requirements

Co-channel CLI mitigation schemes discussed in RAN1 can only alleviate interference received from the DL of the same operator BSs, and they do not have an impact on the CLI received from the DL of an adjacent channel network deployment.

On the other hand, there is no adjacent channel CLI mitigation scheme, as it would require assuming the possibility that other operators will be willing to beam null in the direction of a neighbor operator UL SBFD victim receiver, at the expense of their DL performance. Also, this assumption would require exchange of information between different operators, which was agreed in RAN plenary not to be considered in the context of this WI for different reasons.

RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline.

Specification structure

#### Issue 1-4-1: BS RF Specification structure for SBFD requirements - Agreement: - How to introduce BS RF new requirements for SBFD-capable BS: - FFS firstly the feasibility of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements - FFS detailed how to implement such approach in the later phase of WI. - FFS secondly the new specification for SBFD-capable BS

We prefer to create new specifications for SBFD-capable BS, rather than merging it into TS 38.104.

A dedicated SBFD spec ensures clarity. It includes only relevant information related to bands, bandwidth, and SBFD configurations, etc., and avoids unnecessary complexity and provides a concise, focused reference for SBFD-related details, which makes it easier to understand and maintain. Creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements could reduce the number of 3GPP specifications, but increase the complexity and reduce the readability of TS 38.104, as what we have on UE side. On the other hand, SBFD-capable BS may require new hardware components, which makes it different from legacy base stations.

RAN4 create new specifications for SBFD-capable BS.

SBFD sub-band configuration

#### Issue 1-2-2: How to handle guard band and subband configurations in specification - Agreement: - Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1)

It has been agreed that only DUD and DU configurations are specified for the SBFD sub-band configuration.

As justified in band support topic in this T-doc, we propose to support SBFD with channel bandwidth larger than 50 MHz. In the SI phase, the study was based on the assumption that UL subband occupies 20% BS channel bandwidth. Hence, we propose to continue with the assumption that UL subband occupies 20% BS channel bandwidth for FR1 in the WI phase. To be able to reuse the existing channel bandwidths for DL as well as UL subbands, the channel bandwidths are further narrowed down to 50 MHz and 100 MHz for FR1.

For FR2, we propose that UL subband occupies 25% BS channel bandwidth for FR2-1, and do not define anything for FR2-2, which is out of the scope of WI. For FR2-1, for a similar reason of reusing the existing channel bandwidths for UL subband, the channel bandwidths are down selected to 200 MHz and 400 MHz.

To have a straightforward view, the proposed supported BS channel bandwidth and possible subband configurations are listed in table 1 and table 2.

Table 1: BS SBFD configuration for FR1

| Subband allocation | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 50 | 100 | 100 |
| DL Subband 1 | 20 | 40 | 40 | 80 |
| UL Subband | 10 | 10 | 20 | 20 |
| DL Subband 2 | 20 | N/A | 40 | N/A |

Table 2: BS SBFD configuration for FR2-1

| Subband allocation | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 200 | 200 | 400 | 400 |
| DL Subband 1 | 75 | 150 | 150 | 300 |
| UL Subband | 50 | 50 | 100 | 100 |
| DL Subband 2 | 75 | N/A | 150 | N/A |

Reuse the existing channel bandwidths for SBFD subbands.

SBFD-capable BS channel bandwidths are down selected to 50 MHz and 100 MHz for FR1, 200 MHz and 400 MHz for FR2-1. RAN4 define SBFD subband configurations as shown in table 1 for FR1 and table 2 for FR2-1.

Additionally, transmission bandwidth configuration NRB for SBFD can also reuse the existing ones defined in table 5.3.2-1 and table 5.3.2-2 in TS 38.104.

As to sub carrier spacing (SCS), for FR1, according to UE demodulation requirement, 30 kHz SCS is selected for TDD and 15 kHz SCS is selected for FDD. Considering SBFD is supported in TDD operation, 30 kHz shall be the SCS for SBFD slots in FR1. For FR2-1, 120 kHz is proposed to be the SCS for SBFD slots.

For FR1, use 30 kHz as the SCS for SBFD slots. For FR2-1, use 120 kHz as the SCS for SBFD slots.

Follow the proposals above, 80% of channel bandwidth is used for DL for FR1, and 75% of channel bandwidth is used for DL for FR2-1. SBFD transmission bandwidth configuration NRB for FR1 are calculated and shown in table 3, which are 0.8 times the corresponding NRB values in table 5.3.2-1. SBFD transmission bandwidth configuration NRB for FR2-1 are shown in table 4, which are 0.75 times the corresponding NRB values in table 5.3.2-2.

Table 3: SBFD Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 50 MHz | 100 MHz |
| --- | --- | --- |
|  | NRB | NRB |
| 30 | [106] | [218] |

Table 4: SBFD Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 200 MHz | 400 MHz |
| --- | --- | --- |
|  | NRB | NRB |
| 120 | [99] | [198] |

Reuse the existing transmission bandwidth configuration NRB for SBFD.

RAN4 define SBFD transmission bandwidth configuration NRB as shown in table 3 for FR1 and table 4 for FR2-1.

As to the guardband size, since different BS classes require different techniques to suppress interference, for example, WA BSs would require digital filtering, analogue filtering and linearity of the receiver, etc., while for LA BSs there is no need to build analogue filter and linear receiver, the guardband size can be different for different BS classes. Therefore, our view is that, for SBFD-capable LA BSs, the guardband sizes can be reused; for SBFD-capable WA BSs, wider guardbands (than the guards used for BS in normal operation) are needed, and should be further studied, due to the fact that, a determined test model is needed for conformance testing, instead of declaration based.

For LA BSs, reuse the existing guardbands for SBFD. For WA BSs, wider guardbands than existing ones are needed for SBFD and need FFS.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.

Observation 1	Wide guardband can consume valuable bandwidth. When the bandwidth is low, after excluding the guard band, the remaining PRBs for both DL and UL sub-bands may become insufficient. Such a situation is not practical in real-world deployments.

Proposal 1	A general principle should be applied when deploying SBFD: The channel bandwidth shall be larger than 50 MHz.

Proposal 2	RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline.

Proposal 3	RAN4 create new specifications for SBFD-capable BS.

Proposal 4	Reuse the existing channel bandwidths for SBFD subbands.

Proposal 5	SBFD-capable BS channel bandwidths are down selected to 50 MHz and 100 MHz for FR1, 200 MHz and 400 MHz for FR2-1. RAN4 define SBFD subband configurations as shown in table 1 for FR1 and table 2 for FR2-1.

Proposal 6	For FR1, use 30 kHz as the SCS for SBFD slots. For FR2-1, use 120 kHz as the SCS for SBFD slots.

Proposal 7	Reuse the existing transmission bandwidth configuration NRB for SBFD.

Proposal 8	RAN4 define SBFD transmission bandwidth configuration NRB as shown in table 3 for FR1 and table 4 for FR2-1.

Proposal 9	For LA BSs, reuse the existing guardbands for SBFD. For WA BSs, wider guardbands than existing ones are needed for SBFD and need FFS.

# References

R4-2412915, Impact on SBFD BS RF TX requirements, Ericsson

R4-2412916, Impact on SBFD BS RF RX requirements, Ericsson

R4-2412914, Potentially new SBFD BS RF requirements, Ericsson