3GPP TSG-RAN WG4 Meeting #112	R4-2412914

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.19.2.1

Source:	Ericsson

Title:	Potentially new SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of Potentially new BS RF requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on impact on existing BS RF TX and RX requirements for SBFD slots and SBFD general aspects are presented in separate submissions [2-4].

# 2	Discussion

Transmitter transient period

#### Issue 4-3-1: Requirement for transient period between SBFD and non-SBFD - Agreement: - FFS requirement and cases for transient period between SBFD and non-SBFD: - Option 1: The existing TDD BS transmitter transient period, i.e., 10us for FR1 and 3us for FR2-1 - Other options are not precluded. - FFS location of transient period between SBFD and non-SBFD: - Option 1: The transmitter transient period shall be located within the SBFD slot (symbol) - Other options are not precluded. #### Issue 4-3-2: Requirement for transient period between different SBFD configurations - Agreement: - FFS requirement for transient period between different SBFD configurations

In the WF [5], there are two issues related to the potentially new requirement of transmitter transient period.

Regarding Issue 4-3-1, conventional TDD contains DL-UL and UL-DL switching. For SBFD, there may be DL-SBFD, SBFD-DL, UL-SBFD and SBFD-UL switching.

The overall switching time budget for TDD needs to be designed to prevent cross-link interference after the switch due to propagation time between different cells. For example, the TX signal from one gNB may propagate to a nearby gNB That has already switched to RX and cause interference if the guard period is not enough.

When SBFD is switched, then at least part of the gNB antenna array will be switching between transmit and receive. SBFD needs to be operated in the same configuration across all nodes in the network, and the same guard period dimensioning considerations are needed to prevent cross link interference in the SBFD UL RBs between gNB due to propagation of the TX signal in the SBFD RBs.

Since the overall TX switching time budget needs to be the same both for switching the full UL/DL and the SBFD RBs then in principle the transient time should also be the same both for full TDD switching and for SBFD related switching.

- The same considerations on inter-site interference due to switching occur for SBFD resources when switched between TX/RX as when the whole slot is switched.

Apply the existing TDD switching time and off level requirement to SBFD RBs when they are switched between TX and RX.

Apply the same transient period to transition between non-SBFD slots and SBFD slots as for normal full DL and UL switching.

Regarding Issue 4-3-2, one of the objectives of Core part WI in the latest revised WID [RP-241614] approved in RAN#104 is as follows,

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

Obviously, dynamic SBFD is precluded from the WI, i.e., there is no need to specify transmitter transient period requirement between different SBFD configurations.

There is no need to specify transmitter transient period requirement between different SBFD configurations.

In-channel adjacent sub-band leakage ratio

For the potential new requirement of in-channel adjacent subband leakage ratio, it is concluded that further study is needed on the necessity of this requirement in normative phase.

In TR 38.858 [1], the necessity of In-channel adjacent sub-band leakage ratio requires further study.

Even though conformance testing does not include other sites, a new requirement needs to be set to enable dimension site design in real networks. While self-interference is captured in the OTA sensitivity, interference from other sectors and other sites need to be considered, i.e., requirements on ACLR and ACS between the sub-bands are needed, which is the basis for dimensioning site design. Moreover, in the feasibility analysis, ACLR and ACS suppression between sub-bands were assumed by all companies.

Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

In-channel adjacent sub-band Blocking and adjacent sub-band selectivity

For the potential new requirements of in-channel adjacent subband blocking and selectivity, it is concluded that further study is needed on the necessity of this requirement in normative phase.

In TR 38.858 [1], the necessity of In-channel adjacent sub-band Blocking and adjacent sub-band selectivity requires further study.

The definition of Adjacent channel selectivity (ACS) in TS 38.104 [6] is,

a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

The definition of in-band blocking in TS 38.104 [6] is,

a measure of the receiver's ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

From the definition, we know that ACS is the requirement to consider an unwanted interferer (e.g., -52 dBm for WA) from the adjacent channel, while in-band blocking is the requirement to consider an unwanted interferer at a higher level comparing to ACS (e.g., -43 dBm for WA) within the same frequency band. In SBFD, there is only one adjacent DL sub-band, which makes adjacent sub-band Blocking and adjacent sub-band selectivity somewhat the same requirements with two different interferer levels. Therefore, we propose to define a new requirement on RX sub-band ACS similar to the ACS requirement only and use existing ACS requirement as baseline.

Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	The same considerations on inter-site interference due to switching occur for SBFD resources when switched between TX/RX as when the whole slot is switched.

Proposal 1	Apply the existing TDD switching time and off level requirement to SBFD RBs when they are switched between TX and RX.

Proposal 2	Apply the same transient period to transition between non-SBFD slots and SBFD slots as for normal full DL and UL switching.

Proposal 3	There is no need to specify transmitter transient period requirement between different SBFD configurations.

Proposal 4	Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

Proposal 5	Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2412913, Discussion on SBFD general aspects, Ericsson

R4-2412915, Impact on SBFD BS RF TX requirements, Ericsson

R4-2412916, Impact on SBFD BS RF RX requirements, Ericsson

R4-2409889, Way Forward for [110bis][313] NR_duplex_evo, Moderator (Samsung)

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP