3GPP TSG-RAN WG4 Meeting #112	R4-2412915

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.19.2.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on existing BS RF RX requirements for SBFD slots is presented in R4- 2412916 [2], potentially new requirements for SBFD slot is presented in R4- 2412914 [3], and SBFD general aspects is presented in R4- 2412913 [4].

# 2	Discussion

Output power dynamics

Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus: - To reuse the existing RE power control dynamic range requirement for SBFD BS; - The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, it is agreed to define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

- Reuse the existing RE power control dynamic range requirement for SBFD BS.

- The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots.

- Define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Following the consensus reached in RAN4 during the SI phase, total power dynamic range requirement for SBFD is defined as the ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

The existing total power dynamic range requirement for non-SBFD slots is given in a table in TS 38.104 [5]: the downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.3.3.2-1 for BS type 1-C, BS type 1-H and BS type 1-O.

Table 6.3.3.2-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 3 | 11.7 | N/A | N/A |
| 5 | 13.9 | 10.4 | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |
| 25 | 21.2 | 18.1 | 14.9 |
| 30 | 22 | 18.9 | 15.7 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 23.3 | 20.2 | 17 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 24.3 | 21.2 | 18.1 |
| 60 | N/A | 22 | 18.9 |
| 70 | N/A | 22.7 | 19.6 |
| 80 | N/A | 23.3 | 20.2 |
| 90 | N/A | 23.8 | 20.8 |
| 100 | N/A | 24.3 | 21.3 |

For SBFD slots, considering that the SBFD configurations, which is still under discussion, may be different for FR1 and FR2, and different SBFD configurations, i.e., total number of DL sub-band RBs, will generate separate cases of total power dynamic range, when formulate the requirement in the core specification, instead of using tables, we propose to use equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD).

Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement.

Transmit ON/OFF power

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

- Transmitter ON/OFF power doesn’t apply to SBFD slot.

Transmitter ON/OFF power should apply to normal slot.

Transmitted signal quality

Regarding the transmitter signal quality, RAN4 agreed that all the existing requirement for frequency error, modulation quality (EVM) and time alignment error (TAE) shall also be applied to BS in SBFD symbols/slots. - Further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots during WI phase.

- Reuse the existing requirements for frequency error, EVM and TAE for BS in SBFD symbols/slots.

- Measurement of average EVM for BS in normal DL symbols/slots and SBFD DL symbols/slots needs FFS.

Calculation of averaged EVM is defined in Annex B.7 for FR1 in TS 38.104 [5]:

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

Following the definition above, averaged EVM is over all allocated downlink resource blocks. Therefore, regarding the calculation of averaged EVM for SBFD slots, the data samples where ULs are allocated within the SBFD slots shall be removed from the calculation, and only PRBs carrying DL within SBFD slots shall be calculated.

Considering the averaged EVM for SBFD slots may be different from the averaged EVM for non-SBFD slots, in our opinion, they should be measured separately.

Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Unwanted emissions

#### Issue 2-3-1: The necessity of Co-location ACLR requirement - Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement. #### Issue 2-3-2: OBUE - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

In the approved WF [6] in RAN4#111, there are two open issues related to unwanted emissions that require further study. The proposals involve a new co-existence scenario: a SBFD-capable BS co-exists with another operator’s SBFD-capable BS. The proponents recommend that RAN4 consider more stringent ACLR and OBUE requirements to ensure the co-existence of these two SBFD-capable BSs. However, since the co-existence of two SBFD-capable BSs is a corner case, and the new scenario was not considered in the co-existence study performed in the SI phase, we believe it is unnecessary to impose tougher ACLR and OBUE requirements beyond the existing ones.

RAN4 do not impose tougher ACLR and OBUE requirements beyond the existing ones.

# Conclusion

In this contribution, overview of impact on SBFD BS RF TX requirements is presented with the following observations and proposals.

Observation 1	Reuse the existing RE power control dynamic range requirement for SBFD BS.

Observation 2	The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots.

Observation 3	Define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Observation 4	Transmitter ON/OFF power doesn’t apply to SBFD slot.

Observation 5	Reuse the existing requirements for frequency error, EVM and TAE for BS in SBFD symbols/slots.

Observation 6	Measurement of average EVM for BS in normal DL symbols/slots and SBFD DL symbols/slots needs FFS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement.

Proposal 2	Transmitter ON/OFF power should apply to normal slot.

Proposal 3	Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Proposal 4	Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Proposal 5	RAN4 do not impose tougher ACLR and OBUE requirements beyond the existing ones.

# References

TR 38.858, Study on evolution of NR duplex operation

R4- 2412916, Impact on SBFD BS RF RX requirements, Ericsson

R4- 2412914, Potentially new SBFD BS RF requirements, Ericsson

R4- 2412913, Discussion on SBFD general aspects, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2409889, Way Forward for [110bis][313] NR_duplex_evo, Moderator (Samsung)