3GPP TSG-RAN WG4 Meeting #112	R4-2412916

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.19.2.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF RX requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on existing BS RF TX requirements for SBFD slots is presented in R4-2412915 [2], potentially new requirements for SBFD slot is presented in R4-2412914 [3], and SBFD general aspects is presented in R4-2412913 [4].

# 2	Co-existence simulation for in-band blocking

2.1	Simulation assumption

In RAN4#111 meeting, the necessity to perform further adjacent channel simulations was discussed. In the WF [5], the agreements are summarized as follows:

#### Issue 3-4-1 & Issue 3-4-2: Necessity for additional co-existence study for In-band blocking and Scenario/Case for additional co-existence study (if agreed) - Way forward: - Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting: - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858 - The case to be studied is Case 3 (TDD DL to SBFD UL subband). - FFS the power level which should be collected from the simulation results to derive in-band blocking levels: - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement. - Other options are not precluded. - FFS the reference point for the power level which should be before or after RX beamforming. - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB. - FFS different grid-shift values should be considered. - Option-1: 10%. - Option-2: Other percentage values are not precluded. - FFS how to use the power level probability distributions to derive the in-band blocking levels - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios. - FFS the value of X - FFS the impact of CLI handling scheme introduced in RAN1.

As agreed in the WF during RAN4#111 meeting [5], the desired scenarios for which to conduct this evaluation are scenarios 1, 3, 5, 6, 9, in order to derive relevant in-band blocking requirements for different BS classes. Scenarios are referenced in Table 11.1-1 from TR 38.858. The simulation case of interest is Case 3, as defined in Table 11.1-2 in TR 38.858, since in this case SBFD UL is the victim and the aggressor is the DL of TDD, so the blockers are the BS of the adjacent operator.

Table 11.1-1 (from TR 38.858): Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11.1-2 (from TR 38.858): Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  |
| 2 | SBFD | TDD UL |  |
| 3 | TDD DL | SBFD |  |
| 4 | TDD UL | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |

About the options that are FFS we consider the following assumptions:

- About the power level which should be collected from the simulation results to derive in-band blocking levels, we support option 1:

Absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement.

- About different grid-shift values, we believe different grid-shift values should be considered.

The worst-case scenario is a grid-shift as small as possible, to account for uncoordinated deployments. We propose to consider 5%, 10% and 100%.

- About the reference point for the power level which should be before or after RX beamforming, we support option 1:

After RX subarray beamforming gain and before array gain, i.e, at TAB.

- About how to use the power level probability distributions to derive the in-band blocking levels, we support option 1:

Find the X% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios. We propose X to be 99.

- About the option to consider the impact of CLI handling scheme introduced in RAN1, we do not support this option:

No adjacent channel CLI (cross-link interference) mitigation scheme should be considered in the evaluation of the in-band blocking requirements, as it would require assuming the possibility that other operators will be willing to mitigate CLI caused by them to neighbor SBFD operators, e.g., by performing beam nulling in the direction of a neighbour operator’s UL SBFD victim receiver, at the expense of their DL performance. Also, this assumption would require specification impact to facilitate exchange of information between different operators, which was agreed in RAN plenary not to be considered in the context of this WI for different reasons. In addition, RAN1 is focusing on co-channel CLI handling schemes, and when setting in-band blocking requirements for BSs, the focus is primarily on external sources of interference, from adjacent channel. Co-channel CLI mitigation schemes can only alleviate blocking issues caused by interference received from the DL of the same operator BSs. Therefore, co-channel CLI mitigation schemes should not be considered in the in-band blocking requirements evaluation, as they do not have an impact on the CLI received from the DL of an adjacent channel network deployment. Moreover, in the procedure to derive the in-band blocking requirements, (see point 1), the co-channel CLI is not considered, as the internal co-channel interference from the SBFD network should be removed in the evaluation and the focus should only be on the wideband power received from the adjacent channel in the receiver of the UL SBFD.

2.2	Discussion of simulation results

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR1 scenarios: scenarios 1 (urban macro), 3 (indoor hotspot), 5 (Micro). The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858, and are also summarized in the Annex.

2.2.1 Urban Macro

We consider a coexistence scenario where urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%. The grid-shift examples are depicted in Figure 2.2.1-1.

Figure 2.2.1-1: Grid-shift 10% and 100% between two UMa network deployments

Two different values of BS TRP are considered 1) 49 dBm, which is the default value in the assumptions of the SI; 2) 53 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.1-2.

Figure 2.2.1-2: UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.1-2 it can be observed that the in-band blocking requirement is higher as the TRP increases and the grid-shift decreases. In addition, it can be observed that when considering TRP 49 dBm and grid-shift 100% (best case assumptions), we are already around 20 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to 5 dBm for Wide Area BSs in SBFD networks.

2.2.2 Urban Micro

We consider a coexistence scenario where urban micro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Two different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) a worst case scenario where BSs of the two operators are very close to each other with grid-shift 5% and 10%. The grid-shift examples are depicted in Figure 2.2.2-1.

Figure 2.2.2-1: Grid-shift 10% and 100% between two UMi network deployments

Two different values of BS TRP are considered 1) 46 dBm, which is the default value in the assumptions of the SI; 2) 38 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.2-2.

Figure 2.2.2-2: UMi: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.2-2 it can be observed that the in-band blocking requirement is higher as the TRP increases and the grid-shift decreases.

Current in-band blocking level for Medium Range BS is -38 dBm, so we can observe that when we consider lower power levels and coordinated deployments (100% grid-shift), there would not be need for higher in-band blocking requirements. However, when the transmission power is higher or the grid-shift is reduced, more stringent requirements would be needed.

- In UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments may not need an update in requirements, but uncoordinated deployments may need it.

Define in-band blocking interferer signal power to -1.5 dBm for Medium Range BSs in SBFD networks.

2.2.3 Indoor office

We consider a coexistence scenario where indoor office deployments of two different operators are deployed in the same building and in adjacent channels, as shown in Figure 2.2.3-1. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Figure 2.2.3-1: InH deployment with two coordinated operators

The indoor scenario facilitates coordination of the two operators, compared to outdoor deployments, so that it is inherently coordinated with ISD= 20 mt. The BS transmission power is fixed at 24 dBm. The maximum UE power is set to 23 dBm, so that there is not a big difference between UEs and BS in terms of potential in-band blocking level. This results in that the current requirement is still valid. Figure 2.2.3-2 shows the UL received wideband power from the neighbour operator. For comparison we consider the case where the victim is the UL of SBFD, and the in-band blocking level is represented by the TDD BSs of the neighbour operator transmitting DL, and the synchronized case where the victim is a TDD deployment so that the blockers are the Ues. It can be observed that both the curves are below the current in-band blocking level of -35 dBm for Local Area BSs. This means there is no need to modify the in-band blocking level for this BS class for SBFD networks.

Figure 2.2.3-2: InH: Received UL wideband power from DL of TDD network 2

- In InH deployments, with low-power BSs, the power level of the BSs (24 dBm) is comparable to the maximum power level of the Ues (23 dBm), so that the fact that the BSs are blockers for an SBFD deployment, does not introduce a need for more stringent requirements for this BS class.

- InH deployments are easier to coordinate, so that certain distance between BSs of different networks can be assumed, and this also facilitates that there is no need for more stringent requirements.

Re-use the existing in-band blocking requirement for Local Area BSs in SBFD networks.

# 3	Impact on BS RX requirements

Reference sensitivity level and OTA sensitivity

Regarding Reference sensitivity requirement for SBFD-capable BS, due to the self interference caused internally to receiver side, RAN4 reached the following consensus: - For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed. - Otherwise, OTA sensitivity requirement could be derived based on the following equation: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase. - The following aspects need more discussion during a WI phase - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

- For SBFD-capable BS type 1-H, the existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e., no sensitivity degradation is allowed.

- For SBFD-capable BS OTA sensitivity requirement, [0.5~1.0] dB degradation value needs FFS.

It has been agreed that an OTA sensitivity requirement, based on sensitivity in the RX sub-band with the transmitter turned on in the TX sub-bands is needed for SBFD. Some degradation compared to the sensitivity requirement in non-SBFD slots should be allowed for, where it is important to consider that all sources of interference, i.e., self-interference, inter-sector interference, inter-site interference and interference from other operators, will cause the sensitivity of the receiver to degrade. An open issue for discussion is the value of degradation. The limiting factor can be IM3 in receiver, especially for wide area deployment, the degradation of the receiver is due to IM3 can get many more dBs rapidly.

For example, if the gNB self-interference degradation is 1dB (1.25 in linear with the corresponding self-interference relative to noise being 0.25) and the network can be designed such that the sensitivity degradation from other sector and site interference (without self-interference) would also be 1dB (1.25 in linear with the corresponding other interference relative to noise being 0.25), then the total sensitivity degradation for a real network experiencing both self-interference and other-sector interference would be 1.8dB (1.5 in linear with the corresponding total interference relative to noise being 0.5) if the interferences would just be additive and uncorrelated. If the receiver is IM3 limited, considering the interference at the input to the LNA is doubled, the interference at the output increases by 2 ^ 3, i.e. 8 times, then the total interference becomes 8*0.25 = 2. Hence, the ratio of (interference + noise) / noise becomes (2+1)/1 = 3 in linear, or 4.8dB. These two numbers represent lower and upper boundaries, so the real total sensitivity degradation would be somewhere between 1.8 and 4.8dB for such a deployment.

If the self-interference degradation is 0.5dB and the sites can be designed such that the degradation from other sectors or sites without self-interference would also be 0.5dB then the desensitization in a real network with both sources present would be between 0.95dB and 3.2dB.

Thus, if the desensitization due to self-interference alone is 1dB then in a real deployment, even with highly isolated sites, it would not be possible to keep the desensitization less than 2 - 4dB.

- OTA sensitivity should be defined considering in real life the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

- RAN4 requirements should be conservative enough that the SBFD BS can be expected to perform well in real deployments considering inter-site interference and inter-sector interference.

- Due to the receiver performance is limited by receiver non-linearity, the sensitivity degradation is much greater than just the sum of the sensitivity degradation from each source alone.

Dynamic range

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase.

In normal uplink, the IoT depends on the expected interference from other UEs. While for SBFD, it actually depends on the interference from other base stations. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

- RX dynamic range requirement is applicable for SBFD-capable BS. IoT level and wanted signal power level need further discussion in the WI phase.

In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

In-band blocking requirements for blockers outside of the carrier are considered. Blocking requirements are designed for uplink operation where the blocking transmitters are UEs. In SBFD slots, the slots may be operating in DL, and hence the blocking interferers are other operators BSs. we need to consider other BSs, because the source of blocking is other BSs, not UEs.

- For SBFD, the RX blocking requirement is based on signal levels from the DL of other operators BS.

Other operators BSs may be co-located with the SBFD BS or may be at some distance. There is currently no requirement for supporting co-location of non-synchronized operators in the same band. The nearest requirement is the FDD protection of own receiver requirement. A co-location blocking requirement could be created for SBFD and declaration based; declaring compliance to the requirement would demonstrate sufficient RX performance to enable co-location. However, the requirement would not necessarily ensure that emissions from the co-located BS would not desensitize the SBFD receiver.

- The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

For non-co-located blocking, dedicated co-existence simulations were conducted to determine the expected blocker levels due to other operator site BS during SBFD slots. Results and discussion are presented in Section 2.

Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing OOBB requirement is still applicable for SBFD-capable BS except for OTA sensitivity degradation with the power level of wanted signal taken into account.

- The OOB blocking requirement is the same in SBFD slots as for normal TDD.

Receiver spurious emissions

Regarding the receiver spurious emission requirement, apart from existing requirements for normal reception on UL symbols/slots, it’s not necessary to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols/slots.

- The receiver spurious emissions requirement is the same for both SBFD and non-SBFD slots.

- The receiver spurious emission is only measurable with conducted testing and OTA testing with transmitter deactivated in SBFD slots.

Receiver intermodulation

Regarding the receiver intermodulation requirement, in general, RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement RAN4 reached the following consensus: - Conducted RX intermodulation: Adopt the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level. - OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

The RX intermodulation requirement for conventional TDD consists of a CW signal and a modulated signal that are located in frequency such that their intermodulation produce falls within the receiver frequency range. The RX intermodulation sets a minimum requirement on receiver linearity.

For SBFD, it is possible that a single signal can intermodulate in the receiver with the TX sub-band TX signal. It is necessary to supress such intermodulation sufficiently that the receiver is not desensitized.

The scenarios for 2 input signals and 1 input signal + TX sub-band may need different levels of receiver linearity to meet, depending on the level of the TX sub-band signal in the receiver. Thus, in our view, a requirement based on a single input signal causing IM with the TX sub-band could also be introduced to ensure additional robustness, or such a requirement may anyhow be covered by the RX blocking requirement for SBFD slots.

Investigate whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

In-channel selectivity

Regarding the receiver in-channel selectivity requirement, the requirement shall be studied based on that the wanted signal and UL interfering signal shall be located in the configured UL subband, and the wanted signal and interfering signal levels could be further studied in the WI phase.

- Receiver in-channel selectivity requirement is focused on UL sub-band, and the wanted signal and interfering signal levels is FFS in the WI phase.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

Observation 2	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 3	For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 4	In UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments may not need an update in requirements, but uncoordinated deployments may need it.

Observation 5	In InH deployments, with low-power BSs, the power level of the BSs (24 dBm) is comparable to the maximum power level of the Ues (23 dBm), so that the fact that the BSs are blockers for an SBFD deployment, does not introduce a need for more stringent requrements for this BS class.

Observation 6	InH deployments are easier to coordinate, so that certain distance between BSs of different networks can be assumed, and this also facilitates that there is no need for more stringent requirements.

Observation 7	For SBFD-capable BS type 1-H, the existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e., no sensitivity degradation is allowed.

Observation 8	For SBFD-capable BS OTA sensitivity requirement, [0.5~1.0] dB degradation value needs FFS.

Observation 9	OTA sensitivity should be defined considering in real life the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

Observation 10	RAN4 requirements should be conservative enough that the SBFD BS can be expected to perform well in real deployments considering inter-site interference and inter-sector interference.

Observation 11	Due to the receiver performance is limited by receiver non-linearity, the sensitivity degradation is much greater than just the sum of the sensitivity degradation from each source alone.

Observation 12	RX dynamic range requirement is applicable for SBFD-capable BS. IoT level and wanted signal power level need further discussion in the WI phase.

Observation 13	For SBFD, the RX blocking requirement is based on signal levels from the DL of other operators BS.

Observation 14	The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

Observation 15	The OOB blocking requirement is the same in SBFD slots as for normal TDD.

Observation 16	The receiver spurious emissions requirement is the same for both SBFD and non-SBFD slots.

Observation 17	The receiver spurious emission is only measurable with conducted testing and OTA testing with transmitter deactivated in SBFD slots.

Observation 18	Receiver in-channel selectivity requirement is focused on UL sub-band, and the wanted signal and interfering signal levels is FFS in the WI phase.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define in-band blocking interferer signal power to 5 dBm for Wide Area BSs in SBFD networks.

Proposal 2	Define in-band blocking interferer signal power to -1.5 dBm for Medium Range BSs in SBFD networks.

Proposal 3	Re-use the existing in-band blocking requirement for Local Area BSs in SBFD networks.

Proposal 4	Investigate whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2412915, Impact on SBFD BS RF TX requirements, Ericsson

R4-2412914, Potentially new SBFD BS RF requirements, Ericsson

R4-2412913, Discussion on SBFD general aspects, Ericsson

R4-2409889, Way Forward for [110bis][313] NR_duplex_evo, Moderator (Samsung)