3GPP TSG-RAN WG4 Meeting # 112	R4-2412917

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.20.2.1

Source: 		Qualcomm Incorporated

Title: 	On AIoT deployment scenarios and spectrum usage

Document for:	Approval

# Introduction

In RAN4#111, the agreements on deployment scenarios and spectrum usage were captured in [1]. In this paper, we provide further considerations on AIoT deployment scenarios and spectrum.

# 2. Discussion

## 2.1 Deployment scenarios

In RAN4#111, below agreements for D1T1 deployment scenarios are captured in [1].

Issue 2-1-1: deployment scenarios for D1T1 Agreement in RAN4#111: - Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT - Encourage companies to provide the simulation results for option 1-1 and 1-2 - FFS on co-site scenario (option 2-1 and 2-2)

Based on our simulation in [2], for option 1-1, i.e., legacy NR UE is deployed outdoor, the impact of interference from R2D DL (R2D in DL spectrum) to legacy NR UE DL is marginal due to penetration loss. While for option 1-2, i.e., legacy NR UE is indoor accessing to outdoor NR Marco gNB, the interference level from R2D DL to legacy NR UE DL is much higher due to many factors such as high transmission power from micro BS, close distance between reader and legacy NR UE, relatively low ACIR, etc. The performance degradation would be unacceptable even with 1 reader and it might also lead to legacy NR UE blocking. Note that in this scenario, ACIR is limited by legacy UE ACS, i.e., 33dB, and ACS is using a flat model in the simulation which means that an additional guard band doesn’t help with this coexistence issue. Therefore, it is necessary to study the interference migration solution for option 1-2 in D1T1 scenario.

Figure 1: Illustration of interference from Micro BS/reader to NR UE DL in Topo 1

For example, since R2D link is configured by micro BS, it is feasible for Micro BS to dynamically configure the transmission of R2D to avoid or reduce the interference from Micro BS to NR UE. With a proper interference coordination solution, the coexistence issue between legacy NR UE and micro BS/reader that are close to each other can be handled.

For D2T2, as shown in Figure 2, a similar coexistence issue can be observed between R2D/D2R and legacy NR UE UL (R2D/D2R in UL). While in D2T2, intermediate UE/reader access to outdoor macro BS. Therefore, the transmission of R2D/D2R can be dynamically configured by macro gNB via intermediate UE. For example, macro BS might indicate intermediate UE to NOT transmit D2R when there is a proximate legacy UE to avoid interference from legacy UE UL to AIoT D2R.

Figure 2: Illustration of interference from intermediate UE/reader to NR UE UL in Topo 2

Observation 1: For D1T1 and D2T2, when legacy NR UE is indoor, it is necessary to study the interference migration solution to solve the coexistence issue between AIoT system and legacy NR UE.

Proposal 1: Further study the interference mitigation for the cases legacy NR UE indoor in D1T1 and D2T2.

  - For Option 1-2 in D1T1, micro gNB configures R2D/D2R resource to avoid or reduce the interference from R2D to proximity NR UE DL.

  - For Option 1-2 in D2T2, macro gNB configures R2D/D2R resource via intermediate UE to avoid or reduce the interference from NR UE UL to D2R.

## 2.2 Spectrum usage

For D1R1, the agreements in RAN4#111 for spectrum usage in D1T1 are listed below.

Issue 2-2-1: Spectrum usage for R2D in D1T1 Agreement in RAN4#111: - Use FDD DL as starting point for co-existence evaluation for R2D in D1T1 - FFS on FDD UL spectrum.

For D1T1 option 1-2, as discussed in section 2.1, the main coexistence issue is the interference from R2D to NR UE DL (R2D in DL spectrum). In Proposal 1, we suggest considering interference mitigation solution. The coexistence issue can be solved by dynamically configuring resources or ON/OFF for R2D link. Alternatively, switching the R2D transmission from DL spectrum to UL spectrum will also help to reduce the impact of interference. That is because when R2D is in UL spectrum, the victim is macro gNB outdoor. The interference power will become very low after passing through the wall.

Proposal 2: RAN4 can consider using FDD UL spectrum for R2D in D1T1 to solve the coexistence issue.

# 3.	Conclusion

In this paper, we provide our views on AIoT deployment scenarios and spectrum usage. The following observations and proposals are made:

Observation 1: For D1T1 and D2T2, when legacy NR UE is indoor, it is necessary to study the interference migration solution to solve the coexistence issue between AIoT system and legacy NR UE.

Proposal 1: Further study the interference mitigation for the cases legacy NR UE indoor in D1T1 and D2T2.

  - For Option 1-2 in D1T1, micro gNB configures R2D/D2R resource to avoid or reduce the interference from R2D to proximity NR UE DL.

  - For Option 1-2 in D2T2, macro gNB configures R2D/D2R resource via intermediate UE to avoid or reduce the interference from NR UE UL to D2R.

Proposal 2: RAN4 can consider using FDD UL spectrum for R2D in D1T1 to solve the coexistence issue.

# 4.	References

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC

- R4-2412918, Discussion on Ambient IoT Coexistence Evaluation in NR, Qualcomm

- TR 38.863