3GPP TSG-RAN WG4 Meeting # 112						R4-2412918

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.20.2.2

Source:	Qualcomm Incorporated

Title:	On Ambient IoT Coexistence Evaluation

Document for:	Approval

# 1  Introduction

In RAN4 #111 meeting, the agreements on ambient IoT coexistence study were captured in [1]. 20 cases are agreed for the coexistence study. The evaluation parameters and topology were also provided.

This paper provides our considerations on the issues that identified during the coexistence evaluation and our proposals to Ambient IoT coexistence study.

# 2  Discussion for co-existence evaluation

## 2.1 D1T1 and D2T2 Layouts

Agreements

For D1T1-A2: 18 BSs on a square lattice with spacing D, located D/2 from the walls.

- L=120m x W=60m; D=20m

- BS height = 8 m

For D2T2-A2:

The intermediate UEs selected from the fixed positions.

Number of intermediate UE for calibration:

- Option 1: 2 UE at one drop

- Option 2: 12 UE at one drop

The agreements on the BS/intermediate UE deployments and layouts were listed as follows.

The layout of D1T1 is illustrated as in Figure 1.

Figure 1. D1T1 layout

Through system level simulation, the intra-system interference is very high under current layout and there is some misalignment for the system performance between RAN1 link budget and RAN4 system simulation SINR. A reasonable system performance is essential for the coexistence evaluation as the intra-system interference will also influence coexistence study. For example, if the intra-system interference is high, then the system degradation due to the external system interference will be smaller, and vice versa.

For D1T1, 6 different indoor BSs/readers are selected for the system performance comparison, as shown in Figure 2. The blue point means the locations of BS/readers. The red point means the active BS/readers at the same time, and the other BSs will be active in turn (TDM based) to realize the whole coverage of the factory. The number 18, 9, 6, 3,2 and 1 means the number of the active BS at the same time. The active BS number can also be considered as the active ratio, 18BS corresponds to 100%, 9BS corresponds to 50%, 6BS corresponds to 33.3%, 3BS corresponds to 16.7%, 2 BS corresponds to 11.1% , and 1BS corresponds to 5.6%.

Figure 2. Illustration of different active BS numbers simultaneously

The system SINR performance of the six layouts are evaluated and compared as shown in Figure 3. When the number of active BS decreases, the system SINR performance improves due to less intra-system interference. For the 1BS case, the SNR is higher than 60dB due to no co-channel interference and the low noise floor. While at the same time, the time needed to cover the whole factory increases, and additional implementation efforts are needed to keep synchronization in BSs.  Since the BS in D1T1 is fixed and not mobile, the hardware resource is not fully utilized if the BS works in low active rate. Therefore, there is a trade-off between system performance and the implementation cost/efforts.

Figure 3. R2D SINR Comparison in DL spectrum in D1T1

Similar performance can be observed for D2T2, comparing 12 readers and 2 readers, as shown in Figure 4. It needs to be noticed that the 2 readers layout in D2T2 is different with the 2 indoor BSs/readers layout in D1T1. The 2 readers layout in D2T2 is not fixed, the positions of 2 intermediate UEs are dynamic and can be changed for each snapshot considering that the UE can move.  On the contrary, the 2BS in D1T1 is relatively fixed to keep the same ISD between two BSs. Therefore, even though there are only two readers in D2T2, the hardware resource utilization is 100% to cover the whole topology.

Figure 4. R2D SINR Comparison in UL spectrum in D2T2

Observation 1: The intra-system interference is very high for the 18 BS layout of D1T1 and the system SINR cannot meet the R2D/D2R demodulation requirement discussed in RAN1.  Decreasing the active BS number can improve the system SINR performance. While as the cost, the system latency, hardware idle time and the implementation efforts will be increased.

Proposal 1:  A new parameter, BS active rate, is proposed to be considered in the D1T1 coexistence study to represent the different layout options to compare system performance and resource utilization rate. For example, 18 BS layout represents the 100% active rate, and 9 BS layout represents the 50% active rate.

Observation 2: Similarly, in D2T2, the intra-system interference is high for 12-Reader layout and the 2-Reader layout can improve the system performance. What’s more, given the UE can be mobile, 2-reader layout could also have complete coverage.

Proposal 2: 1 or 2 UE at each drop should be used for the D2T2 coexistence study.

An example on the impact of different active BS numbers to the coexistence evaluation is shown in Figure 5. The Figure 4 presents the coexistence results of NR DL to the Ambient IoT R2D link. Due the no selectivity of ambient IoT device, the ACIR value is 0dB. Quite different performances can be observed with the same inter-system interference from NR DL. For the cases 18BS, 9BS, 6BS, 3BS, and 2BS, the NR DL interference’s impact is not obvious. While for the case 1BS, the NR DL brings in about 30dB SINR degradation at 50 percentile point.

Figure 5. Coexistence Performance of NR DL interfering to the Ambient IoT R2D link

Observation 3: The coexistence results of different active BS numbers vary significantly due to the different levels of intra-system interference.  When the active BS number decreases, the intra-system interference level drops, and the impact of inter-system interference rises.

Proposal 3: The impact of active BS/UEs ratio need to be considered in the coexistence criteria discussion and selection.

In addition to affecting the system SINR, the reader’s coverage will also be impacted, directly reflecting the demodulation performance. The performance mapping between SINR and the 2D distance between the reader and device is illustrated in Figure 6.

Figure 6. Mapping between R2D SINR and 2D Distance of Reader and Device in D1T1

(the 1BS, w/ACI case is the NR DL interfering to the R2D link)

Given an ISD of 20 meters for D1T1 18BS layout, the maximum allowable 2D distance between the reader and device is approximately 10 meters. Considering the shadowing factor, there might be slight variations. The 2D distance is used here for statistical purposes to reflect reader coverage.  With a 10-meter distance, different SINR performance can be observed for different active BS ratios. For the 18BS layout, the SINR falls below 0 dB, making the demodulation for the R2D link very challenging. Consequently, a large portion of devices will experience outage under this topology.

Typically, the D2R link becomes the bottleneck in deployment due to the physics of backscattering and higher required SINR. The mapping between D2R SINR and 2D Distance of Reader and Device in D1T1 is illustrated in Figure 7.  The ACI case means the NR DL’s interference to the D2R link.

Figure 7. Mapping between D2R SINR and 2D Distance of Reader and Device in D1T1

(the 1BS, w/ACI case is the NR DL interfering to the D2R link)

Observation 4: In addition to affecting the system SINR, the reader coverage will also be impacted, directly reflecting the demodulation performance. Given an ISD of 20 meters for the D1T1 18BS layout and the demodulation SINR requirement (e.g., over 10dB) for ambient IoT devices, some devices will experience outages under different active BS ratios, rendering the system unworkable. Consequently, certain layouts are not feasible for real-world implementation.

Proposal 4:  The required minimum SINR values for the R2D and D2R links are needed. These minimum SINR values can be used to determine the maximum active reader ratio. RAN4 should send an LS to RAN1 check the required SINR for R2D and D2R links.

Observation 5: When the number of active BS decreases, the ISD of BS increases. A receiver sensitivity of -35 dBm is used to determine if the device is activated. However, for the D2R link, no performance criteria are defined to determine if the reader can receive the device’s backscattering signal.

Proposal 5:  The reader receiver sensitivity is essential for a reasonable D2R performance evaluation. This sensitivity is related to the reader’s CW cancellation capability. RAN1 and RAN4 should study the CW cancellation capability and provide the values for coexistence study.

## 2.2 UE Deployment

Agreements

For scenario option 1-1, uniformly distributed outdoor.

For scenario option 1-2, uniformly distributed, 80% indoor, 20% outdoor

For scenario option 2-1, uniformly distributed outdoor.

For scenario option 2-2, uniformly distributed, 80% indoor, 20% outdoor

For each topology, two options will be studied to cover the legacy UE in outdoor and indoor case. The agreements on the UE deployments were listed as follows.

Penetration loss significantly affects the coexistence performance.  For instance, D1T1 R2D interference can impact NR DL UE reception. The legacy UE distributed 100% outdoor and 100% indoor are evaluated and the coexistence performance are illustrated in Figure 8 and Figure 9 respectively.

Figure 8. D1T1 18 BS R2D interfering to NR DL (UE 100% outdoor)

Figure 9. D1T1 R2D interfering to NR DL (UE 100% indoor)

( D1T1 with different active BS number )

Comparing the Figure 8 and Figure 9, the impact of R2D interference to the outdoor UE is marginal. The impact of R2D interference to the indoor UE depends on the active BS number and is very severe.

Setting 100% indoor UE allows for studying the pure impact on indoor UE performance and exploring mitigation methods for severe interference. Indoor coverage is usually good for frequencies below 1GHz. A 20% outdoor and 80% indoor UE setting reduces overall interference and doesn’t accurately reflect the impact when legacy UE moves indoors. If evaluations show that coexistence with indoor UE is unfeasible under current frequency planning, further mitigation methods can be investigated.

Observation 6:  Penetration loss causes significant differences in the performance of legacy UE between outdoor and indoor environments. A 20% outdoor and 80% indoor UE setting reduces interference but doesn’t accurately reflect the impact when legacy UE moves indoors. If evaluations show that coexistence with indoor UE is unfeasible under current frequency planning, mitigation methods can be further explored. Setting 100% UE indoors helps identify potential interference impacts.

Proposal 6: For option 2, the indoor UE ratio is proposed to be 100%, instead of 80%.

- For scenario option 1-1, uniformly distributed outdoor.

- For scenario option 1-2, uniformly distributed, 100% indoor.

- For scenario option 2-1, uniformly distributed outdoor.

- For scenario option 2-2, uniformly distributed, 100% indoor.

## 2.3 Guardband and UE Blocking

As agreed in [1], 20 cases will be studied for ambient IoT coexistence. The ACIR values vary with different bandwidth settings and device types. In some cases, ACIR is limited by ACLR, while in others, it is limited by ACS.  For ACLR-limited cases, if coexistence results show severe interference, a guard band can help mitigate interference. For ACS-limited cases, other mitigation methods like frequency coordination and isolation distance need to be investigated, as ACS modeling typically assumes to be flat. Additionally, when ACS becomes the bottleneck for coexistence, the blocking issue also needs evaluation.

The scenario of R2D interfering with NR DL UE reception on DL spectrum can serve as an example of an ACS-limited case. When the aggressor with smaller bandwidth and therefore ACLR keep the same value with symmetrical bandwidth. The reader ACLR is 45dB, and the UE ACS is 33dB.  Setting the UE 100% indoor located, Figure 10 illustrated the UE received power distribution, the intra-system interference distribution and the inter-system interference distribution from different active BS number.

Figure 10. the distribution of signal power and interference power for the R2D interfering to the NR DL (100% indoor)

As shown in Figure 10, the interference from reader to NR DL would be very high which might result in the UE being blocked with a certain probability.

Observation 7: In some cases, the interference power from reader to NR DL would be very high, potentially causing the UE to be blocked.

Proposal 7: The UE blocking issue and corresponding maximum interference power need further study in RAN4.

# 3  Conclusion

In summary, we have following observations and proposals.

Observation 1: The intra-system interference is very high for the 18 BS layout of D1T1 and the system SINR cannot meet the R2D/D2R demodulation requirement discussed in RAN1.  Decreasing the active BS number can improve the system SINR performance. While as the cost, the system latency, hardware idle time and the implementation efforts will be increased.

Proposal 1:  A new parameter, BS active rate, is proposed to be considered in the D1T1 coexistence study to represent the different layout options to compare system performance and resource utilization rate. For example, 18 BS layout represents the 100% active rate, and 9 BS layout represents the 50% active rate.

Observation 2: Similarly, in D2T2, the intra-system interference is high for 12-Reader layout and the 2-Reader layout can improve the system performance. What’s more, given the UE can be mobile, 2-reader layout could also have complete coverage.

Proposal 2: 1 or 2 UE at each drop should be used for the D2T2 coexistence study.

Observation 3: The coexistence results of different active BS numbers vary significantly due to the different levels of intra-system interference.  When the active BS number decreases, the intra-system interference level drops, and the impact of inter-system interference rises.

Proposal 3: The impact of active BS/UEs ratio need to be considered in the coexistence criteria discussion and selection.

Observation 4: In addition to affecting the system SINR, the reader coverage will also be impacted, directly reflecting the demodulation performance. Given an ISD of 20 meters for the D1T1 18BS layout and the demodulation SINR requirement (e.g., over 10dB) for ambient IoT devices, some devices will experience outages under different active BS ratios, rendering the system unworkable. Consequently, certain layouts are not feasible for real-world implementation.

Proposal 4:  The required minimum SINR values for the R2D and D2R links are needed. These minimum SINR values can be used to determine the maximum active reader ratio. RAN4 should send an LS to RAN1 check the required SINR for R2D and D2R links.

Observation 5: When the number of active BS decreases, the ISD of BS increases. A receiver sensitivity of -35 dBm is used to determine if the device is activated. However, for the D2R link, no performance criteria are defined to determine if the reader can receive the device’s backscattering signal.

Proposal 5:  The reader receiver sensitivity is essential for a reasonable D2R performance evaluation. This sensitivity is related to the reader’s CW cancellation capability. RAN1 and RAN4 should study the CW cancellation capability and provide the values for coexistence study.

Observation 6:  Penetration loss causes significant differences in the performance of legacy UE between outdoor and indoor environments. A 20% outdoor and 80% indoor UE setting reduces interference but doesn’t accurately reflect the impact when legacy UE moves indoors. If evaluations show that coexistence with indoor UE is unfeasible under current frequency planning, mitigation methods can be further explored. Setting 100% UE indoors helps identify potential interference impacts.

Proposal 6: For option 2, the indoor UE ratio is proposed to be 100%, instead of 80%.

- For scenario option 1-1, uniformly distributed outdoor.

- For scenario option 1-2, uniformly distributed, 100% indoor.

- For scenario option 2-1, uniformly distributed outdoor.

- For scenario option 2-2, uniformly distributed, 100% indoor.

Observation 7: In some cases, the interference power from reader to NR DL would be very high, potentially causing the UE to be blocked.

Proposal 7: The UE blocking issue and corresponding maximum interference power need further study in RAN4.

# 4  References

[1] R4-2410567, WF on co-existence evaluation for ambient IoT

[2] RP-240826, Revised SID, Study on solutions for Ambient IoT (Internet of Things) in NR

[3] TR 38.848 Study on Ambient IoT (Internet of Things) in RAN