3GPP TSG-RAN WG4 Meeting # 112	R4-2412919

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.13.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR2 OTA test method on STxMP

Document for:	Approval

# Introduction

In RAN4#111, the WF on NR FR2 OTA in [1] was approved. In this paper, we provide our views on FR2 OTA test method on transmission with multi-panel (STxMP) testing.

# 2. Discussion

## 2.1 General

The signal level condition for distinguishing EIRP per TCI was discussed in last meeting and the following agreements were approved.

Issue 1-1-1: Signal level condition for measuring/distinguishing EIRP per TCI - Agreement: - RAN4 assume TE has the capability of distinguishing EIRP per TCI with the configuration of rank 2 PUSCH transmission in EIRP measurement of STxMP. under SDM scheme to further study the test methodology - Signal level conditions (e.g., SNR, power imbalance) and test procedure need to be further checked. - RAN4 to conclude the feasibility issue in next meeting.

In legacy UL SISO EIRP measurement, it is possible that the power level from two polarizations are quite different. But there is no specific signal level condition for power and SNR imbalance.

In multi-Tx UL measurement, as rank 2 PUSCH transmission under SDM scheme is configured (i.e., one layer per TCI), TE’s receive will jointly decode EIRP1 and EIRP2 by 2 x 4 matrix (Tx x Rx). Since DMRS resources are orthogonal for TCI1 and TCI2, TE should be able to decode and measure EIRP per TCI even with power imbalance. Note that the intention of this measurement is peak EIRP value. Therefore, it is not possible to for the power of two TCIs to differ greatly.

Observation 1: For multi-Tx UL measurement, since rank 2 PUSCH transmission under SDM scheme (i.e., one layer per TCI) is configured and DMRS resources are orthogonal for TCI1 and TCI2, TE should be able to decode and measure EIRP per TCI.

Proposal 1: RAN4 concludes not to specify signal level conditions and assumes EIRP can be distinguished per TCI in STxMP testing.

## 2.2 EIRP PUMAX,f,c,k testing

With the assumption that TE has the capability of distinguishing EIRP per TCI, the remaining testability issues for peak EIRP PUMAX,f,c,k testing are listed as follows:

- Issue#1: How to select the proper AoA pair and UE orientation

- Issue#2: Measurement grid

For issue#1, the below agreements were made on AoA separation and UE orientation for EIRP PUMAX,f,c,k testing.

Issue 1-4-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreements: - FFS on AoA separation and UE orientation for EIRP PUMAX,f,c,k testing considering the following options - Option 1: Consider similar as Multi-Rx UE declaration approach for one AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and one proper UE orientation listed in TS38.101-2 for EIRP per TCI measurement - How to process the measurement data is FFS - Data processing should avoid repeated selection of peak EIRP per TCI. - Option 2: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2), another AoA is decided by UE declared orientation (listed in TS38.101-2) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

For AoA separation and UE orientation, option 1 is similar to the UE declaration approach in multi-Rx testing. While the potential issue is how to process the measurement data to avoid repeated selection of peak EIRP per TCI. Below figure provides the illustration assuming Dq=Df=30°, and AoA separation is 60°. EIRP results for TCI1 are in Set 1: {EIRP_AoA1 of pair 1, EIRP_AoA1 of pair 2, EIRP_AoA1 of pair 3…}, EIRP results for TCI2 are in Set 2: {EIRP_AoA2 of pair 1, EIRP_AoA2 of pair 2, EIRP_AoA2 of pair 3…}.

In addition, as shown in Figure 2, the mapping between AoA/probe and module index is changing with different test points. Therefore, if the peak EIRP value of Set 1 and Set 2 are selected, then we will get two identical peak EIRP values.

Figure 1: Illustration of EIRP measurement for Option 1

Figure 2: Illustration of repeated selection of peak EIRP per TCI

Hence for Option 1, it is hard to avoid repeated selection of peak EIRP per TCI unless the mapping between AoA/probe and module index is fixed.

The rationale of Option 2 is that since the beam peak direction for each of the indicated TCI targets to be selected, therefore one of the two peak directions should be the same as the beam peak direction with legacy panel carrier operation. The other AoA can be selected based on maximizing the EIRP from all the test points. But for option 2, the challenge is how to guarantee one of AoA is fixed at beam peak direction with legacy single panel operation when rotating UE. And for option 2, per TCI peak direction might not transmit simultaneously in reality.

Proposal 2: RAN4 further study the feasibility of Option 2 assuming that test system defined in TR 38871 is used.

For both Option 1 and Option 2, since the peak EIRP is measured per TCI, legacy PC1 and PC5 measurement grid for beam peak search defined in Annex M.2 of TS38.521-2 should be able to be reused.

Proposal 3: Legacy PC1 and PC5 constant step size measurement grid for beam peak search defined in Annex M.2 of TS38.521-2 can be reused for Option 1 and Option 2.

If option 1 and option 2 have testability issues and could not be solved, alternatively, we can consider the peak directions are declared by UE without beam peak search procedure.

Proposal 4: RAN4 to consider UE declaration on the peak directions, i.e., no beam peak search procedure.

## 2.2 Aggregated EIRP PUMAX,f,c testing

In [1], the skipping rule for EIRPmax is agreed as follows.

Issue 1-5-1: Skipping rule for EIRPmax testing - Agreements: - RAN4 agree on the skipping rule proposal for EIRPmax, i.e., “If the peak EIRP of single CC operation without STxMP (from Peak EIRP for MOP in 6.2 of TS 38.101-2) is smaller than EIRPmax - 3dB –additional margin , the corresponding EIRPmax verification can be skipped”. - Further check on the handling of the test case skip as part of conformance test. FFS on additional margin, such as systematic error of beam peak search, etc. - The slightly change of antenna radiation pattern of one panel when the other panel is activated if any, which might varry the test results of EIRPmax, can be compensated by addition margin.

In case, the DUT cannot pass the skipping rule, i.e., peak EIRP of single CC operation is larger than EIRPmax - 3dB - additional margin. The EIRPmax per TCI should be tested. Due to the TE’s restriction and testing time, it is not possible to verify all the possible AoA pairs. Therefore, it is proposed to reuse the beam peak directions selected in EIRP PUMAX,f,c,k testing if option 2 is adopted.

Issue 1-5-2: AoA separation and UE orientation EIRPmax testing - Agreements: - RAN4 to further study the AoA separation and UE orientation EIRPmax testing considering the following options: - Option 1: RAN4 to consider AoA offset = [30°] as the worst case. FFS on the UE orientation selection. - Option 2: For PUMAX,f,c (aggregated EIRP), re-use the UE declared AoA separation and orientation of Multi-RX. FFS on the details of data processing. - Option 3: Define a few AoA separations/offsets in the OTA test for EIRPmax verification. Reuse the orientation definitions in current standards.

Proposal 5: If the DUT cannot pass the skipping rule, i.e., peak EIRP of single CC operation is larger than EIRPmax - 3dB - additional margin, the beam peak directions selected in EIRP PUMAX,f,c,k testing is reused in EIRPmax testing if Option 2 is adopted. FFS on Option 1 and Option 3.

## 2.2 Aggregated TRP PTMAX,f,c

For Aggregated TRP PTMAX,f,c, testing, the issue is similar to aggregated EIRP PUMAX,f,c testing. A similar skipping rule is agreed in [1].

Issue 1-6-1: Skipping rule for TRPmax testing - Agreements: - RAN4 agree on the skipping rule proposal for TRPmax, i.e., “If the peak TRP of single CC operation without STxMP (from TRP for MOP in 6.2 of TS 38.101-2) is smaller than TRPmax - 3dB – additional margin[], the corresponding TRPmax verification can be skipped”. - Further check on the handling of the test case skip as part of conformance test. FFS on additional margin, such as systematic error of beam peak search, etc. - The slightly change of antenna radiation pattern of one panel when the other panel is activated if any, which might vary the test results of TRPmax, can be compensated by addition margin.

Similarly, if the DUT cannot use the skipping rule, i.e., peak TRP of single CC operation is larger than TRPmax - 3dB - additional margin, the beam peak directions selected in EIRP PUMAX,f,c,k testing is reused if Option 2 is adopted. FFS on other options.

Proposal 6: If the DUT cannot use the skipping rule, i.e., peak TRP of single CC operation is larger than TRPmax - 3dB - additional margin, the beam peak directions selected in EIRP PUMAX,f,c,k testing is reused if Option 2 is adopted. FFS on Option 1 and Option 3.

The legacy PC1 and PC5 constant-step measurement grid for TRP defined in Annex M.4 of TS38.521-2 can be used in TRPmax testing if Option 2 is adopted. FFS on Option 1 and Option 3.

Proposal 7: The legacy PC1 and PC5 constant step size measurement grid for TRP defined in Annex M.4 of TS38.521-2 can be used in TRPmax testing.

# 3.	Conclusion

In this paper, we provide our views on test methodology for STxMP and have the following observations and proposals:

Observation 1: For multi-Tx UL measurement, since rank 2 PUSCH transmission under SDM scheme (i.e., one layer per TCI) is configured and DMRS resources are orthogonal for TCI1 and TCI2, TE should be able to decode and measure EIRP per TCI.

Proposal 1: RAN4 concludes not to specify signal level conditions and assumes EIRP can be distinguished per TCI in STxMP testing.

Proposal 2: RAN4 further study the feasibility of Option 2 assuming that test system defined in TR 38871 is used.

Proposal 3: Legacy PC1 and PC5 constant step size measurement grid for beam peak search defined in Annex M.2 of TS38.521-2 can be reused for Option 1 and Option 2.

Proposal 4: RAN4 to consider UE declaration on the peak directions, i.e., no beam peak search procedure.

Proposal 5: If the DUT cannot pass the skipping rule, i.e., peak EIRP of single CC operation is larger than EIRPmax - 3dB - additional margin, the beam peak directions selected in EIRP PUMAX,f,c,k testing is reused in EIRPmax testing if Option 2 is adopted. FFS on Option 1 and Option 3.

Proposal 6: If the DUT cannot use the skipping rule, i.e., peak TRP of single CC operation is larger than TRPmax - 3dB - additional margin, the beam peak directions selected in EIRP PUMAX,f,c,k testing is reused if Option 2 is adopted. FFS on Option 1 and Option 3.

Proposal 7: The legacy PC1 and PC5 constant step size measurement grid for TRP defined in Annex M.4 of TS38.521-2 can be used in TRPmax testing.

# 4.	References

- R4-2409921, Way Forward for [111][339] NR_FR2_OTA, Qualcomm

- R4-2409428, On FR2 OTA test method on STxMP, Qualcomm

- R4-2404672, Discussion on the sTxMP OTA test, vivo

- TR 38.871

- TS 38.521-2