3GPP TSG-RAN WG4 Meeting # 112	R4-2412920

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.12.2.2

Source: 		Qualcomm Incorporated

Title: 	On FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN#103, the WID on TRP, TRS and MIMO OTA testing enhancement Phase 3 was approved in [1] where the following objectives of test method for FR1 NTN devices.

- Study and define test methodology and radiated performance metric for FR1 NTN devices (including NR NTN and IoT NTN) - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop preliminary Measurement Uncertainty (MU) (RAN5)

In RAN4#111, WF for [111][338] TRP_TRS_MIMO_OTA was approved in [2] capturing agreements and open issues for FR1 NTN OTA testing.  In this paper, we provide our views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

## 2.1 UE type and usage scenarios

For NR-NTN devices, Handheld UE with PC3 is prioritized. For IoT-NTN, as agreed in [1], companies are encouraged to provide more device information for IoT-NTN device types, e.g., detailed definition, device size, weight, typical antenna patterns. We analyze the usage scenarios of different UE types as below.

NR NTN handheld UE has two main usage scenarios.

- Talk mode

  - Talk mode with Head+Hand

  - Talk mode with Hand only (via speaker)

- Browsing mode

  - Browsing mode with Hand only

IoT NTN devices could be used for many scenarios including industry, shipping ports, airports, utility monitoring, etc. The usage scenarios for IoT NTN  (non-handheld UE) are mainly Free Space. Note that voice is not supported by IoT NTN, therefore for IoT NTN handheld UE, browsing mode with Hand only should be prioritized.

Proposal 1: RAN4 consider the following usage scenarios for NR NTN and IoT NTN devices;

- For NR NTN handheld UE

  - Talk mode

    - Talk mode with Head+Hand

    - Talk mode with Hand only (via speaker)

  - Browsing mode

    - Browsing mode with Hand only

- For IoT NTN handheld UE: Browsing mode with Hand only.

- For IoT NTN non-handheld UE (for vertical): Free Space.

## 2.2 UE performance metric

In [2], the issues of performance metric for NR-NTN handheld UE and whether performance metric should be different for GEO and NGEO (including LEO and MEO) were discussed.

Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

For GSO scenario, the relative position between UE and satellite is almost static. Therefore, UE antenna radiated performance is the main target to be verified in OTA testing. In addition, omni-radiated performance such as TRP and TRS are not suitable for NTN device OTA testing any more since usually device needs to point to the satellite with the help of software before starting the communication with satellite. Therefore, D3a, i.e., Peak EIRP/EIS+ partial TRP/TRS should be considered as the starting point for the NR-NTN handheld UE in GSO scenario. FFS on the range of angles.

Proposal 2: For GSO scenario, D3a, i.e., Peak EIRP/EIS+ partial TRP/TRS should be considered as the starting point for the NR-NTN handheld UE. FFS on the range of angles.

While for NGSO scenario, the relative position between UE and satellite is changing. Hence, both UE antenna radiated performance for upper hemisphere and spherical coverage performance should be verified in this scenario. Therefore, CDF-like performance metric, i.e., D3b, Peak EIRP/EIS+ partial Spherical coverage should be considered as the starting point.

Proposal 3: For NGSO scenario, D3b, Peak EIRP/EIS+ partial Spherical coverage should be considered as the starting point. FFS on the range of angles.

## 2.3 Other aspects

For NTN devices, antenna design could be based on circular polarization or cross-polarization. Therefore, in NTN OTA testing, the test method should not limit UE implementation on pol., and whether the antenna is shared or separate from TN antenna. From TE PoV, the test setup needs to support different UE implementations on the antenna design.

Proposal 4: RAN4 to confirm the TE can accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following proposals were made:

Proposal 1: RAN4 consider the following usage scenarios for NR NTN and IoT NTN devices;

- For NR NTN handheld UE

  - Talk mode

    - Talk mode with Head+Hand

    - Talk mode with Hand only (via speaker)

  - Browsing mode

    - Browsing mode with Hand only

- For IoT NTN handheld UE: Browsing mode with Hand only.

- For IoT NTN non-handheld UE (for vertical): Free Space.

Proposal 2: For GSO scenario, D3a, i.e., Peak EIRP/EIS+ partial TRP/TRS should be considered as the starting point for the NR-NTN handheld UE. FFS on the range of angles.

Proposal 3: For NGSO scenario, D3b, Peak EIRP/EIS+ partial Spherical coverage should be considered as the starting point. FFS on the range of angles.

Proposal 4: RAN4 to confirm the TE can accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

# 4.	References

- RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo

- TR 38.863