3GPP TSG-RAN WG4 Meeting # 112	R4-2412921

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.12.2.3

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR1 dynamic MIMO OTA test methodology

Document for:	Approval

# Introduction

In RAN4#111, FR1 dynamic MIMO OTA test method were discussed and the WF was agreed in [2]. In this paper, we provide our further views on FR1 dynamic MIMO OTA test methodology.

# 2. Discussion

To develop a dynamic channel model that accurately reflects real-world conditions, we should initially investigate the approach commonly employed by operators for evaluating their networks using actual devices. This approach, often referred to as ‘drive testing,’ involves driving along a predefined route with one or more reference devices to assess factors such as throughput performance and dropped calls. Before OTA testing for transmitter and receiver performance metrics including TRP/TRS and MIMO OTA, drive testing was the standard practice to assess new UE performance before deployment on the network. Dynamic OTA testing is to facilitate the realistic test in the chamber which has better repeatability and could save the test cost.

To create the dynamic channel parameters, large scale channel characteristics, such as angular power distribution, power delay, Doppler shift, and LOS/NLOS condition, are varying over the emulation time. The path loss profile, UE speed, and UE orientation can also be defined to be dynamic.

It is noted that CTIA already has studied the test plan for FR1 dynamic MIMO OTA testing which is based on SIR test condition. The outcome from CTIA could be the starting point for 3GPP dynamic MIMO OTA discussion. Referring to CITA status, the following aspects should be considered in RAN4 dynamic MIMO OTA discussion.

Proposal 1: RAN4 to consider the following aspects regarding the dynamic OTA modelling:

- How to create a driving route, DoT (Direction of Travel), velocity, AoA, etc, parameters

- How to define the midway points on the drive route

- How to interpolate channel parameters for continuous channel modelling

- Whether/how to change UE orientation, such as via switching the probes and/or UE positioner

- Other aspects are not precluded.

To avoid the standard fragmentation, it is better to algin test methodology with CTIA.

Proposal 2: RAN4 should try to align FR1 dynamic OTA test methodology with CTIA. Therefore, RAN4 should consider using the interference-limited environment condition in RAN4 FR1 Dynamic MIMO OTA test method.

In RAN4#111, it was agreed that RAN4 adopt dynamic link adaptation. FFS details, e.g., mapping tables between CQI and MCS for available rank values. Link adaptation is implemented by BS emulator/gNB to verify DUT’s performance in real-world conditions. The mapping between CQI and MCS and PMI codebook will have an influence in the measurement results. To make the testing consistent in different test systems and make the test condition more realistic, the CQI and MCS mapping specified in section 5.2.2.1 of TS 38.124 should be used. In addition, the PMI will also have impact on the DUT’s performance under link adaptation for the PMI-feedback based MIMO precoding testing. Therefore, PMI index specified in section 5.2.2.2 of TS 38.124 should be used here.

Proposal 3: The mapping between CQI and MCS specified in section 5.2.2.1 of TS 38.124 and PMI codebook specified in section 5.2.2.2 of TS 38.124 should be used for link adaptation in FR1 dynamic OTA testing.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 dynamic MIMO OTA testing. The following proposals were made:

Proposal 1: RAN4 to consider the following aspects regarding the dynamic OTA modelling:

- How to create a driving route, DoT (Direction of Travel), velocity, AoA, etc, parameters

- How to define the midway points on the drive route

- How to interpolate channel parameters for continuous channel modelling

- Whether/how to change UE orientation, such as via switching the probes and/or UE positioner

- Other aspects are not precluded

Proposal 2: RAN4 should try to align FR1 dynamic OTA test methodology with CTIA. Therefore, RAN4 should consider using the interference-limited environment condition in RAN4 FR1 Dynamic MIMO OTA test method.

Proposal 3: The mapping between CQI and MCS specified in section 5.2.2.1 of TS 38.124 and PMI codebook specified in section 5.2.2.2 of TS 38.124 should be used for link adaptation in FR1 dynamic OTA testing.

# 4.	References

- RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo