3GPP TSG-RAN WG4 Meeting # 112	R4-2412922

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.8.2.1

Source: 		Qualcomm Incorporated

Title: 	Coexistence study for NTN HPUE

Document for:	Approval

# Introduction

The new Rel-19 WI on enhanced requirements and test methodology for NR and IoT NTN was approved in [1] in RAN #103. The objective of the coexistence analysis for NTN HPUE is listed below:

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

  - Example bands include bands n256 and 256, n255 and 255

  - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

  - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

In this paper, we provide our views on coexistence study for NTN HPUE.

# 2. Discussion

## 2.1 General

In [1], the example bands are including n256/256 and n255/255. Therefore, 2GHz could be used in coexistence study for NTN HPUE. In Table 6.3.2-1 of TR 38.863, the following selected options were presented.

Table 1: Selected option for each scenario (Table 6.3.2-1 of TR38.863)

| Scenario | Aggressor system | Victim system | Environment | Contributing |
| --- | --- | --- | --- | --- |
| 1 | TN DL | NTN GEO DL | Urban | NTN UE ACS |
| 2 | TN UL | NTN GEO UL | Urban | NTN SAN ACS |
| 3 | NTN LEO-600 DL | TN DL | Rural | NTN SAN LEO ACLR |
|  | NTN GEO DL | TN DL | Rural | NTN SAN GEO ACLR |
| 4 | NTN GEO UL | TN UL | Urban | NTN UE ACLR |
| 5 | NTN GEO UL | TN DL | Rural | NTN UE ACLR |
| 61 | NR-TN DL | NTN LEO-600 UL | Rural 2 | NTN SAN ACS |
|  | NR-TN DL | NTN GEO UL | Rural 2 | NTN SAN ACS |
| 7 | HAPS DL | TN DL | Rural | HAPS ACLR |
| 8 | TN UL | HAPS UL | Rural | HAPS ACS |
| NOTE 1: Agreed representative case for Scenario 6. NOTE 2： The initial results suggested that the NR-NTN SAN would suffer more interference in urban deployment scenario. It is agreed that a more relevant environment for case 6 is a mixture of Urban and Rural environment (e.g., urban area with a 50km diameter inside a GEO beam with a 250km diameter). Further studies based on the mixed urban environment could be considered. As compromise, rural only scenario was then selected. |  |  |  |  |

Since the targeted coexistence analysis is related to higher UE transmit power, the concerned scenarios are scenario 4 and scenario 5 which are related to NTN UE ACLR.

Proposal 1: The coexistence simulation scenarios for NR NTN and IoT NTN HPUE should focus on Scenario 4 and Scenario 5 as described in TR 38.863. The example carrier frequency in the simulation is 2GHz.

In addition, wraparound function should be disabled since for Scenario 4 and 5, we are looking at the case NTN UEs are distributed at the edge of TN cluster.

Proposal 2: Wraparound function should be disabled for Scenario 4 and 5 in Rel-19 NTN HPUE coexistence study.

In [1], three UE power classes, i.e., PC2, PC1.5 and PC1 are included. From coexistence simulation PoV, there is no difference except maximum transmit power and power control sets among three UE power classes. Therefore, we suggest RAN4 to conduct coexistence study for PC2, PC1.5 and PC1. For TN UE power class, PC3 should be assumed which is the worst case scenario.

Proposal 3: RAN4 to conduct coexistence study for PC2, PC1.5 and PC1 for NR NTN and IoT NTN. The power control sets for PC2, PC1.5 and PC1 for NTN HPUE in below equation should be used. For TN, PC3 UE is assumed which is the worst case.

Where,

-	Pmax = maximum UE Tx power, i.e. 26 dBm for PC2, 29dBm for PC1.5 and 31dBm for PC1 NR NTN and IoT NTN UE;

-	Rmin = minimum power reduction ratio, i.e. -66/-69/-71 dB by assuming NTN UE PC2/PC1.5/PC1 min Tx power as -40dBm;

-	CLx-ile and γ are set as following:

-	γ = 1;

-	   CLx-ile = 88 + Delta_maxpower+10*log10 (200/X) + 11 – Y,  where X is UL transmission BW (MHz), Y is the BS noise figure, Delta_maxpower is the gap between PC1/1.5/2 and PC3. For example, For PC1, Delta_maxpower = 8dB.

## 2.2 Coexistence study on NR NTN HPUE

As stated in [1], the same simulation assumptions for NR NTN and TN coexistence in TR 38863 should be reused including the geographical separation between NTN UE and TN. We should note that the geographical separation of 1.5km was only considered in Scenario 1 simulation due to limited time as shown in Figure 1.

Figure 1: Isolation distance for Scenario 1 in TR 3886

Observation 1: The geographical separation of 1.5km was only considered in Scenario 1 in Rel-17 NR NTN and TN coexistence.

But it is more reasonable to consider 1.5km geographical separation in Rel-19 NTN HPUE coexistence study as this separation distance should apply for all the scenarios. Otherwise, it will have impact on the coexistence conclusion. In Table 1, we provide the preliminary results for Scenario 4 with and without 1.5km separation.

Table 1: Required ACIR for Scenario 4 with and without 1.5km separation

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S4 Avg. T-put loss | 24.9 | 19.3 | 14.5 | 10.5 | 7.5 | 5.1 | 3.3 | 2.1 | 1.3 | 0.7 |
| S4 edge T-put loss | 100 | 100 | 86.1 | 72.6 | 55.7 | 38.6 | 24.8 | 15.4 | 8.3 | 3.7 |
| S4_iso Avg. T-put loss | 1.5 | 0.7 | - | - | - | - | - | - | - | - |
| S4_iso edge. T-put loss | 3.9 | 1.6 | - | - | - | - | - | - | - | - |

Observation 2: It is reasonable to apply for geographical separation for all the coexistence scenarios such as S4 and S5 for NTN HPUE coexistence study in Rel-19.

Proposal 4: RAN4 to consider geographical separation for both Scenario 4 and Scenario 5 in NR NTN HPUE coexistence study.

## 2.3 Coexistence study on IoT NTN HPUE

The simulation assumptions and results for Rel-18 IoT NTN and TN coexistence are not captured in the TR 38.863. When RAN4 discussed coexistence between IoT NTN and TN, the agreed assumptions are included in R4-2217473. For IoT NTN coexistence study, one of the most important assumptions is ACLR/ACS modelling. Since only Scenario 4 and Scenario 5 need to be considered, ACLR/ACS modelling for Scenario 4 scenario in [3] can be reused for Scenario 4 and 5 in IoT NTN HPUE coexistence study

Proposal 5: For Scenario 4 and 5 in IoT NTN, the symmetric bandwidth ACLR model according to TR 36.942 section 5.1.1.4.2 with Flat ACLR model and FACLR = 0 dB is used, assuming flat ACS. The number of active UL NTN UEs is 9 UEs for 15kHz and 36UEs for 3.75kHz

Similar as NR NTN coexistence study, 1.5km geographical separation should be considered for both Scenario 4 and Scenario 5 in IoT NTN HPUE coexistence study.

Proposal 6: Geographical separation should be considered for both Scenario 4 and Scenario 5 in the IoT NTN HPUE coexistence study.

In IoT NTN coexistence simulation, the NTN UEs (as the aggressor) are randomly distributed at the edge of TN cluster.The UE distribution is fine for NR NTN since the 9 UEs are assumed. But it would be not correct for the case of UEs number increased to 36UEs for IoT NTN as it would be condensed in some places around the cluster which is not realistic scenario for IoT NTN deployments. In Figure 2 (left figure), the NTN IoT UEs were dropped based on max distance to the cell centre, as seen several locations were condensed with UEs especially when the number of UEs go to higher number like 36 UEs. On the other hand, in Figure 2 (right figure), the NTN IoT UEs were dropped in circle uniformly around the TN cluster which guarantee equal dropping around the cluster hence not biasing the results.

Figure 2: UE distribution of 36 NTN IoT UEs

Proposal 7: RAN4 to consider a uniform UE distribution dropping in a circle around the TN cluster for Scenario 4 and 5 in IoT NTN HPUE coexistence study.

# 3.	Conclusion

In this paper, we provide our views on the coexistence study for NTN HPUE. The following proposals and observations are made:

Proposal 1: The coexistence simulation scenarios for NR NTN and IoT NTN HPUE should focus on Scenario 4 and Scenario 5 as described in TR 38.863. The example carrier frequency in the simulation is 2GHz.

Proposal 2: Wraparound function should be disabled for Scenario 4 and 5 in Rel-19 NTN HPUE coexistence study.

Proposal 3: RAN4 to conduct coexistence study for PC2, PC1.5 and PC1 for NR NTN and IoT NTN. The power control sets for PC2, PC1.5 and PC1 for NTN HPUE in below equation should be used. For TN, PC3 UE is assumed which is the worst case.

Where,

-	Pmax = maximum UE Tx power, i.e. 26 dBm for PC2, 29dBm for PC1.5 and 31dBm for PC1 NR NTN and IoT NTN UE;

-	Rmin = minimum power reduction ratio, i.e. -66/-69/-71 dB by assuming NTN UE PC2/PC1.5/PC1 min Tx power as -40dBm;

-	CLx-ile and γ are set as following:

-	γ = 1;

-	   CLx-ile = 88 + Delta_maxpower+10*log10 (200/X) + 11 – Y,  where X is UL transmission BW (MHz), Y is the BS noise figure, Delta_maxpower is the gap between PC1/1.5/2 and PC3. For example, For PC1, Delta_maxpower = 8dB.

Observation 1: The geographical separation of 1.5km was only considered in Scenario 1 in Rel-17 NR NTN and TN coexistence.

Observation 2: It is reasonable to apply for geographical separation for all the coexistence scenarios such as S4 and S5 for NTN HPUE coexistence study in Rel-19.

Proposal 4: RAN4 to consider geographical separation for both Scenario 4 and Scenario 5 in NR NTN HPUE coexistence study.

Proposal 5: For Scenario 4 and 5 in IoT NTN, the symmetric bandwidth ACLR model according to TR 36.942 section 5.1.1.4.2 with Flat ACLR model and FACLR = 0 dB is used, assuming flat ACS. The number of active UL NTN UEs is 9 UEs for 15kHz and 36UEs for 3.75kHz

Proposal 6: Geographical separation should be considered for both Scenario 4 and Scenario 5 in the IoT NTN HPUE coexistence study.

Proposal 7: RAN4 to consider a uniform UE distribution dropping in a circle around the TN cluster for Scenario 4 and 5 in IoT NTN HPUE coexistence study.

# 4.	References

- RP-240857, ‘New WID: Enhanced requirements and test methodology for NR and IoT NTN’

- TR 38.863, ‘Non-terrestrial networks (NTN) related RF and co-existence aspects’

- R4-2217473, ‘Way Forward on coexistence study for IoT over NTN’