3GPP TSG-RAN WG4 Meeting #112	R4-2412928

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.1.3.2

Source:            	Google

Title:                	Discussion on MIMO layer for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the 6 MIMO layers support for 6Rx handheld and FWA UE.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#111, the 6 MIMO layers support for 6Rx handheld and FWA UEs has been discussed and some open issues are captured in the approved WF [2]. In WF, the main issue is to discuss whether 6 MIMO layer is supported for 6Rx handheld and FWA UEs, and several other related issues also need further discussion. In this paper, we would like to share our view in the following.

# Topic 3: MIMO layer evaluation for 6Rx UE ## Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE Issue 3-1-1: Tightening BS EVM requirement Way Forward: RAN4 to further discuss if the BS EVM requirement needs to be tightened when discussing the feasibility of 6-Layer MIMO for 6Rx UEs. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-1-3: Whether to use same requirement for handheld UE and FWA Way Forward: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices. ## Sub-topic 3-2: 6-layer Support Issue 3-2-1: 6-layer Support Agreement: - Decision on support of 6 MIMO layers for handheld is deferred until performance level differences in comparison with 4 MIMO layers are known based on evaluations considering realistic antenna correlation assumptions and deployment scenarios Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature. Issue 3-2-3: Release independence of 6-layer Support Way Forward: Follow RAN4 Chair guidance and defer any release independence discussions until the requirements are stable.

# Discussion

In 3GPP technology, multiple antennas technique is a key feature to enhance system data DL throughput. In modern commercial implementation, the handheld UE is often designed to be equipped with four Rx antennas when the frequency ranges are around mid/high band (MHB), e.g., n7 or n41 and so on, and ultra-high band (UHB), e.g., n77 or n79, and so on. However, due to physical form factor constraints, the number of multiple antennas cannot be increased without limitation. Hence, in current specification, 4Rx has been introduced for handheld UE and FWA UE from Rel-15 and 8Rx was introduced only for FWA UE in Rel-18.

In RAN4#111, MIMO layer evaluation for 6Rx handheld and FWA UE has been discussed and some open issues for ΔRIB,6R on the example bands are captured in the WF, and some open issues for 6Rx UE with 6 MIMO layer are captured in the WF. One of the open issues is to discuss whether 6 MIMO layer should be introduced for handheld UE and FWA UE. In our understanding, in order to accommodate as many RF frequency bands as possible for the high-end handheld UE product, there has been already as many physical antennas as possible in current commercial handheld UE design. Hence, if the handheld UE would like to support 6Rx, it would be challenging because there is very limited space which can put two additional new antennas without impact to overall legacy antennas. In other way, if the UE would like to perform antennas sharing for two additional legacy antennas to support 6Rx, it would also be challenging to guarantee that the legacy antenna performance would not degrade too much because these legacy antennas need to cover very large frequency range. In addition, the distance among each 6Rx antenna may also be decreased, and the antenna correlation may become higher which may cause unacceptable antenna performance.

One may question that the foldable phone may have large physical size which can highly reduce antenna correlation for 6Rx handheld UE. However, we think that it may not be necessary true for foldable phone in all scenarios. For example, if the foldable UE is in the unfolded mode, the larger distance among 6Rx antennas would make sense to reduce antenna correlation. However, if the foldable UE is in the close mode, the antenna correlation may not be reduced because one of the four side metal frames need to be used to support mechanical operation from open mode to close mode operation. For this one side metal frame, it is rarely designed to accommodate any antenna and only other three side metal frames can be used to cover the overall antenna design. This is different from the non-foldable phone that all four metal frames can be designed to allocate antennas. Hence, we think it is obvious that the antenna correlation for 6Rx UE may not be reduced for the foldable phone in the close mode, and there might be even worse than the non-foldable phone.

In our view, as above mentioned, it is observed that the antenna implementation complexity is very high for the handheld UE to be equipped with 6RX. The antenna correlation would be much higher for 6Rx handheld UE, than 4Rx handheld UE. Considering that 6 MIMO layer may suffer more performance degradation from higher antenna correlation than 4 MIMO layers, it is proposed not to introduce 6 MIMO layer for 6Rx handheld UE in Rel-19. If RAN4 consensus is to support 6 MIMO layer for 6Rx UE, it is proposed that the 6 MIMO layer performance requirement for 6Rx handheld UE should be separated from FWA UE, and that 6 MIMO layer for 6Rx UE should be optional feature from Rel-19.

Proposal 1: For the handheld UE equipped with 6Rx, considering that 6 MIMO layer may suffer more performance degradation from higher antenna correlation than 4 MIMO layers and that the antenna implementation complexity is very high, it is proposed not to introduce 6 MIMO layer for 6Rx handheld UE in Rel-19.

Proposal 2: If RAN4 consensus is to support 6 MIMO layer for 6Rx UE, it is proposed that the 6 MIMO layer performance requirement for 6Rx handheld UE should be separated from FWA UE, and that 6 MIMO layer for 6Rx UE should be optional feature from Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: For the handheld UE equipped with 6Rx, considering that 6 MIMO layer may suffer more performance degradation from higher antenna correlation than 4 MIMO layers and that the antenna implementation complexity is very high, it is proposed not to introduce 6 MIMO layer for 6Rx handheld UE in Rel-19.

Proposal 2: If RAN4 consensus is to support 6 MIMO layer for 6Rx UE, it is proposed that the 6 MIMO layer performance requirement for 6Rx handheld UE should be separated from FWA UE, and that 6 MIMO layer for 6Rx UE should be optional feature from Rel-19.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111

- R4-2409050, “Discussion on MIMO layer for 6Rx UE”, Google, RAN4#111