3GPP TSG-RAN WG4 Meeting # 112	R4-2412932

# Maastricht, Netherland, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to introduce two-band inter-band CA with BCS4 and 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Softbank |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19-Core |  |  |  |  |  | Date: |  |  | 2024-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce CA_n3A-n77(3A)_BCS4/5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce CA_n3A-n77(3A)_BCS4/5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec can’t support CA_n3A-n77(3A)_BCS4/5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-1>>

Table 5.5A.3.1-1c: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A | CA_n3A-n5A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n5A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3A-n7A | n38 n78 CA_n3A-n7A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7B | CA_n3A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3A-n7(2A) | CA_n3A-n7A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
| CA_n3(2A)-n7A | CA_n3A-n7A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | CA_n3(2A)_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3(2A)-n7(2A) | CA_n3A-n7A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
| CA_n3B-n7A | CA_n3A-n7A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3B-n7B | CA_n3A-n7A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n3 | CA_n3B_BCS 4 and 5 | 4 and 5 |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
| CA_n3A-n8A | CA_n3A-n8A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3(2A)-n8A | CA_n3A-n8A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3A-n18A | CA_n3A-n18A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
| CA_n3A-n20A | CA_n3A-n20A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n26A | CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n26(2A) | CA_n26(2A) CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n26A | CA_n3A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n26(2A) | CA_n26(2A) CA_n3A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n28A | n38 CA_n3A-n28A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 | 3 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n28A | CA_n3A-n28A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n28A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n34A | CA_n3A-n34A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n34 | 5, 10, 15 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n38A | CA_n3A-n38A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 40 |  |
| CA_n3B-n38A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3(2A)-n38A | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n39A | n3 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| CA_n3A-n40A | n408,9 CA_n3A-n40A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41A | n418,9 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41B | CA_n3A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n41C | n418 CA_n41C8 CA_n3A-n41A8 CA_n3A-n41C8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n3A-n41(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n3A-n67A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n74A | CA_n3A-n74A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n3A-n75A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n77A | n778,9 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n77(2A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n3A-n77(3A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |
| CA_n3A-n78A | n38 n788,9 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n78C | CA_n78C CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  | CA_n78C CA_n3A-n78A CA_n3A-n78C | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n3A-n78(2A) | n38 n788,9 CA_n3A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3(2A)-n78A | CA_n3A-n78A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | CA_n3(2A)_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n78A | CA_n3A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n78C | CA_n78C CA_n3A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3B-n78(2A) | CA_n3A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3A-n79A | n38 n798,9 CA_n3A-n79A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3(2A)-n79A | CA_n3A-n79A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | CA_n3(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n79C | n38 n798,9 CA_n79C8 CA_n3A-n79A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  | CA_n79C CA_n3A-n79A CA_n3A-n79C | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3(2A)-n79C | CA_n3A-n79A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | CA_n3(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3B-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n102A | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n3A-n102(2A) | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n3A-n102B | CA_n3A-n102A CA_n3A-n102B | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n3A-n102C | CA_n3A-n102A CA_n3A-n102C | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n3A-n102D | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n3A-n102E | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n3A-n105A | CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

<<End of Change>>