# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412934

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C

Agenda Item:	7.3.3

Document for:	Approval

Introduction

In the RAN4#110bis meeting, one issue was raised in the contribution [1] to discuss whether MSD test configuration with intra-band UL CA should be reconsidered. And the way forward [2] was approved with the following issues to be further discussed.

Proposal 1: Companies are encouraged to consider the following options for handling the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations: Option 1: No change from TR 38.862 guidelines Option 2: Do not consider all MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations. Option 3: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations. Proposal 2: If Option 2 or Option 3 in Proposal 1 would be considered, which release to start taking effect? Option 1: From earliest release where such MSD requirements have been specified. Option 2: Rel-18 Option 3: Rel-19 Proposal 3: Is there a need to introduce cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW? Option 1: Yes Option 2: No

In last meeting, RAN4 also discuss the MSD for CA_n40A-n41C with UL intra-band CA_n41C. There is no conclusion on this band combination. Based on the operator’s demand, they would like to check the MSD with full RB allocation as the full RB allocation is the main scenario for the deployment. In this paper, we’d like to share our views on MSD with the fully allocated maximum aggregated BW.

Discussion

fully allocated maximum aggregated BW

For CA_n40A-n41C with UL intra-band CA_n41C, if RAN4 choose the fully allocated maximum aggregated BW, the DL band n40 will be affected by the first adjacent channel leakage of UL intra-band CA_n41C. The PA non-linearity in 1st adjacent channel leakage (30dB) and band n41 Tx filter performance at DL frequency range of band n40 will dominate the REFSENS degradation. As RAN4 has specified the MSD due to cross band isolation from ACLR2 for the fallback CA_n40A-n41A, RAN4 can consider the similar method and discuss whether to specify the MSD due to cross band isolation from ACLR1 for CA_n40A-n41C with UL intra-band CA_n41C.

Generally, separate antenna RF architecture is used for DL_n40A-n41C_UL_n41C as shown below.

Figure 1 separate antenna RF architecture for DL_n40A-n41C_UL_n41C

And the MSD calculation for CA_n40A-n41C cross band isolation is shown below. The PA noise PSD at PA output port is -29.79dBm/MHz for the first adjacent channel region. Band n40 filter rejection at 2515~2675MHz is assumed as 30dB. And n41 Tx filter rejection at 2300~2400MHz is assumed as 20 / 30 dB. 10dB antenna isolation is assumed. It can be found that there is no MSD for full RB allocation of UL n41C.

table 1 MSD calculation for 20dB n41 Tx filter rejection at 2300~2400MHz

|  | parameters | n40 main path | n40 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 190 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n40 receival singal at ANT port, dBm |  | 13 | 13 |
| n40 filter rejection at 2515~2675MHz, dB | 30 |  |  |
| signal After n40 filter, dBm |  | -21 | -21 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -92 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -88 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n40 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.47 | -98.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n41 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -29.79 |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -23.26 |  |
| n41 Tx filter rejection at 2300~2400MHz, dB | 20 |  |  |
| n41 PA noise power at Rx n105 ant port at 2300~2400MHz, dBm |  | -53.26 | -53.26 |
| Total noise level at ANT port |  | -53.25 | -53.25 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -54.25 | -54.25 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -55.26 |  |  |
| MSD | 44.74 |  |  |

table 2 MSD calculation for 30dB n41 Tx filter rejection at 2300~2400MHz

|  | parameters | n40 main path | n40 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 190 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n40 receival singal at ANT port, dBm |  | 13 | 13 |
| n40 filter rejection at 2515~2675MHz, dB | 30 |  |  |
| signal After n40 filter, dBm |  | -21 | -21 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -92 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -88 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n40 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.47 | -98.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n41 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -29.79 |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -23.26 |  |
| n41 Tx filter rejection at 2300~2400MHz, dB | 30 |  |  |
| n41 PA noise power at Rx n105 ant port at 2300~2400MHz, dBm |  | -63.26 | -63.26 |
| Total noise level at ANT port |  | -63.24 | -63.24 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -64.24 | -64.24 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -65.25 |  |  |
| MSD | 34.75 |  |  |

If n41 Tx filter rejection at 2300~2400MHz is assumed as 20 ~ 30 dBm, the MSD calculation is about 34dB~44dB for DL_n40A-n41C_UL_n41C.

Currently, RAN4 has specified the MSD due to cross band isolation for the fallback CA_n40A-n41A as below, which can guarantee the performance of n41 Tx filter rejection at 2300~2400MHz. Not sure whether RAN4 need to specify one more MSD test configuration for DL_n40A-n41C_UL_n41C.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | ACLR2 |

Observation 1: RAN4 has specified the MSD due to cross band isolation for the fallback CA_n40A-n41A as below, which can guarantee the performance of n41 Tx filter rejection at 2300~2400MHz.

Proposal 1: RAN4 can discuss whether to specify one more MSD test configuration due to cross band isolation for DL_n40A-n41C_UL_n41C as the MSD test configuration for the fallback CA_n40A-n41A has been specified.

Summary

Observation 1: RAN4 has specified the MSD due to cross band isolation for the fallback CA_n40A-n41A as below, which can guarantee the performance of n41 Tx filter rejection at 2300~2400MHz.

Proposal 1: RAN4 can discuss whether to specify one more MSD test configuration due to cross band isolation for DL_n40A-n41C_UL_n41C as the MSD test configuration for the fallback CA_n40A-n41A has been specified.

- References

[1] R4-2404180, Reconsideration on MSD requirements with intra-band contiguous UL CA, Apple

[2] R4-2406676, WF on MSD requirements with intra-band contiguous UL CA, Apple