# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412940

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the reasonable UL RB configurations for intra-band UL CA

Agenda Item:	7.3.2

Document for:	Approval

Introduction

In the RAN4#110bis meeting, one issue was raised in the contribution [1] to discuss whether MSD test configuration with intra-band UL CA should be reconsidered. And the way forward [2] was approved with the following issues to be further discussed.

Proposal 1: Companies are encouraged to consider the following options for handling the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations: Option 1: No change from TR 38.862 guidelines Option 2: Do not consider all MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations. Option 3: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations. Proposal 2: If Option 2 or Option 3 in Proposal 1 would be considered, which release to start taking effect? Option 1: From earliest release where such MSD requirements have been specified. Option 2: Rel-18 Option 3: Rel-19 Proposal 3: Is there a need to introduce cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW? Option 1: Yes Option 2: No

In last meeting, RAN4 extended the discussion for this issue and there is no new conclusion on this. In this paper, we’d like to discuss this issue from the BS deployment perspective and UE implementation perspective.

Discussion for 1RB+1RB allocation

The difficulties for UE implementation

Referring to the contribution [3] in the last meeting, the wideband PSD for different RB allocations can be found below.

Figure 1 Wideband PSD for all waveforms at 0dB MPR

Due to the discrete RB allocations, the discrete spurious interference can be observed within the whole spectrum. It will lead UE to reduce the output power in order to meet the RF requirements including SEM and spurious emission. In addition, it may have impacts on the REFSENS of the other DL bands and cause some in-device coexistence issues. And as this is the corner case, UE may have to optimize the performance for this extreme scenario, but sacrifice the other UE performance, e.g. increasing UE power consumption or EVM or decreasing output power.

Observation 1: the extreme discrete RB allocations for intra-band UL CA will lead to the poor RF performance for UE and decrease the output power.

The gain VS loss for network deployment

Generally, if higher UL throughput is expected and one component carrier can’t provide the enough RB resources, network will configure the intra-band UL CA for UE. Then, the configured RB number may be larger than the Maximum bandwidth transmission configuration of one carrier. For intra-band UL CA with such extreme discrete RB allocation, e.g. 1RB+1RB allocation, such allocation will loss meaning or target of achieving higher UL throughput. From network perspective, the extreme discrete RB allocations for intra-band UL CA not only can’t achieve higher UL throughput, but also network have to face the system performance loss due to the reduction of UE output power.

Observation 2: From network perspective, the extreme discrete RB allocations for intra-band UL CA not only can’t achieve higher UL throughput, but also network have to face the system performance loss due to the reduction of UE output power.

The potential solution

Based on the analysis above, both network and UE have to face the difficulties and the system performance loss, when the extreme discrete RB allocations are configured for intra-band UL CA. 1RB+1RB allocation is the worst case for UE implementation in theory or it is more convenient to calculate MSD, but the extreme discrete RB allocations have no benefits and cause system performance loss.

Referring to the RF requirements for single carrier, all the RF requirements are specified based on the contiguous RB allocation or almost contiguous RB allocation without extreme discrete RB allocations.

Thus,from RAN4 perspective, the RF requirements should be specified based on the reasonable UL RB allocation for intra-band UL CA.

Proposal 1: From RAN4 perspective, the RF requirements should be specified based on the reasonable UL RB allocations for intra-band UL CA.

For example, it’s proposed to introduce the following condition into the specification.

For intra-band UL CA, the RF requirements are specified based on the UL RB allocations with the following assumed conditions. Lcrb1×2μ1+Lcrb2×2μ2≥Z Where, Z=min(NRB1×2μ1, NRB2×2μ2) Lcrb1 is the Transmission bandwidth in CC1, which represents the length of a contiguous resource block allocation expressed in units of resources blocks. Lcrb2 is the Transmission bandwidth in CC2, which represents the length of a contiguous resource block allocation expressed in units of resources blocks. NRB1 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks. NRB2 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks. μ1/μ2 is same as μ defined in TS 38.211 [6]

Summary

Observation 1: the extreme discrete RB allocations for intra-band UL CA will lead to the poor RF performance for UE and decrease the output power.

Observation 2: From network perspective, the extreme discrete RB allocations for intra-band UL CA not only can’t achieve higher UL throughput, but also network have to face the system performance loss due to the reduction of UE output power.

Proposal 1: From RAN4 perspective, the RF requirements should be specified based on the reasonable UL RB allocations for intra-band UL CA.

For example, it’s proposed to introduce the following condition into the specification.

For intra-band UL CA, the RF requirements are specified based on the UL RB allocations with the following assumed conditions. Lcrb1×2μ1+Lcrb2×2μ2≥Z Where, Z=min(NRB1×2μ1, NRB2×2μ2) Lcrb1 is the Transmission bandwidth in CC1, which represents the length of a contiguous resource block allocation expressed in units of resources blocks. Lcrb2 is the Transmission bandwidth in CC2, which represents the length of a contiguous resource block allocation expressed in units of resources blocks. NRB1 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks. NRB2 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks. μ1/μ2 is same as μ defined in TS 38.211 [6]

- References

[1] R4-2404180, Reconsideration on MSD requirements with intra-band contiguous UL CA, Apple

[2] R4-2406676, WF on MSD requirements with intra-band contiguous UL CA, Apple

[3] R4-2407372, On UL configuration for intra-band ULCA IMDs, Skyworks Solutions Inc.