3GPP TSG-RAN WG4 Meeting # 112	R4-2412941

# Maastricht, Netherland, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to introduce the reasonable UL RB configurations for intra-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19-Core |  |  |  |  |  | Date: |  |  | 2024-07-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Referring to the WF R4-2406676 and contribution R4-2407372, the extreme discrete RB allocations for intra-band UL CA will lead to the poor RF performance for UE and decrease the output power. From network perspective, the extreme discrete RB allocations for intra-band UL CA not only can’t achieve higher UL throughput, but also network have to face the system performance loss due to the reduction of UE output power. From RAN4 perspective, the RF requirements should be specified based on the reasonable UL RB allocations for intra-band UL CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For intra-band UL CA, the RF requirements are specified based on the UL RB allocations with the assumed conditions without extreme discrete RB allocations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The extreme discrete RB allocations for intra-band UL CA are not precluded in the spec. However, not only it will lead to the poor RF performance for UE and decrease the output power, but also the system performance will loss without good UL throughput. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-1>>

6.1A	General

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

UE supporting the IE [feature 41-4-7] for SRS bandwidth aggregation shall meet the minimum requirements for intra-band contiguous CA for SRS transmission only.

For intra-band UL CA, the RF requirements are specified based on the UL RB allocations with the following assumed conditions.

Lcrb1×2μ1+Lcrb2×2μ2≥Z

- Where,  Z=min(NRB1×2μ1,  NRB2×2μ2)

- Lcrb1 is the Transmission bandwidth in CC1, which represents the length of a contiguous resource block allocation expressed in units of resources blocks.

- Lcrb2 is the Transmission bandwidth in CC2, which represents the length of a contiguous resource block allocation expressed in units of resources blocks.

- NRB1 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks.

- NRB2 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks.

- μ1/μ2 is same as μ defined in TS 38.211 [6].

<<End of Change>>