3GPP TSG-RAN4 Meeting #112	R4-2412942

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2461 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ATG-Core) CR for TS 38.101-1 to clarify the Tx requirements definition for ATG UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the CR R4-2406592, the general clarificaiton on how to measure Tx requirements for ATG UE was removed. Based on the discussion paper R4-2409320, RAN4 need to clarify how to measure the Tx requirements referring to UL MIMO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For ATG UE, it’s proposed to clarify how to measure the each of Tx requirements referring to UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to measure Tx requirements for ATG UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J, 6.4J, 6.5J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-1>>

6.3J	Output power dynamics for ATG

6.3J.1	Minimum output power for ATG

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the sum of the mean power from all transmit connectors in at least one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3J.1-1for ATG UE with omni-directional antenna and in Table 6.3J.1-2 for ATG UE with antenna array.

Table 6.3J.1-1: Minimum output power for ATG UE with omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 6.3J.1-2: Minimum output power for ATG UE with antenna array

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -19 | -19+10log10 (BWChannel /20) | -19+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

6.3J.2	Transmit OFF power for ATG

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power requirements as specified in clause 6.3.2 are applicable for ATG UE.

6.3J.3	Transmit ON/OFF time mask for ATG

The transmit ON/OFF time mask requirements as specified in clause 6.3.3 are applicable for ATG UE at each transmit antenna connector.

6.3J.4	Power control for ATG

The power control requirements specified in clause 6.3.4 are applicable for ATG UE with all transmit antenna connectors in closed-loop spatial multiplexing scheme.

<<Next of Change>>

6.4J	Transmit signal quality for ATG

6.4J.1	Frequency error for ATG

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [7].

NOTE 1:	the ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

6.4J.2	Transmit modulation quality for ATG

The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. For ATG UE, the transmit modulation quality requirements are specified based on measurements made at each transmit antenna connector. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

<<Next of Change>>

6.5J	Output RF spectrum emissions for ATG

6.5J.1	Occupied bandwidth for ATG

The requirements for occupied bandwidth in clause 6.5.1 apply. For ATG UE, the requirements for occupied bandwidth are defined as the sum of the powers from all UE transmit antenna connectors.

6.5J.2	Out of band emission for ATG

6.5J.2.1	General

This clause contains requirements for out of band emissions for ATG UE, the requirement defined in general part of clause 6.5.2.1 should apply.

6.5J.2.2	Spectrum emission mask

If the actual transmission power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in clause 6.5.2.2 apply; if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB. For ATG UE, the requirements for Spectrum emission mask are defined as the sum of the emissions from all UE transmit antenna connectors.

NOTE:	This scaling factor is only applicable to ATG airborne UE.

6.5J.2.3	Adjacent channel leakage ratio

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. For ATG UE, the requirements for ACLR are defined as the sum of the emissions from all UE transmit antenna connectors.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5J.2.3-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value 30dBc.

Table 6.5J.2.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

6.5J.3	Spurious emissions for ATG

The requirements for spurious emission in general part of clause 6.5.3.0 and clause 6.5.3.1 apply. For ATG UE, the requirements for Spurious emissions are defined as the sum of the emissions from all UE transmit antenna connectors.

<<End of Change>>