# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412943

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	(NR_NTN_solutions-Core) Discussion on clarification for Terminology GSO

Agenda Item:	4.2

Document for:	Approval

Introduction

In the RAN4#111 meeting, one discussion paper [1] was provided to coordinate the accurate definition of “GSO” and “GEO”. In the end, there is no conclusion on this issue. In this paper, I’d like to share our views on this issue.

Discussion

In Rel-17 NTN WID [2], only the GEO (Geostationary Earth Orbiting) was mentioned in the objectives without any definition about “GSO”.

In Rel-18 NTN WID [3], GSO (Geo Synchronous Orbit) and NGSO (Non Geo Synchronous Orbit) were clearly defined.

As companies mentioned in their paper [1], most of 3GPP spec adopted the definition of GSO (Geo Synchronous Orbit).

Observation 1: in the satellite industry, most of satellite operators and vendors think that GSO represented Geo Synchronous Orbit.

However, as we all know, all of regulatory bodies, including ECC, FCC and ITU, think that GSO represent geostationary-satellite orbit. E.g. it can be found that GSO represent geostationary-satellite orbit at the link [4].

In ITU RR, the definition in items 1.188, 1.189 and 1.190 can be found below. It’s noted that there is an extension for geostationary satellite in ITU definition since most of geostationary satellite can only remain approximately fixed relative to the Earth.

1.188 geosynchronous satellite: An earth satellite whose period of revolution is equal to the period of rotation of the Earth about its axis. 1.189 geostationary satellite: A geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth’s equator and which thus remains fixed relative to the Earth; by extension, a geosynchronous satellite which remains approximately fixed relative to the Earth. 1.190 geostationary-satellite orbit: The orbit of a geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth's equator.

In FCC 47 CFR Part 25 [5], we can also find the similar definition of GSO and NGSO.

Observation 2: all of regulatory bodies, including ECC, FCC and ITU, think that GSO represents geostationary-satellite orbit.

In Rel-18 NR NTN, Ka bands were introduced into 3GPP and captured into TS 38.101-5 and TS 38.108. Some of regulatory requirements were captured into 3GPP spec. If RAN4 change the meaning of “GSO” and have a different understanding with other regulatory bodies, some regulatory requirements will be changed.

Observation 3: If RAN4 has a different understanding of GSO definition with other regulatory bodies, we have to face the risks that there is a different understanding of the regulatory requirements in the future. It may cause some issues about regulation compliance.

One of way forward is to avoid using the abbreviation of GSO in RAN4’s spec and clearly indicate the applicability with Geo Synchronous Orbit or geostationary-satellite orbit in the spec.

Proposal 1: In order to solve this issue, RAN4 can discuss whether to clearly indicate the applicability with Geo Synchronous Orbit or geostationary-satellite orbit in the spec instead of using the abbreviation of “GSO”/”NGSO” in RAN4’s spec.

Summary

Observation 1: in the satellite industry, most of satellite operators and vendors think that GSO represented Geo Synchronous Orbit.

Observation 2: all of regulatory bodies, including ECC, FCC and ITU, think that GSO represents geostationary-satellite orbit.

Observation 3: If RAN4 has a different understanding of GSO definition with other regulatory bodies, we have to face the risks that there is a different understanding of the regulatory requirements in the future. It may cause some issues about regulation compliance.

Proposal 1: In order to solve this issue, RAN4 can discuss whether to clearly indicate the applicability with Geo Synchronous Orbit or geostationary-satellite orbit in the spec instead of using the abbreviation of “GSO”/”NGSO” in RAN4’s spec.

- References

[1] R4-2409681, (NR_NTN_Solutions) On the definition of geosynchronous satellites, Nokia

[2] RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

[3] RP-234011, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales

[4] https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

[5] 47 CFR Part 25, satellite communication