# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412949

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	(NR_NTN_enh-Core) Discussion on potential solution on Doppler shift issues for guard band and transmission bandwidth configuration

Agenda Item:	5.23.5

Document for:	Approval

Introduction

In the RAN4#110 meeting, although the draft big CR [1] was endorsed in RAN4#110 meeting, doppler shift issues were not extensively discussed. Especially, RAN4 didn’t fully discuss the relationship among the guard band and transmission bandwidth configuration under large doppler shift condition. In RAN4#110bis meeting, this issue was raised by one paper [2]. Finally, a WF [3] was approved to further discuss this issue.

- Way forward Further discuss if extra guard-band is required/should be specified when VSAT UE is pre-compensating Doppler.

In this paper, we’d like to discuss the potential solutions for this issue.

Discussion

Doppler shift issues were discussed for FR1-NTN in Rel-18, which was raised by RAN5 LS [4]. In the end, the following testing configuration related to satellite access in table 1 were captured into the TS 38.101-5 for FR1-NTN UE. However, there is no discussion on this issue for Ka band VSAT. For NGSO scenario, the doppler shift could be up to 24ppm especially for LEO scenario. For 30GHz UL frequency, the doppler shift is about 720kHz. One RB could be shifted into the guard for 60kHz SCS when VSAT is handling the doppler pre-compensation as figure 1.

Figure 1 Edge RB falling into Guard band due to doppler shift

One solution is to follow the approach for FR1-NTN UE. All requirements for transmitter and receiver characteristics, other than frequency error, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

However, in the real deployment scenario, the doppler conditions are not zero especially for LEO scenario. It’s unclear how to specify or reuse current RF requirements when Edge RB falling into Guard band due to doppler shift.

Observation 1: The issue is identified that one RB could be shifted into the guard for 60kHz SCS when VSAT is handling the doppler pre-compensation.

Currently, the Out-of-Band Emission requirements specified in clause 6.5.2 and 9.5.2 from TS 38.101-5 are clearly defined that the Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

- 6.5.2 Out of band emission - 6.5.2.1 General The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

- 9.5.2 Out of Band Emissions - 9.5.2.1 General The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. Additional requirements to protect specific bands are also considered. The requirements in clause 9.5.2.2 only apply when both UL and DL of a VSAT UE are configured for single CC operation, and they are of the same bandwidth. All out of band emissions for FR2-NTN are TRP.

This definition is quoted from ITU regulation. If we read the SM.329, the similar definition can be found below.

- 1.3bis Out-of-band domain2 (of an emission): The frequency range, immediately outside the necessary bandwidth but excluding the spurious domain, in which out-of-band emissions generally predominate. NOTE 1 Out-of-band emissions, defined based on their source, occur in the out-of-band domain and, to a lesser extent, in the spurious domain. Spurious emissions likewise may occur in the out-of-band domain as well as in the spurious domain.

Observation 2: Both 3GPP spec and ITU regulation specified the out-of-band domain as the frequency range immediately outside the channel bandwidth.

From NTN UE perspective, if the digital filter is designed based on the spectrum utilization and guard band specified in clause 5.3.2 and 5.3.3 from TS 38.101-5, that means NTN UE can’t meet current unwanted emission requirements which are specified immediately outside the channel bandwidth when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Observation 3: based on the spectrum utilization and guard band specified in clause 5.3.2 and 5.3.3 from TS 38.101-5, that means NTN UE can’t meet current unwanted emission requirements which are specified immediately outside the channel bandwidth when the edge RB fall into the guard band due to the doppler shift pre-compensation.

In order to solve this issue, the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN.

Solution 1: the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN when the edge RB fall into the guard band due to the doppler shift pre-compensation.

One possible solution to improve the spectrum utilization is that UE may overcome the adverse effects that the edge RB fall into the guard band by implementation and meet current RF requirements. However, the most important thing is that network need to know whether the UE is capable of handling the adverse effects that the edge RB fall into the guard band. Thus, for UE supporting the deployment that the edge RB fall into the guard band in NGSO scenario, UE can report a static capability and then network can deploy the edge RBs for these UEs without worries.

Solution 2: UE can report a static capability whether UE is capable of supporting the deployment in NGSO scenario that the edge RB fall into the guard band.

If we study more on this issue, it can be found that different elevation angles for UE lead to different doppler shifts below. That means the doppler shift is dynamically varying for NGSO scenario.

Table 1 For a certain orbit, different doppler shifts for different elevation angles

| Elevation angles for UE | 10° | 30° | 50° | 70° | 80° | 90° |
| --- | --- | --- | --- | --- | --- | --- |
| Doppler shift Fd(ppm, LEO600) | 22.62 | 19.82 | 14.68 | 7.8 | 3.6 | 0 |
| Doppler shift Fd(ppm, LEO1200) | 19.99 | 17.51 | 12.97 | 6.89 | 3.18 | 0 |

Based on this phenomenon, UE can indicate a dynamic information to network whether the edge RB can be deployed or not.

Solution 3: UE can indicate a dynamic information to network whether the edge RB can be deployed or not.

Proposal 1: The following solutions are proposed to address the issue that the edge RB fall into the guard band due to the doppler shift pre-compensation.

Solution 1: the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Solution 2: UE can report a static capability whether UE is capable of supporting the deployment in NGSO scenario that the edge RB fall into the guard band.

Solution 3: UE can indicate a dynamic information to network whether the edge RB can be deployed or not based on whether the doppler shift is negligible or not.

Summary

Observation 1: The issue is identified that one RB could be shifted into the guard for 60kHz SCS when VSAT is handling the doppler pre-compensation.

Observation 2: Both 3GPP spec and ITU regulation specified the out-of-band domain as the frequency range immediately outside the channel bandwidth.

Observation 3: based on the spectrum utilization and guard band specified in clause 5.3.2 and 5.3.3 from TS 38.101-5, that means NTN UE can’t meet current unwanted emission requirements which are specified immediately outside the channel bandwidth when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Proposal 1: The following solutions are proposed to address the issue that the edge RB fall into the guard band due to the doppler shift pre-compensation.

Solution 1: the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Solution 2: UE can report a static capability whether UE is capable of supporting the deployment in NGSO scenario that the edge RB fall into the guard band.

Solution 3: UE can indicate a dynamic information to network whether the edge RB can be deployed or not based on whether the doppler shift is negligible or not.

- References

[1] R4-2401116, Big CR on TS38.101-5 for UE RF Requirements, Samsung Electronics France SA

[2] R4-2405340, Doppler shift issues for guard band and transmission bandwidth configuration, Huawei, HiSilicon

[3] R4-2405995, Way Forward for [110bis][305] NR_NTN_enh_Part1 Doppler pre-compensation into the guard band, Huawei, Thales

[4] R5-233672, LS on clarifications for Non-Terrestrial Networks, RAN5#99, May. 2023

2 	The terms “out-of-band domain” and “spurious domain” have been introduced in order to remove some inconsistency now existing between, on one hand, the definitions of the terms “out-of-band emission” and “spurious emission” in RR Article 1 and, on the other hand, the actual use of these terms in RR Appendix 3, as revised by the World Radiocommunication Conference (Istanbul, 2000) (WRC-2000). Out-of-band domain emission and spurious domain emission limits apply, respectively, to all unwanted emissions in the out-of-band and spurious domains.