# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412953

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the remaining issues for Rel-19 NTN RedCap UE

Agenda Item:	8.25.2.1

Document for:	Approval

Introduction

In the RAN4#110bis and RAN4#111 meetings, RAN4 has reached a lot of agreements on Rel-19 NR NTN RedCap UE according to the WF [1] and [2]. However, there are some remaining issues to be further concluded. In this paper, we’d like to show our views on these remaining issues.

Discussion

1) Maximum output power

In the RAN4#110bis meeting, RAN4 has agreed the following MOP requirements.

- Sub-topic 2-1: Target power class Agreement: Target power class is PC3.

In the RAN4#111 meeting, the following options are proposed to be further discussed.

- Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

Based on the agreements and proposals, I think the power class for full duplex FDD RedCap UE is PC3.

Observation 1: RAN4 has agreed that the power class for full duplex FDD NR NTN RedCap UE is PC3.

However, the insertion loss for HD-FDD RedCap UE could be reduced depending on the specific UE implementation. Referring to the paper [3], this point is also mentioned and proposed below.

Regarding the HD-FDD two antenna-port reference sensitivity allowance ΔRHD-FDD compared to full-duplex FDD operation, in our understanding, the refsens is improved by the adoption of HD-FDD. Therefore, as for 1Tx HD-FDD PC3 of NTN RedCap UE, because of the single-band 1Rx Refsens difference caused by using FD-FDD and HD-FDD, we wonder whether it is reasonable to think about the HD-FDD higher Tx output power value than FD-FDD Tx output power value. Observation 4: In 38.101-1, two antenna-port reference sensitivity allowance ΔRHD-FDD for HD-FDD RedCap UE obtained according to Tables 7.3.2-1a and 7.3I.2-2 ranges from -0.7dB to -0.9dB. The adoption of HD-FDD shows an improvement in the reference sensitivity compared to full-duplex FDD operation. Observation 5: By comparing the one antenna-port RF FD-FDD architecture with one antenna-port RF HD-FDD architecture, it seems that the IL improvement occurs in both one antenna-port Rx Refsens and Tx output power. Proposal 3: As only for 1Tx HD-FDD PC3 of NTN RedCap UE, because of the single-band 1Rx Refsens difference caused by using FD-FDD and HD-FDD, clarify whether it is reasonable to think about the HD-FDD higher Tx output power value than FD-FDD Tx output power value (i.e., 23dBm).

For NR NTN HD-FDD RedCap UE, there are three kinds of implementations from our perspective listed.

Implementation 1: To reuse the current Tx-Rx duplexer for HD-FDD band.

Implementation 2: To separate the Tx filter and Rx filter for HD-FDD band. The insertion loss could be reduced 0.5~1dB

Implementation 3: No Tx filter for HD-FDD band, so the insertion loss could be reduced more than 1dB.

Proposal 1: RAN4 can discuss whether to increase the power class 3 level for NR NTN HD-FDD RedCap UE, considering the following three kinds of implementation.

Implementation 1: To reuse the current Tx-Rx duplexer for HD-FDD band.

Implementation 2: To separate the Tx filter and Rx filter for HD-FDD band. The insertion loss could be reduced 0.5~1dB

Implementation 3: No Tx filter for HD-FDD band, so the insertion loss could be reduced more than 1dB.

2) Specification structure

Referring to the draft CR [4], two suffix letter “A” and “I” are used for NR NTN RedCap feature. As TS 38.101-5 is a novel specification, suffix letter could start from suffix letter “A”. In addition, it’s better to distinguish FR1-NTN and FR2-NTN in clause 4.3 as the supported features for FR1-NTN and FR2-NTN are different. The power class clarification in the draft CR is needed.

Proposal 2: It’s preferred to use suffix letter “A” for NR NTN RedCap UE feature. It’s better to distinguish sub-clause for FR1-NTN and FR2-NTN in clause 4.3. And the power class clarification for NR NTN RedCap UE is needed as what we did for TN RedCap UE.

Summary

Observation 1: RAN4 has agreed that the power class for full duplex FDD NR NTN RedCap UE is PC3.

Proposal 1: RAN4 can discuss whether to increase the power class 3 level for NR NTN HD-FDD RedCap UE, considering the following three kinds of implementation.

Implementation 1: To reuse the current Tx-Rx duplexer for HD-FDD band.

Implementation 2: To separate the Tx filter and Rx filter for HD-FDD band. The insertion loss could be reduced 0.5~1dB

Implementation 3: No Tx filter for HD-FDD band, so the insertion loss could be reduced more than 1dB.

Proposal 2: It’s preferred to use suffix letter “A” for NR NTN RedCap UE feature. It’s better to distinguish sub-clause for FR1-NTN and FR2-NTN in clause 4.3. And the power class clarification for NR NTN RedCap UE is needed as what we did for TN RedCap UE.

- References

[1] R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc.

[2] R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm Inc.

[3] R4-2407757, Discussion on UE RF requirements for NTN RedCap with HD-FDD, MediaTek Inc.

[4] R4-2408816, draft CR to TS 38.101-5: Addition of RedCap and eRedCap, Qualcomm Inc.