3GPP TSG-RAN WG4 Meeting # 112	R4-2412954

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL intra-band contiguous CA

Agenda Item:	8.10.2.1

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In the RAN4#111 meeting, we provided one more contribution [4]. RAN4 has agreed the following agreements in the way forward [3] for intra-band DL CA.

Issue 2-1-1: BCS for DL CA_n79C Agreement: - Reuse the existing BCS table for ATG - Clarify that uplink CA is not supported for ATG UE elsewhere for Rel-19 - FFS on whether BCS#4 and #5 are not supported for ATG UE for Rel-19 Issue 2-1-2: whether/how to reflect operation band information in spec Agreement: - The information that only DL CA is supported in Rel-19 for ATG can be reflected in the clause of operating band for ATG Issue 2-2: ATG Rx requirements applicability Agreement: - Following requirements needs to be updated for ATG CA compared with TN CA. - Max input level - ACS testing case 2 - Following requirements can reuse legacy TN CA requirements - Blocking including in-band and out of band blocking, narrow-band blocking - REFSENSE - Spurious response - Intermodulation - Following requirement is not applicable for ATG CA - No CA Rx Spurious emission - No CA narrow-band blocking - Following single carrier requirement of ATG is enough and no need to define ATG CA specific requirements - diversity characteristics requirement Issue 2-3: max input level Agreement: Table 3: Maximum input level for ATG with CA_n79C Rx Parameter; Units; ATG UE Types. Omni-directional antenna; Antenna array. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; -422; -302. -443; -323. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. The applicable NR CA Bandwidth Class; MHz; C. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM.. Issue 2-4: ACS testing parameters for case 2 Agreement: - the principle of TN contiguous CA could be reused

In this paper, we’d like to discuss the remaining open issues about intra-band DL CA for ATG UE.

Discussion

BCS configuration for DL CA_n79C

In last meeting, it has not been decided whether BCS#4 and #5 are not supported for ATG UE for Rel-19. Currently, the configurations for CA_n79C are specified below.

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA NR CA configuration / Bandwidth combination set. NR CA configuration; Uplink CA configurations or single uplink carrier5; Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Maximum aggregated bandwidth (MHz); Bandwidth combination set. CA_n79C; CA_n79C3; 50; 60, 80, 100; 200; 0. 60; 60, 80, 100. 80; 80, 100. 100; 100. See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 200; 4 and 5. CA_n79D; -; 100; 100; 100; 300; 0. See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 300; 4 and 5. NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: The aggregated bandwidth must be greater than or equal to the minimum for the bandwidth class defined in Table 5.3A.5-1, and smaller than or equal to the maximum aggregated bandwidth. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 4: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Only single uplink carriers with power class other than PC3 are listed..

Since RAN4 has clearly specified the CA_n79C_BCS0, CA_n79C_BCS4&5 in current specification, there are some risks to lead to some ambiguities if RAN4 specify a separate configuration table for ATG UE.

Thus, in order to keep the consistency as much as possible in the specification, it is no encouraged to create a separate configuration table for ATG UE.

RAN4 could consider the following solutions based on whether to support CA_n79C_BCS4&5 for ATG UE in Rel-19.

Option 1: if RAN4 decides not to support CA_n79C_BCS4&5 for ATG UE, RAN4 can clarify the applicability of BCS4/5 for ATG UE in the existing configuration table. But don’t create a new separate configuration table for ATG UE supporting CA.

Option 2: if RAN4 decides to support CA_n79C_BCS4&5 for ATG UE, RAN4 can reuse the existing configuration table for ATG UE supporting DL CA_n79C.

Proposal 1: RAN4 can discuss the following solutions based on whether to support CA_n79C_BCS4&5 for ATG UE in Rel-19.

Option 1: if RAN4 decides not to support CA_n79C_BCS4&5 for ATG UE, RAN4 can clarify the applicability of BCS4/5 for ATG UE in the existing configuration table. But don’t create a new separate configuration table for ATG UE supporting CA.

Option 2: if RAN4 decides to support CA_n79C_BCS4&5 for ATG UE, RAN4 can reuse the existing configuration table for ATG UE supporting DL CA_n79C.

Summary

Proposal 1: RAN4 can discuss the following solutions based on whether to support CA_n79C_BCS4&5 for ATG UE in Rel-19.

Option 1: if RAN4 decides not to support CA_n79C_BCS4&5 for ATG UE, RAN4 can clarify the applicability of BCS4/5 for ATG UE in the existing configuration table. But don’t create a new separate configuration table for ATG UE supporting CA.

Option 2: if RAN4 decides to support CA_n79C_BCS4&5 for ATG UE, RAN4 can reuse the existing configuration table for ATG UE supporting DL CA_n79C.

- References

[1] R4-2406594, WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344, General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2410687, WF on Rel-19 ATG UE requirements, CMCC

[4] R4-2409333, Discussion on Rel-19 ATG UE RF with DL intra-band contiguous CA, Huawei, HiSilicon