3GPP TSG-RAN WG4 Meeting # 112	R4-2412955

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39

Agenda Item:	8.10.2.2

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In the RAN4#111 meeting, we provided one possible RF architecture to achieve DL CA_n3-n39 with UL band n39 in our contribution [4]. In the WF [5], the following issues are still open and we’d like to discuss them in this paper.

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - FFS on whether to allow n39 UL for CA_n3A-n39A Issue 3-3: BCS for DL CA_n3_n39 Agreement: - FFS on BCS for DL CA_n3_n39

Discussion

RF architecture

Referring to the contribution [3] for handheld UE supporting DL CA_n3-n39, the following RF architecture is proposed for the only UL configuration in band n3.

Figure 1 RF reference architecture for handheld UE supporting CA_n3-n39

The UL/DL frequency ranges for band n3 and n39 are listed below.

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 | – | 1785 | 1805 | – | 1880 | FDD |
| n39 | 1880 | – | 1920 | 1880 | – | 1920 | TDD |

It can be observed that band n39 is adjacent to the DL band of band n3. Thus, when band n39 is transmitting the signal, the receiver of DL band n3 will suffer from the interference from band n39 for ATG UE. However, if WG is targeted to introduce UL configuration in band n39 for DL CA_n3-n39, the frequency gap between DL band n3 and band n39 should be assumed and considered in the real implementation as the transition guard band for the filter to protect each other.

Observation 1: if WG is targeted to introduce UL configuration in band n39 for DL CA_n3-n39, the frequency gap between DL band n3 and band n39 should be assumed and considered in the real implementation as the transition guard band for the filter to protect each other.

Referring to the collocated deployment between band n39 and band n3 in China, 5MHz frequency gap is reserved for the transition guard band between DL band n3 and band n39. Thus, technically it is possible for ATG UE to implement the DL_n3-n39_UL_n39 with 5MHz frequency gap by using the similar technology.

If the transition guard band between DL band n3 and band n39 is allowed to implement, one possible RF architecture with common antenna is shown in figure 2.

figure 2 One exemplary RF architecture with common antenna for DL CA_n3-n39 with UL in either band n3 or n39

Another possible RF architecture with separate antenna is shown in figure 3. Generally, 30dB isolation can be considered for collocated deployment. That means the separate antenna solution can provide more isolation between DL band n3 and UL band n39 than the common antenna solution.

Figure 3 Another RF architecture with separate antenna for DL CA_n3-n39 with UL in either band n3 or n39

For the RF architecture in the figures, only partial frequency range can be implemented for the filter(s) in DL band n3 or band n39 or both DL band n3 and n39.

Observation 2: it is possible to enable DL CA_n3-n39 with UL in either band n3 or n39 by implementing partial frequency range filter from technical perspective with common antenna or separate antenna.

Proposal 1: it’s allowed to achieve n39 UL for CA_n3A-n39A with 5MHz frequency gap in band n39.

Summary

Observation 1: if WG is targeted to introduce UL configuration in band n39 for DL CA_n3-n39, the frequency gap between DL band n3 and band n39 should be assumed and considered in the real implementation as the transition guard band for the filter to protect each other.

Observation 2: it is possible to enable DL CA_n3-n39 with UL in either band n3 or n39 by implementing partial frequency range filter from technical perspective with common antenna or separate antenna.

Proposal 1: it’s allowed to achieve n39 UL for CA_n3A-n39A with 5MHz frequency gap in band n39.

- References

[1] R4-2406594	WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344	General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2405342	Discussion and TP for TR 38.718-02-01 to introduce CA_n3A-n39A, Huawei, HiSilicon

[4] R4-2409334 Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39, Huawei, HiSilicon

[5] R4-2410687, WF on Rel-19 ATG UE requirements, CMCC