# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412956

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with UL MIMO

Agenda Item:	8.10.2.3

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic. In this paper, we’d like to discuss more details about ATG UE RF impacts with UL MIMO.

In last meeting, one way forward [3] was approved with some agreements. In this meeting, we’d like to share our views on the remaining open issues.

Discussion

Based on the way forward [3], there are some opens issues for further discussion in this meeting.

Issue 4-1-1: ULFPTx mode for UL-MIMO Agreement: - FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE Issue 4-2-6: whether to apply coherent UL MIMO requirement for ATG UE Agreement: - FFS on whether to apply coherent UL MIMO requirement for ATG UE Issue 4-3: receiver requirements Agreement: - FFS on whether to have Rx requirements for UL-MIMO for ATG UE

Since ATG UE can report the rated maximum output power, it’s totally different from handheld UE (power class report). Currently, the supported range of MOP for ATG UE is about 23~40dBm. Thus, the typical scenarios for handheld ULFPTx mode are 2*23dBm PA supporting 23dBm full power Tx per layer with 23dBm power class 3 report and 2*26dBm PA supporting 26dBm full power Tx per layer with 26dBm power class 2 report. However, for these scenarios, ATG UE can report higher MOP without full power mode as shown below.

| PA implementation | Handheld ULFPTx mode | ATG UE MOP report |
| --- | --- | --- |
| 2*23dBm PA | 23dBm full power Tx per layer with 23dBm power class 3 | 26dBm without ULFPTx mode |
| 2*26dBm PA | 26dBm full power Tx per layer with 26dBm power class 2 report | 29dBm without ULFPTx mode |

It’s unclear what the issues are expected to be resolved for ATG UE by leveraging the ULFPTx mode. Unless WG identify some benefits for ATG UE supporting ULFPTx mode, there is no need to extend ULFPTx mode to ATG UE.

Observation 1: Unless WG identify some benefits for ATG UE supporting ULFPTx mode, there is no need to extend ULFPTx mode to ATG UE.

Generally, the number of Tx port for handheld UE is assumed as 2 or 4. It’s unclear what’s assumption for ATG UE supporting UL MIMO. If only polarization diversity or multiplex is assumed, it seems that only two Tx ports can be assumed for ATG UE.

Proposal 1: there is no need to apply coherent UL MIMO for ATG UE since only polarization diversity or multiplex is assumed for ATG UL MIMO.

Since no matter whether UL MIMO is configured for ATG UE or not, the impacts on the Receiver are similar. Thus, as the Rx requirements have been defined or tested for general ATG UE, there is no need to repeat the same requirements under the UL MIMO scenario. Otherwise, it will become the test burden for ATG UE without meaning.

Proposal 2: Don’t specify Rx requirements for UL-MIMO for ATG UE.

Summary

Observation 1: Unless WG identify some benefits for ATG UE supporting ULFPTx mode, there is no need to extend ULFPTx mode to ATG UE.

Proposal 1: there is no need to apply coherent UL MIMO for ATG UE since only polarization diversity or multiplex is assumed for ATG UL MIMO.

Proposal 2: Don’t specify Rx requirements for UL-MIMO for ATG UE.

- References

[1] R4-2406594	WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344	General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2410687	WF on Rel-19 ATG UE RF requirements, CMCC