# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412957

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the RF impacts for Rel-19 PC2 TDD RedCap UE

Agenda Item:	8.7.2

Document for:	Approval

Introduction

In the RAN104 meeting, a new WI [1] for NR PC2 RedCap UE was established. TDD bands n41, n77, n78 and n79 were requested as exemplary frequency bands for PC2 RedCap UE. In this paper, we’d like to share our views on this WI.

Discussion

The following objectives for this WI can be found below.

The objectives of the core part are as follows: - Apply the PC2 RF requirements for non-RedCap UE to RedCap UE, based on 1Tx architecture with 26 dBm PA. - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed. - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. - Note: Not differentiate the applicable form factors for TDD bands in the specification. - Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined). Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI. Table: Requested example frequency bands for PC2 RedCap UE NR band; contact name, company; Contact email; other supporting companies (min. 3); status (new, ongoing, completed, stopped). n77, n78; Bo Liu, China Telecom; liubo1@chinatelecom.cn; MediaTek, ZTE, Sanechip, Vodafone, Verizon; completed for non-RedCap PC2, new for RedCap PC2. n41; Xiaoran ZHANG, CMCC; zhangxiaoran@chinamobile.com; MediaTek, ZTE, Sanechip; completed for non-RedCap PC2, new for RedCap PC2. n79; Xiaoran ZHANG, CMCC; zhangxiaoran@chinamobile.com; MediaTek, ZTE, Sanechip; completed for non-RedCap PC2, new for RedCap PC2.

Currently, RAN plenary reached consensus on the introduction of PC2 in TDD bands for RedCap UE. And for TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

Observation 1: for TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way based on the WID.

Generally, the RFFE implementation for PC2 RedCap UE in TDD bands is similar to the normal PC2 UE. Thus, a lot of Tx requirements for 1Tx normal PC2 UE in the exemplary TDD bands, e.g. Tx power tolerance, MPR and AMPR, can be reused for RedCap UE.

Observation 2: From technical implementation perspective, the Tx requirements for 1Tx normal PC2 UE in the exemplary TDD bands can be reused for RedCap UE due to the similar RFFE implementation.

As the general Tx requirements for 1Tx PC2 are applicable to PC2 RedCap UE in TDD bands, the spec impacts can refer to the Tx requirements for PC3 RedCap UE, i.e. clause 6.2I.1 Maximum output power for RedCap.

Proposal 1: for PC2 RedCap UE in TDD bands, it’s identified that the spec impact is to clarify the power class in the clause 6.2I.1.

Summary

Observation 1: for TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way based on the WID.

Observation 2: From technical implementation perspective, the Tx requirements for 1Tx normal PC2 UE in the exemplary TDD bands can be reused for RedCap UE due to the similar RFFE implementation.

Proposal 1: for PC2 RedCap UE in TDD bands, it’s identified that the spec impact is to clarify the power class in the clause 6.2I.1 of TS 38.101-1.

- References

[1] RP-241665, New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek