# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412959

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the NTN S band definition

Agenda Item:	7.17.2

Document for:	Approval

Introduction

In the RAN#104 meeting, a new WI [1] for region 2 S band was established. It’s expected that a new NR NTN band will be introduced with the following frequency range.

Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz

In this paper, we’d like to share our initial views on this WI.

Discussion

As the UL frequency range for this S band is 2000 – 2020 MHz, it’s overlapping with the DL frequency range 1995 ~ 2020 MHz of band n70, which is deployed in North America. Thus, it’s unclear how this issue will be addressed from RAN4 perspective.

Observation 1: the UL frequency range 2000 – 2020 MHz is overlapping with the DL frequency range 1995 ~ 2020 MHz of band n70.

In addition, the UL frequency range 2000 – 2020 MHz is very close to the DL frequency range of band n2/n25. There are only 5MHz gap between the DL frequency of band n25 and the UL 2000 – 2020 MHz. The first adjacent channel interference from UL 2000 – 2020 MHz will fall into the DL frequency of band n25/n2.

Observation 2: The first adjacent channel interference from UL 2000 – 2020 MHz will fall into the DL frequency of band n25/n2.

RAN4 need to discuss how to specify the UE-to-UE coexistence requirements between UL 2000 – 2020 MHz and other DL frequency range of band n2/n25/n70.

Proposal 1: RAN4 need to discuss how to specify the UE-to-UE coexistence requirements between UL 2000 – 2020 MHz and other DL frequency range of band n2/n25/n70.

After checking 47 CFR Part 25 [2], the following regulatory requirements related to S-band (2000 – 2020 MHz) are cited below.

25.265 Acceptance of interference in 2000-2020 MHz. (a) MSS receivers operating in the 2000-2020 MHz band must accept interference from lawful operations in the 1995-2000 MHz band, where such interference is due to: (1) The in-band power of any operations in 1995-2000 MHz (i.e., the portion of transmit power contained in the 1995-2000 MHz band); or (2) The portion of out-of-band emissions contained in 2000-2005 MHz

25.216 Limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service. (e) The e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after FEDERAL REGISTER publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

For the emission requirements, it seems that the frequency separation between UL 2000 – 2020 MHz and protected frequency range 1559 ~ 1610 MHz is large enough, so there is no need additional power reduction to comply with this requirements. But it can be reflected in the UE-to-UE coexistence table.

Proposal 2: RAN4 can discuss whether the additional power reduction is needed or not in order to comply with the regulatory requirements in the protected frequency range 1559 ~ 1610 MHz.

Summary

Observation 1: the UL frequency range 2000 – 2020 MHz is overlapping with the DL frequency range 1995 ~ 2020 MHz of band n70.

Observation 2: The first adjacent channel interference from UL 2000 – 2020 MHz will fall into the DL frequency of band n25/n2.

Proposal 1: RAN4 need to discuss how to specify the UE-to-UE coexistence requirements between UL 2000 – 2020 MHz and other DL frequency range of band n2/n25/n70.

Proposal 2: RAN4 can discuss whether the additional power reduction is needed or not in order to comply with the regulatory requirements in the protected frequency range 1559 ~ 1610 MHz.

- References

[1] RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

[2] 47 CFR Part 25, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25