# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412960

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Initial discussion on NR NTN Ku band

Agenda Item:	8.9.1

Document for:	Approval

Introduction

In the RAN#104 meeting, the WI [1] for NR NTN Ku band introduction was established to standardize Ku band for NR NTN. There are two general issues to be discussed initially.

Firstly, the following frequency ranges were proposed to further discuss which band will be discussed.

- For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required.

Secondly, RAN4 will select which one of the following approaches for DL and UL will be pursued by RAN #106.

Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106.

In this paper, we’d like to investigate the relative regulation for the potential frequency range and provide our views on how to select which approach can be pursued by RAN#106.

Investigation on relative regulation

If we check the satellite regulatory information from CEPT in the link [2], the following decisions can be summarized for Ku band. It can be observed that the 14.0~14.5GHz UL frequency range (Earth-to-space) was considered for satellite communication in Europe.

Table 1 The ECC decisions for Ku band

| ECC Decisions | space-to-Earth | Earth-to-space | Scenario | Type of Terminal |
| --- | --- | --- | --- | --- |
| ECC/DEC/(05)10 | 10.7-11.7 GHz/12.5-12.75 GHz | 14-14.5 GHz | GSO FSS | Earth Stations on board Vessels (ESV) |
| ECC/DEC/(05)11 | 10.7-11.7 GHz/12.5-12.75 GHz | 14-14.5 GHz | GSO | Aircraft Earth Stations (AES) |
| ECC/DEC/(18)04 | 10.7-12.75 GHz | 14.0-14.5 GHz | GSO FSS | Land based Earth Stations In-Motion (ESIM) |
| ECC/DEC/(18)05 | 10.7-12.75 GHz | 14.0-14.5 GHz | NGSO FSS | Earth Stations In-Motion (ESIM) |
| ECC/DEC/(19)04 | 10.7-12.75 GHz | 12.75-13.25 GHz | GSO FSS networks and NGSO FSS | Earth stations on-board aircraft |

Figure 1 Overview of relevant ECC Decisions for different categories of ESIM

Referring to the 47 CFR Part 25 (SATELLITE COMMUNICATIONS) [3], the following frequencies are available for use by ESIM communicating with GSO/NGSO FSS space stations in the US. It can be observed that the 14.0~14.5GHz UL frequency range (Earth-to-space) was considered for satellite communication in Europe.

In China, the MIIT (Ministry of Industry and Information Technology) released an administration document [4] on earth station in motion communicating with Geostationary orbit satellite in 2023. The UL frequency range 14-14.5GHz (Earth-to-Space) and DL frequency range 12.25-12.75GHz (Space-to-Earth) are available for the use of ESIM Geostationary scenario in Ku band. And a lot of regulatory RF requirements for coexistence can be found in the respective document.

第 三 条 动 中 通 地 球 站 上 行 业 务 链 路 仅 限 使 用 5925-6425MHz、14-14.5 GHz 和 27.5-30 GHz 频段，并应严 格遵守附件 1、2 和 3 所列技术要求和使用条件。下行业务 链 路 仅 限 使 用 3700-4200MHz 、 12.25-12.75GHz 和 17.7-20.2GHz 频段。其中，上行业务链路 5925-6425MHz 和 下行业务链路 3700-4200MHz 频段仅限船载动中通地球站在 距我国海岸线 300千米以外时使用。

Observation 1: In total, it seems that UL frequency range 14-14.5GHz is harmonized in the world for the use of ESIM in Ku band.

Observation 2: For the DL frequency range, RAN4 need more discussion on how to coordinate different regions. For example, one DL frequency range to cover all the regions or different bands for different regions considering different regulatory requirements.

In the WID, Priority 1 for Ku band #1a/#1b and Priority 2 for Ku band #2a/#2b are listed together. When companies start to discuss the Priority 2, the following conclusion [5] in agenda 1.15 for Ku band from WRC-23 can be referred as a baseline.

Observation 3: Once RAN4 starts to discuss the Priority 2, the conclusion in agenda 1.15 for Ku band from WRC-23 can be considered as an input from ITU.

Summary

Observation 1: In total, it seems that UL frequency range 14-14.5GHz is harmonized in the world for the use of ESIM in Ku band.

Observation 2: For the DL frequency range, RAN4 need more discussion on how to coordinate different regions. For example, one DL frequency range to cover all the regions or different bands for different regions considering different regulatory requirements.

Observation 3: Once RAN4 starts to discuss the Priority 2, the conclusion in agenda 1.15 for Ku band from WRC-23 can be considered as an input from ITU.

- References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

[2] https://www.cept.org/ecc/topics/satellite-regulatory-information

[3] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25

[4] https://www.miit.gov.cn/jgsj/wgj/wjfb/art/2023/art_8ff006c5a6304a67829e4670e0a4cb96.html

[5] Provisional Final Acts WRC-23, https://www.itu.int/en/publications/ITU-R/pages/publications.aspx?parent=R-ACT-WRC.15-2023&media=electronic