# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412961

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Initial discussion on VSAT requirements for NR NTN Ku band

Agenda Item:	8.9.4

Document for:	Approval

Introduction

In the RAN#104 meeting, the WI [1] for NR NTN Ku band introduction was established to standardize Ku band for NR NTN. Based on the WID, it’s assumed that VSAT type 1 to 5 specified in the Table 9.2.1.0-1 of TS 38.101-5 can be considered for Ku band. In this paper, we’d like to share our initial views on the VSAT requirements for NR NTN Ku band.

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: UE may need power reduction for meeting OFF-axis EIRP requirement defined in clause 9.2.2. Value is implementation dependent. |  |  |

Discussion

One of the assumptions captured in the WID is that NTN VSAT devices are to be considered (i.e. NTN capable UE type 1 to 5), and no handheld devices. Those VSAT types are listed below for reference:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: UE may need power reduction for meeting OFF-axis EIRP requirement defined in clause 9.2.2. Value is implementation dependent. |  |  |

Observation 1: Both Fixed VSAT, as well as Mobile VSAT (GSO connectivity only) are considered in this WI.

Observation 2: Both mechanical and electrically steered antennas are considered for VSAT terminals in this WI.

While VSAT types are expected to be reused from Rel-18 discussions, there are related VSAT device aspects to raise:

- Ku regulation: it seems that the Ku regulation is similar to related Ka band regulation,

- Antenna design: due to VSAT form factor, the starting point is expected to be legacy work on the Ka band, considering both antenna arrays, as well as parabolic antennas,

- RF architecture: Due to significant difference in frequency allocation, RF reuse from Ka band is far from obvious; As no implementation shall be precluded by the standard, underlying RF requirements need to be taken into consideration here.

- Baseband capabilities: refer to contribution under system parameters where we discuss on channel bandwidths for Ku band, where two main options were already captured in the WID.

- Multi-band integration: one more aspect to be answered is question on the need for VSAT terminals to support both Ka and Ku bands. Even if those will not be required to be used at the same time/region, it may be considered beneficial from VSAT terminals manufacturing point of view, economy of scale, global products, etc. It shall be noted, that there are already available commercial VSAT devices which do support both Ka and Ku bands (at least). On the other band, we cannot preclude devices which will support just Ku band.

Observation 3: While there are some similarities among Ka/Ku bands, Ku VSAT specification shall be flexible enough to consider various implementations.

Referring to the TR 38.821 [2], before Rel-19, 3GPP RAN never studied the system parameters in Ku band. Thus, it’s necessary to discuss the following basic assumptions for VSAT firstly.

1) How long is the Equivalent antenna aperture for VSAT assume?

This parameter will affect the antenna gain or G/T for VSAT in Ku band.

2) RAN4 can discuss whether the common Tx/Rx antenna or separate Tx/Rx antenna is assumed for VSAT in Ku band.

In Rel-18, the separate Tx/Rx antenna is assumed for VSAT in Ka band due to the large Tx/Rx frequency separation. However, due to the small Tx/Rx frequency separation in Ku band, it’s possible to implement the common Tx/Rx antenna. If this is the case, the Tx/Rx isolation should be considered when some receiver requirements will be affected by Tx interference.

3) For electronic steering antenna, the parameters for the parameterized array antenna model can be discussed in RAN4 referring to the table 8.1.1-1 of TR 38.921 [3].

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

4) The range of TRP is needed considering both mechanical steering antenna and electronic steering antenna.

5) RAN4 need to discuss the range of EIRP considering the applicable scenarios and implementation.

6) The UL/DL budget is needed to derive the minimum EIRP and EIS requirements.

Summary

Observation 1: Both Fixed VSAT, as well as Mobile VSAT (GSO connectivity only) are considered in this WI.

Observation 2: Both mechanical and electrically steered antennas are considered for VSAT terminals in this WI.

Observation 3: While there are some similarities among Ka/Ku bands, Ku VSAT specification shall be flexible enough to consider various implementations.

Observation 4: RAN4 can generally clarify and discuss the following issues in the first meeting.

1) How long is the Equivalent antenna aperture for VSAT assume?

2) RAN4 can discuss whether the common Tx/Rx antenna or separate Tx/Rx antenna is assumed for VSAT in Ku band.

3) For electronic steering antenna, the parameters for the parameterized array antenna model can be discussed in RAN4 referring to the table 8.1.1-1 of TR 38.921

4) The range of TRP is needed considering both mechanical steering antenna and electronic steering antenna.

5) RAN4 need to discuss the range of EIRP considering the applicable scenarios and implementation.

6) The UL/DL budget is needed to derive the minimum EIRP and EIS requirements.

- References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

[2] TR 38.821 v1.0.0

[3] TR 38.921 v17.1.0