# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412962

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence study for NR NTN Ku Band

Agenda Item:	8.9.2

Document for:	Approval

Introduction

In the RAN#104 meeting, the WI [1] for NR NTN Ku band introduction was established to standardize Ku band for NR NTN. The objective for adjacent coexistence study is shown below.

- Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services .

In this paper, we’d like to share our initial views on how to start the work for adjacent coexistence study for Ku band.

Discussion

Based on the ITU WRC-23 outcomes, the frequency range 10~10.5GHz is identified as IMT bands. In WRC-27 AI 1.17, the following frequency ranges including 14.8-15.35 GHz are considered to be studied for New IMT bands.

1.7 to consider studies on sharing and compatibility and develop technical conditions for the use of International Mobile Telecommunications (IMT) in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz (or parts thereof), and 14.8-15.35 GHz taking into account existing primary services operating in these, and adjacent, frequency bands, in accordance with Resolution COM6/26 (WRC-23);

It is obvious that the adjacent frequency range of the UL/DL Ku band will be used to deploy the IMT service. Thus, the scenarios for aggressor and victim specified in Table 6a.1-2 of TR 38.863 can be reused for Ku band adjacent channel coexistence study.

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1b | TN with NTN | NTN UL | TN UL | 14 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2b | TN with NTN | TN UL | NTN UL | 14 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3b | TN with NTN | NTN UL | TN DL | 14 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4b | TN with NTN | TN DL | NTN UL | 14 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5b | TN with NTN | TN DL | NTN DL | 11 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6b | TN with NTN | NTN DL | TN DL | 11 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7b | TN with NTN | NTN DL | TN UL | 11 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8b | TN with NTN | TN UL | NTN DL | 11 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |

Proposal 1: the scenarios for aggressor and victim specified in Table 6a.1-2 of TR 38.863 can be reused for Ku band adjacent channel coexistence study.

For Ka band, the 120kHz SCS and 200MHz BW were assumed referring to the clause 6a.2.2.1 of TR 38.863. If RAN4 agreed to go with approach #2, these parameters can be reused for Ku band adjacent channel coexistence study.

The parameters for satellite and VSAT in Ku band should be analyzed considering the difference between Ka band and Ku band, e.g. EIRP, antenna gain, TRP and antenna types.

Proposal 2: RAN4 should discuss and analyze the RF parameters for satellite and VSAT in Ku band, e.g. EIRP, antenna gain, TRP and antenna types.

For the TN parameters, currently, RAN4 established the SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz. It’s better to wait for the corresponding conclusion firstly.

Proposal 3: for the TN RF parameters, it’s better to align the outcomes of SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz as much as possible.

Summary

Proposal 1: the scenarios for aggressor and victim specified in Table 6a.1-2 of TR 38.863 can be reused for Ku band adjacent channel coexistence study.

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1b | TN with NTN | NTN UL | TN UL | 14 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2b | TN with NTN | TN UL | NTN UL | 14 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3b | TN with NTN | NTN UL | TN DL | 14 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4b | TN with NTN | TN DL | NTN UL | 14 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5b | TN with NTN | TN DL | NTN DL | 11 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6b | TN with NTN | NTN DL | TN DL | 11 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7b | TN with NTN | NTN DL | TN UL | 11 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8b | TN with NTN | TN UL | NTN DL | 11 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |

Proposal 2: RAN4 should discuss and analyze the RF parameters for satellite and VSAT in Ku band, e.g. EIRP, antenna gain, TRP and antenna types.

Proposal 3: for the TN RF parameters, it’s better to align the outcomes of SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz as much as possible.

- References