# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412963

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	General discussion on coexistence study for NR NTN HPUE

Agenda Item:	8.8.2.1

Document for:	Approval

Introduction

In the RAN#103 meeting, the WI [1] for Enhanced requirements and test methodology for NR and IoT NTN was established to introduce for HPUE feature. The objectives for NTN HPUE are listed below.

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

One of these objectives is to perform the adjacent channel coexistence studies for PC2 and other power classes. In this paper, we’d like to share our views on how to analyze the adjacent channel coexistence studies.

Discussion

1) Scenarios:

Referring to the table 6.1-2 from TR 38.863, there were 6 scenarios studied in Rel-17. As we all know, the scenario 4 and 5 can be used to derive the UE ACLR requirement and scenario 1 is used to derive the UE ACS requirement.

Proposal 1: the scenarios where NTN UL is the aggressor can be used to derive ACLR requirement for NTN HPUE.

table 1 The scenarios for NTN adjacent coexistence studies

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |

2) NTN UE UL power control

In Rel-17, for the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted to align with UE UL power control parameters used in TR 38.821. However, the propagation model between TN BS and TN UE is totally different from the propagation model between NTN SAN and UE. Thus, it’s necessary to reopen the discussion on how to derive the parameters for NTN UE UL power control.

Generally, the TPC model specified in Section 9.1 TR 36.942[2] is applied for TN with following parameters.

where:

-	Pmax = 26dBm for PC2,

-	Rmin = 56 dB assuming -30dBm minimum output power,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ) = 26 – [ 5 + (-174 + 67 + 4.3) ] = 123.7,

Where:

SNRtarget is proposed as 5dB.

NF is the SAN noise figure, i.e. 4.3dB.

B is the UL channel bandwidth, and 5MHz is assumed.

10log10(kT) = -174dBm/Hz.

-	γ = 1 For uplink scenario.

Generally, the free space path loss is 32.45+10log10(fc)+10log10(Distance).

where:

-	fc is the centre frequency in GHz. Here 2GHz is assumed,

-	Distance is the centre frequency in meter. Here it can be 600000m, 1200000m and 36000000m,

The general FSPL can be 154dB, 160dB and 190dB for LEO600, LEO1200 and GEO scenario. Based on assumption in Rel-17, 30dBi antenna gain is assumed for LEO and 51dBi antenna gain is assumed for GEO.

Then the coupling loss (CL) is 124dB, 130dB and 139dB.

Observation 1: if CLx-ile is assumed as 123.7dB, the power control for PC2 can work for LEO scenario with UL 5MHz bandwidth. However, for GEO scenario, PC2 UE may always transmit the maximum output power without any power control with UL 5MHz bandwidth.

Proposal 2:

For LEO scenarios, UL 5MHz bandwidth can be assumed. And correspondingly CLx-ile is assumed as 123.7dB for PC2.

For GEO scenarios, in order to guarantee the enough UL SINR, FFS UL channel bandwidth/RB allocation for PC2.

If we look into the current ecosystem and whole industry, perhaps only LEO scenario can be considered for NR NTN PC2 and GEO scenario can be considered for IoT NTN PC2.

3) Aggressor/Victim Channel bandwidth:

Based on the analysis above, 5MHz channel bandwidth can achieve a good UL SINR for NR NTN.

Proposal 3: it’s assumed 5MHz channel bandwidth for aggressor NTN UE and victim TN system.

Summary

Proposal 1: the scenarios where NTN UL is the aggressor can be used to derive ACLR requirement for NTN HPUE.

Observation 1: if CLx-ile is assumed as 123.7dB, the power control for PC2 can work for LEO scenario with UL 5MHz bandwidth. However, for GEO scenario, PC2 UE may always transmit the maximum output power without any power control with UL 5MHz bandwidth.

Proposal 2:

For LEO scenarios, UL 5MHz bandwidth can be assumed. And correspondingly CLx-ile is assumed as 123.7dB for PC2.

For GEO scenarios, in order to guarantee the enough UL SINR, FFS UL channel bandwidth/RB allocation for PC2.

Proposal 3: it’s assumed 5MHz channel bandwidth for aggressor NTN UE and victim TN system.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)

[2] TR 36.942